CE-QUAL-W2:  A Two-Dimensional, Laterally Averaged, Hydrodynamic and Water Quality Model, Version 4.2.2

User Manual:

Part 5 – Model Utilities

Edited by

Scott A. Wells

Professor

Department of Civil and Environmental Engineering

Portland State University

Portland, OR  97207-0751

Department of Civil and Environmental Engineering

Portland State University

Portland, OR 97207-0751

February 2021

# Contents

Contents	ii

List of Figures	iv

List of Tables	v

Preface	vi

1.	Introduction	7

2.	Water Balance Utility	7

How to use the water balance utility GUI application	7

Running the water balance program	7

Incorporating the computed flows into the simulation	10

Example from Tom Cole	11

How to use the water balance utility console application	12

Input File: “WatBal.npt”	12

Output File: “WatBal_Errors.opt”	12

Output File: “el_stats.opt”	12

Output File from water balance	13

How to Run the Console Water Balance Utility	13

3.	Control File Converter for CE-QUAL-W2	14

4.	GUI Interface for CE-QUAL-W2 for Versions 3.7-4.20	19

Introduction	19

Installing the GUI Interface	19

Opening the GUI Interface	20

Menus	20

Button Bar	20

Editing the Control File	21

Editing the Bathymetry File	23

Merging or Splitting Segments or Layers	24

Running the Preprocessor within the GUI	26

A Note on Saving Files	26

Typical Errors with the GUI	26

Command Line Processing	27

Using the GUI W2Control on Touchscreen Laptops and Monitors	27

5.	Post-Processor for CE-QUAL-W2 by DSI, Inc.	27

6.	Excel Macro Utility for CE-QUAL-W2	28

7.	W2 V4.2 Bug Fixes, Enhancements, and User Manual Changes	31

8.	W2 Known Code Limitations	42

9.	Appendix A: Differences between CE-QUAL-W2 Versions	44

Differences between Version 4.2.2 and Version 4.2.1	44

Differences between Version 4.2.1 and Version 4.2	45

Differences between Version 4.2 and Version 4.1	45

Differences between Version 4.1 and Version 4.0	45

Differences between Version 4.0 and Version 3.72	45

Differences between Version 3.72 and Version 3.71	46

Differences between Version 3.71 and Version 3.7	46

Differences between Version 3.7 and Version 3.6	46

Differences between Version 3.6 and Version 3.5	55

Differences between Version 3.2 and Version 3.5	57

Differences between Version 3.1 and Version 3.2	69

10.	Appendix B: BUG FIXES AND ENHANCEMENTS BETWEEN VERSIONS	74

W2 V4.1 Bug Fixes, Enhancements, and User Manual Changes	74

W2 V4.0 Bug Fixes, Enhancements, and User Manual Changes	89

W2 V3.7 Bug Fixes, Enhancements and User Manual Changes	93

W2 V3.6 Bug Fixes, Enhancements, and User Manual Changes	110

W2 V3.5 Bug Fixes, Enhancements, and User Manual Changes	124

W2 V3.2 Bug Fixes, Enhancements, and User Manual Changes	141

# List of Figures

Figure 1. Dialog box for water balance utility.	8

Figure 2. Dialog box for water balance utility if successful completion.	8

Figure 3. An example of an existing w2_con.xlsm file. Columns A and B are not used in the control file and we will be pasting the w2_con.csv into column C1, not A1.	14

Figure 4. An example of the w2_con.csv file from the converter utility.	15

Figure 5. Location in Excel template where the constituent order is shown. In this example, the required number of constituents is 30. This is a formula computed in the Excel sheet. Make sure that you have that same number in the list that follows. There is guidance on another tab in the Excel file on the required order based on your dimensioning of number of algal groups, BOD groups, macrophyte groups, periphyton groups, suspended solids groups, and generic constituent groups. Also note that in column A, guidance is given on setting the number of columns needed in this section.	16

Figure 6. Location in Excel file where the number of rows needs to be increased in case there are more than 5 structures.	16

Figure 7. Location in Excel file where the number of rows needs to be increased in case there are more than 5 epiphyton/periphyton groups.	17

Figure 8. Location in Excel file where the number of rows needs to be increased in case there are more than 5 zooplankton groups.	17

Figure 9. Location in Excel file where the number of rows needs to be increased in case there are more than 5 macrophyte groups.	18

Figure 10. About W2 Control GUI preprocessor.	19

Figure 11. Dialog box menu.	20

Figure 12. Menu of w2control.	20

Figure 13. Example of w2control menu.	21

Figure 14. Dialog box example.	22

Figure 15. Dialog box example.	23

Figure 16. Bathymetry dialog box example.	24

Figure 17. Segments of a model dialog box.	24

Figure 18. Splitting or merging layers example.	25

Figure 19. Merge/split dialog box.	25

Figure 20. Running dialog box within w2control.	26

Figure 21. Splash screen for W2_Post.	28

# List of Tables

Table 1. List of bug fixes and enhancement code changes since CE-QUAL-W2 Version 4.1. Note that shaded areas toggle between release versions.	31

Table 2. Bug fixes and enhancements for Version 4.1.	74

Table 3. Bug fixes and enhancements for Version 4.0.	89

Table 4. Bug fixes and enhancements for Version 3.7.	93

Table 5. Bug fixes and enhancements for Version 3.6.	110

Table 6. Bug fixes and enhancements for Version 3.5.	124

Table 7. Bug fixes and enhancements for Version 3.2.	141

# Preface

This manual documents the two-dimensional, laterally averaged, hydrodynamic and water quality model CE-QUAL-W2. As in all complex models, there have been many contributors. This re-write of the User Manual was based on prior User Manuals: Environmental and Hydraulic Laboratories (1986), Cole and Buchak (1995) Version 2, and Cole and Wells (2000) Version 3.0 through Cole and Wells (2019) Version 4.1. Hence, one can think of the primary author as an editor of past documents, rather than reflecting one person’s sole authorship. This updated User Manual contains numerous corrections, new figures, new sections, additional documentation, and improvements in organization and presentation of information compared to Cole and Wells (2019).

This section of the User Manual Part 5 documents the model utilities used in supporting the model, the model release notes and a detailed list of bug fixes.

The other sections of the User Manual are divided into multiple sections for ease of updating and editing:

- User Manual Part 1: Introduction to CE-QUAL-W2, Model download package, how to run the model, model versions, changes between model versions

- User Manual Part 2: Theoretical basis for CE-QUAL-W2: hydrodynamics and water quality, particle transport and numerical scheme

- User Manual Part 3: Model input and output file descriptions and input/output file examples

- User Manual Part 4: Model examples

- User Manual Part 5: Release notes, bug fixes, differences in model versions, history of bug fixes, and other user manuals such as for the GUI interface, the water balance algorithm, and other external codes.

This report should be cited as follows:

Wells, S. A. (2020) "CE-QUAL-W2:  A two-dimensional, laterally averaged, hydrodynamic and water quality model, version 4.2.2, user manual part 5, model utilities and release notes," Department of Civil and Environmental Engineering, Portland State University, Portland, OR.

# Introduction

The CE-QUAL-W2 model offers several useful model utilities for editing the control file, post-processing model results, and setting up model input files. A description of these utilities is included in this part of the User Manual.

Release notes are available that document model changes and bug fixes and provide background in running the model.

Hence, this section of the User Manual includes the following sections:

- Model utilities

  - Water balance utility

  - GUI control file editor and bathymetry viewer

  - Post-processor provided by DSI, Inc. including a bathymetry viewer

  - Excel macro utility for writing out input files for CE-QUAL-W2

- Model release notes

  - Current Bug fixes

  - Model Know Limitations

  - Model changes between versions

  - Model bug fixes between versions

# Water Balance Utility

Primary author: Tom Cole

### How to use the water balance utility GUI application

#### Running the water balance program

When the executable is run, a window appears that allows the following inputs (note that the executable runs under the Windows operating system only), as shown in Figure 1:

Figure 1. Dialog box for water balance utility.

When the dialog box first appears, default values populate the edit boxes.  The user can then edit each one if the default values are not correct.   Selecting Run will run the water balance utility to completion as shown in the following dialog box (Figure 2).

Figure 2. Dialog box for water balance utility upon successful completion of run.

Observed elevations filename.  This file consists of a Julian day and observed elevation, as in the following example:

2000-2001 Oologah Reservoir observed water surface elevations

JDAY     ELO

90.792 195.453

90.833 195.456

90.875 195.459

90.917 195.441

90.958 195.444

91.000 195.450

91.042 195.441

91.083 195.441

91.125 195.447

91.167 195.441

91.208 195.444

91.250 195.438

91.292 195.432

This example skips the first three lines and is of a similar format to all other CE-QUAL-W2 time-varying inputs, with a fixed format of eight columns each for the JDAY and ELO values.  However, the utility will read in values using variable field lengths so long as the JDAY and ELO values are separated by a space.  Data need not be at regular intervals that might cause a repeat of the same values. Sometimes better results are obtained when values that repeat over a time interval are not included (such as the water surface elevation at day 91.083 listed above).

Also note that the degree of accuracy in the observed elevations can have an impact on the computed flows.  It is up to the user to decide the precision necessary for the observed water surface elevations.

Computed elevations filename.  The following shows an example output file from a time series file (columns to the right are not displayed):

JDAY,       DLT,      ELWS, ..............................

92.000,    456.39,    195.37, ..............................

93.000,    952.28,    195.28, ..............................

94.000,    494.28,    195.27, ..............................

95.000,     34.43,    195.17, ..............................

96.000,    170.88,   195.08, .............................

97.000,    211.93,    194.99, ..............................

98.000,    255.11,    194.92, ..............................

99.000,   1459.08,    194.78, ..............................

The water balance utility reads in the [JDAY] and [ELWS] values and uses these in the water balance computations.  The user must turn on time series output in the model control file and specify the segment at which the water surface elevation values are output (typically the segment next to the dam for reservoirs).  Information on accomplishing this is offered in the User Manual under the Time Series output file discussion.

Add to previous water balance.  For various reasons, the water balance utility may not perfectly close the water balance the first time through the computations.  Depending on the discrepancies between computed and observed elevations, the utility may need to be used iteratively by rerunning the model using output from the first run of the water balance utility and then rerunning the water balance utility on the water surface elevations output in the new time series file.  For a system with multiple branches, each iteration of the utility and the resulting output file can be saved as a separate file that is then incorporated as a distributed tributary for branch 2, then branch 3, etc.  In the case of a system with only one branch, this approach cannot be used.  Rather, the new flows generated at the second iteration need to be added to the previously computed flows and incorporated as an “improved” distributed tributary inflow file.  This option allows the user to continue adding flows to the same inflow file.

The computed flows are contained in the qwb.opt file.  For most simulations, these flows will generate water surface elevations sufficiently close to the observed elevations that further refinement is unnecessary.  However, as mentioned above, the solution may need to be iterated.  Rarely, manual adjustment of the generated flows may be required.  This is usually only needed when observed water surface elevations change significantly over a short time period.

Previous water balance filename.  If the “Add to previous water balance” option is used, you must specify the existing water balance output file to which the computed flows should be added.

Skip interval.  Some reservoirs have a lot of noise in the observed water surface elevation data, such as in peaking hydropower operations, and this option allows the user to specify how many observed elevations are ignored when computing the flows between observed elevations.  For example, if water surface elevations are available on an hourly interval, the resulting flows generated by the water balance utility can have large + and – flows that are unrealistic, as opposed to using observed elevations on a daily basis taken during periods of no hydropower generation.  In order to smooth out the computed flows, a skip factor of 24 would result in computed flows being output on a daily basis, with all of the “noise” generated by hydropower operations ignored over the 24-hour period.

Averaging interval.  This option computes a running average of the water surface elevation based on the input value.  This is an additional aid to smooth out water surface elevation “noise”.  For example, consider the case in which there is no inflow/outflow to the system, but there is considerable wind seiching.  The water balance utility would compute alternating inflows and outflows from the system that, depending on the amount of seiching, could be very large, when in reality no flows should be added to or subtracted from the system.  Using a running average alone or in combination with skipping over a number of observed elevations specified in (4) can help alleviate many of the problems caused by an automated water balance computation.

Waterbody number.  In the case of multiple waterbodies in which each has a separate bathymetry input file, the user must specify for which waterbody (and thus which bathymetry file) the water balance is being computed.  This capability is necessary for modeling systems with multiple reservoirs.

#### Incorporating the computed flows into the simulation

The water balance utility can be used for lakes and reservoirs in which water surface elevations are a function of inflows and controlled outflows from the system.  The utility computes the flows necessary to match observed water surface elevations (typically taken at the dam) and outputs them to the “qwb.opt” file.  This file is composed of a Julian date and an inflow (m3 sec-1).  The flows can be either positive or negative.  Temperatures and/or constituent concentrations must also be provided in the corresponding temperature and constituent concentration input files if the computed flows are incorporated as inflows to the system.  The water balance utility does not provide this information, but this information must be provided by the user depending on how the computed flows are incorporated into the simulation.  Considerable thought should go into how to best incorporate temperature and constituent concentrations, which is discussed in more detail below.

Note that negative flows use temperatures/concentrations in the waterbody when calculating the impact on the system of these flows rather than the temperatures/concentrations in the corresponding inflow temperature and constituent concentration files.  This ensures that negative flows generate no change in temperature or constituent concentrations.  However, positive flows can impact simulation results, and care must be taken when deciding how these flows are incorporated into the simulation.

The flows required to complete the water balance are computed as a step function.  If they are incorporated into the model as an additional inflow or outflow whose current values are being linearly interpolated, such as a branch inflow, then the resulting water balance will not be correct.  Typically, the flows in the “qwb.opt” file are first included as a distributed tributary inflow assigned to the mainstem branch, and interpolation [DTRIC] is turned “OFF”.  The corresponding distributed tributary inflow temperatures are usually set to air temperatures in the qdt_br1.npt file.  When running water quality, care must be taken regarding what constituents to include in the corresponding inflow constituent concentration file.  Typically, only DO values are included if the distributed tributary option of incorporation is used, and these are set to saturated values corresponding to the observed air temperatures.  Keep in mind that if the water balance flows are incorporated as branch inflows, then the mass loading of organic matter and nutrients will be increased as well.

The branch corresponding to the distributed tributary inflow is usually assigned to the mainstem branch of a reservoir.  Using a distributed tributary minimizes the impact of the flow, temperature, and/or water quality associated with the distributed tributary by distributing the flow throughout all segments in a branch weighted by surface area.  Be aware that large flows as a result of large errors in inflow/outflow measurements can and have had a significant impact on temperature and water quality calibration in the surface layers.  Usually, this is not a problem, but sensitivity analyses should be conducted to see if the flow and associated temperature/constituent concentrations have an impact on the simulation results.  If so, then the following discussion is of particular relevance.

As emphasized previously, a great deal of thought should go into how the flows generated from the water balance utility are incorporated into the simulation.  As discussed previously, these are typically incorporated as distributed tributary inflows so as to minimize the impact of the flows on the simulation.  However, this may not always be the best, most accurate, or most realistic method.  For example, suppose that the water balance flows are consistently negative.  This would indicate that either inflows are consistently overestimated or outflows consistently underestimated.  Obviously, incorporating the flows as a positive increase in the outflows as opposed to subtracting them from the inflows can potentially have a very significant impact on simulation results.  In this case, sensitivity analyses should be conducted to determine which method improves the simulation results.  If, say, hypolimnetic temperatures are consistently being underestimated, then incorporating the flows into a hypolimnetic outflow could improve the simulation results.  Conversely, if hypolimnetic temperatures were being overpredicted, then the inflows should probably be reduced.  The key point to keep in mind is that there are a number of different ways to incorporate computed flows, and generally, they should all be tested to determine the best way to incorporate the computed flows into the simulation.

As another example, consider the case in which the generated flows are consistently positive and a branch in which sometimes significant inflows are ungauged.  In this case, a sensitivity analyses should be performed to determine if incorporating the flows or a portion of the flows into the ungauged branch inflow improves model results.  Often, the model can serve as a guide as to how to best incorporate the computed flows into the simulation.

#### Example from Tom Cole

Walter F. George is a U.S. Army Corps of Engineer reservoir located on the Chattahoochee River in Alabama.  The reservoir is operated as a peaking hydropower facility.  During calibration, the model consistently underpredicted hypolimnetic temperatures by 0.5-1ºC.  Wind sheltering could be adjusted to increase hypolimnetic temperatures, but this adjustment always adversely impacted thermocline depth.  After considerable thought, it was concluded that including possible seepage at the dam might improve hypolimnetic temperature predictions.  A portion of the distributed tributary flows were incorporated as an additional outflow at the bottom of the dam.  The final value used was 5 m3 sec-1, which was less than 1% of the average outflows and brought hypolimnetic temperatures into almost exact agreement with observed temperatures.  Further investigation of the outflows revealed that during times of no power generation, an additional flow of 5.1 m3 sec-1 was specified in a file that was not originally sent as part of the outflow data.  Thus, the model indicated how to best incorporate the computed flows, and was a surprisingly accurate indicator of what was occurring in the prototype.

### How to use the water balance utility console application

A console application that reads an input file is also available to perform the water balance. This new console version of the water balance code provides the following updates:

- This is not a windows dialog box driven code so it can easily be used in batch files.

- There is now an input file for model parameters for the water balance utility and an output file for model errors.

- There are several enhancements: one has more flexibility over file naming, the number of header lines to skip in input files, and the number of waterbodies to use in the analysis.

#### Input File: “WatBal.npt”

The new input file, “WatBal.npt” has the following format:

Water Balance input file for Console Application

"el_obs.npt"       , file for observations, time and water level time series in 2 columns

"tsr_8_seg36.opt"  , tsr file for model predictions-assuming only one line skipped in file header

"qwb1.npt"         , Output file name

1                  , NSKIPS - number of skips of data

1                  , NAV - averaging interval, number of data points to average

1,1      , waterbody to perform water balance: JW1:STARTWB,JW2:ENDWB:1,2=WB1&2;1,1==WB1 ONLY

3        , number of lines to skip in the header for the water level data file

0                , past water balance PWB file: Yes==1, No==0

"qwb.npt"        , Previous Water Balance file name

The explanations of each line are shown in the example file above. Keep in mind that if performing over multiple waterbodies, all waterbodies must have the same grid (i.e., ELBOT and vertical spacing must be the same). Currently all flow correction is given to only one water balance file. A later option will be having multiple WB files, so that one can distribute them across several waterbodies.

#### Output File: “WatBal_Errors.opt”

If there are errors, any errors will be shown in this text file. If no errors, then this file will not be written to the disk.

An example of the error file output is shown below:

Could not open simulated elevations file tsr_8_seg39.opt

#### Output File: “el_stats.opt”

This output file displays model error statistics of the water level and average flow rate in the qwb output file. Typical outputs from this program are shown below:

N   Mean Error  Absolute ME    RMS Error

238         0.00         0.00         0.00

Average water balance flow correction = .00 m^3/s for period covering Julian day 1.04 to 239.94

#### Output File from water balance

The main output file from the water balance utility is a time series of flows necessary to match the water level.

A typical output file is shown below:

Computed flow to complete water balance

1       1

JDAY     QWB

1.040    0.00

2.000   24.34

3.000    2.33

4.000    2.34

5.000    3.72

6.000    2.33

7.000    4.67

8.000    2.34

9.000    3.71

10.000    3.29

11.000    4.66

12.000    2.34

13.000    3.69

14.000    2.34

15.000    3.32

16.000    3.31

17.000    3.49

#### How to Run the Console Water Balance Utility

The water balance utility is run by executing the exe file for 32 (WBconsole32.exe) or 64 bit (WBconsole64.exe) Windows operating systems. One also needs the input file WatBal.npt in the directory of the executable.

# Control File Converter for CE-QUAL-W2

This section outlines the steps required to convert a legacy application control file, w2_con.npt, and its companion graph.npt file into the Excel version of the control file. This is a relatively simple process that involves these steps:

- Copy ConverterControlFile.exe into the directory of a legacy application, or copy the three files read by the program (w2_con.npt, graph.npt, and the bathymetry file) into a directory with the executable. After execution, the following files are written to the directory:

  - w2_con.csv – this is a csv file format of the control file. This will then need to be copied into a w2_con.xlsm template.

  - A csv form of the bathymetry file (if the bathymetry file was not already in csv format). The output bathymetry file is named bthX.csv, where X is the waterbody number. This new format is much easier for editing and analysis than the older file format. The file name in the w2_con.csv is also changed to bthX.csv.

- Copy w2_con.csv into an existing w2_con.xlsm from another application (you can use one from the W2 Model Examples) by following these steps:

  - Open w2_con.xlsm from a problem from the CE-QUAL-W2 example problems.

Figure 3. An example of an existing w2_con.xlsm file. Columns A and B are not used in the control file, and the w2_con.csv will be pasted into column C1, not A1.

  - The easiest option is to open the file w2_con.csv from the converter utility in a text editor, like Notepad++. Select the entire file (CTRL-A, CTRL-C), then paste it into cell C1 in w2_con.xlsm (CTRL-V). Select what was pasted and use the DATA/Text to Columns function in Excel to convert the text to columns. Choose ‘Delimited’ and check comma as a column delimiter. It should then populate the columns correctly.

    - Alternately, one can open w2_con.csv in Excel and select the columns and rows with data. Do not select the entire sheet since you cannot copy and paste these into Cell C1 in w2_con.xlsm. Select the rows and columns necessary to select all the data. Be careful you select all the columns – especially for the SOD specification per segment. You will need to select as many columns as the # of segments (IMX).

Figure 4. An example of the w2_con.csv file from the converter utility.

    - Copy the selected cells from w2_con.csv and paste to w2_con.xlsm cell C1 using Paste Values and Number Formatting so that colors and other formatting is preserved in the original document.

  - The next step is to adjust rows for the number of constituents. The template file you used, w2_con.xlsm, had an assumed number of water quality constituents that may be different from the number you use. Go to approximately row 384 as shown below and delete or add cells only in columns A and B to match the specified number in the formula. So, if you need to add cells, then select the correct number of cells in column A and B, right-click, and choose Insert, then shift cells down. If you need to delete cells, then select the correct number of cells in column A and B, right-click, choose Delete, then select move cells up. Now the information in column A and B should line up with the contents of Column C.

Figure 5. Location in Excel template where the constituent order is shown. In this example, the required number of constituents is 30. This is a formula computed in the Excel sheet. Make sure that you have that same number in the list that follows. Another tab in the Excel file offers guidance regarding the required order, based on your dimensioning of number of algal groups, BOD groups, macrophyte groups, periphyton groups, suspended solids groups, and generic constituent groups. Also note that in column A, guidance is given on setting the number of columns needed in this section.

  - If you have more than five structures, five epiphyton/periphyton groups, five algae groups, five macrophyte groups, or five zooplankton groups, you will need to add additional rows where necessary only in columns A and B. You can search in Column A for “increase # of rows” for where these areas are located.

    - For structures, look at rows 136-165 – there are notes in Col A and B describing where to add rows if more than five structures.

Figure 6. Location in Excel file where the number of rows needs to be increased in case there are more than five structures.

    - For more than five epiphyton/periphyton groups, look in the epiphyton section (search for epiphyton).

Figure 7. Location in Excel file where the number of rows needs to be increased in case there are more than five epiphyton/periphyton groups.

    - For more than five algae and five zooplankton groups, look in the zooplankton section (search for zooplankton).

Figure 8. Location in Excel file where the number of rows needs to be increased in case there are more than 5 zooplankton groups.

    - For more than five macrophyte groups, look in the macrophyte section (search for macrophyte).

Figure 9. Location in Excel file where the number of rows needs to be increased in case there are more than five macrophyte groups.

  - You will need to put quotes around the TITLE field, HNAME field, CNAME field, and CDNAME field. This can be done by setting up a formula using the existing cells as ‘=char(34)&[CELLREF]&char(34),’ where CELLREF is the cell reference number of the original text. Then paste the values with the double quotes to the expected locations.

  - The w2_con.xlsm file should now be working. As edits are made in this file, you will push the button on the top of the file in Column B to export it to w2_con.csv, which is read in by the W2 model. The preprocessor will ‘yell’ at you in case there is an error in the set up.

# GUI Interface for CE-QUAL-W2 for Versions 3.7-4.20

Figure 10. About W2 Control GUI preprocessor.

## Introduction

This section describes the GUI interface that is released with CE-QUAL-W2. This works with the old version of the control file, w2_con.npt, but is not expected to be coded to work with the new Excel version of the control file (w2_con.csv). The manual describes the uses of the GUI for editing the control file (w2_con.npt) and the bathymetry file (user-defined filename, typically called bth.npt or bth.csv). Please note that the model files can be edited using a text editor or in Excel, and the GUI is therefore not required to run the model. The GUI is often very helpful, though, for the new CE-QUAL-W2 user, as is the Excel version of the control file. The bathymetry editor can be very useful in viewing the model grid and making changes to the grid (e.g., grid refinement). The bathymetry editor though does not assist in setting up the bathymetry. The model user must have already developed the bathymetry file using other programs. The bathymetry editor only edits an existing bathymetry file. The bathymetry is often developed using GIS software or programs, such as SURFER or other 3-D contour plotting software packages.

## Installing the GUI Interface

After downloading the GUI interface setup files and unzipping them, from Windows Explorer double-click on ‘setup.exe’ to guide you in the set-up process.

## Opening the GUI Interface

By double-clicking on the W2 icon from the Desktop or from Windows Explorer, the user is presented with the following screen:

Figure 11. Dialog box menu.

This interface is meant to be simple and uncomplicated.

Figure 12. Menu of w2control.

Menus

The user has 3 menus: File, Window, and Help. The File menu controls which control file to open and to save. The user can only open a CE-QUAL-W2 control file – such as w2_con.npt. Opening any other file type will result in an error. The File save button and file open buttons and menu items only affect the control file and the associated files (bathymetry and graph files). Note that when you save a file, you overwrite the original file with which you started. The GUI program though copies the original output file to a similar name but with a number designator. This allows the user always to go back and track changes made over time to the control file or bathymetry files.

### Button Bar

The buttons become active as the user opens the files that affect the buttons. For example, once the bathymetry file is opened, the 4 grid viewing options are then displayed (see more information below). The open file button opens the w2_con.npt file or another file that is in the format of the control file. The save button becomes active once a file is opened and changes are made. The print button prints the current screen (I would not use this since it is not a very useful print out. It is much more convenient to just print the text file directly from Notepad or other text editor.) The next buttons are described below.

### Editing the Control File

Figure 13. Example of w2control menu.

Once you have chosen a control file to open (w2_con.npt), open it and adjust the window so that the window options are visible. If the file does not open, there may be serious formatting errors in the file, or you may have not chosen a CE-QUAL-W2 control file. Check the file for errors using the CE-QUAL-W2 preprocessor (a separate program released with W2).

You will notice that the BTH button is now active, meaning that you can also open the bathymetry file. The left pane now has 7 headings – all subheadings that can be shown by double-clicking on the main heading. For example, several of the subheadings have been expanded, as shown to your right.

By double-clicking on a subheading, the user can edit the information in a cell. Information about each model parameter is shown in Appendix C in the User Manual.

Once you have edited the cells (either by changing a number or changing a menu parameter), two buttons appear at the bottom of the page, as shown below.

Figure 14. Dialog box example.

The Apply button is used to set the parameters of the table. For the example above, the # of epiphyton groups is ‘1”. If one wanted to change this to “2”. One would enter 2 in the cell for NEP and then press “Apply”. The table will be re-drawn as shown below.

Figure 15. Dialog box example.

After making changes, the OK button MUST be pressed; however, this does not SAVE the file to the hard disk: The User must also then press the SAVE button on the button bar above (or the menu FILE/SAVE file) to save these changes to the disk. The save button will be highlighted after a selection is changed.

### Editing the Bathymetry File

After opening the control file, the bathymetry file can be opened by clicking the BTH button. Note that the bathymetry file name is under the ‘INPUT-OUTPUT FILE NAMES’ Menu and the submenu ‘WATERBODY INPUT FILE NAMES’. At this point there are four views that become active: Side View, Top View, Data View, End View.

The buttons on the top allow one to toggle between the different bathymetric views. By expanding the Top View, one can enlarge the view by holding down the left mouse button and scrolling from right to left (see example below). To decrease the image size, hold down the left mouse button and scroll from right to left.

To change the active segment and layer, place the mouse cursor in the Data View at a segment and layer of interest and click the ‘Refresh View’ button. This affects which segment is shown in the End View and which segment is highlighted in the Side View.

Figure 16. Bathymetry dialog box example.

To edit the bathymetry, click on the variable and change its value. Then click the ‘OK’ button to accept these changes. Pressing the SAVE button saves all the user files.

Figure 17. Segments of a model dialog box.

### Merging or Splitting Segments or Layers

In order to perform this operation, open the Data View and select the range of layers and/or segments to merge or split by highlighting them with the mouse. You will notice in the left hand corner a box that shows which segments and layers have been selected.

In the example below, layers 5 through 14 and segments 2 and 3 have been selected. You will also notice that the layer and segment merge/split buttons are now active. To merge or split segments, click on the Merge/Split segment button. A small dialog box will come up with options for merging or splitting the segments (see below). You then have the option to merge the two segments or split them into many more segments. The control file (w2_con.npt) will automatically adjust segment numbers when using this option. But keep in mind that the following files will need to be edited to adjust for changes in the number of model segments: the wind sheltering coefficient file (wsc.npt) and the shading file (shade.npt).

A similar process is followed for merging or splitting layers, as illustrated below.

Figure 18. Splitting or merging layers example.

Note that in the bottom left corner of the above screen shot, the layers and segments that were selected are shown.

Figure 19. Merge/split dialog box.

## Running the Preprocessor within the GUI

The preprocessor which checks all input files can be executed from within the GUI by clicking on the CHECK button. At which point a dialogue box will pop up with information about running the preprocessor. The preprocessor executable must be in the root directory with the input files.

Figure 20. Running dialog box within w2control.

## A Note on Saving Files

The GUI interface makes a backup copy of each file opened during a session. It will add an integer number to the files opened. This integer is based on whether previous backup files have been created or not. For example, the first time the GUI opens the files w2_con.npt, graph.npt, and bth.npt, the program will write the following as backup copies of the unaltered original files: w2_con.npt2, graph.npt2, and bth.npt2. The next time the program opens these files, the files w2_con.npt3, graph.npt3, and bth.npt3 will be created (if you have already deleted the *.*2 files from your hard disk, the GUI will again use the integer ‘2’ rather than ‘3’).

## Typical Errors with the GUI

Sometimes the GUI cannot read a control file due to the format of the text. UNIX/Linux systems often use a line-feed at the end of a line, while Windows text files include a carriage-return and a line-feed. You must convert the file to a Windows text file for the GUI to work properly. In a text editor you can view all the characters and verify you have the proper Windows format if the GUI is having problems reading the file. Many text editors automatically convert files from one type to the other (such as the text editor ‘Notepad ++’).

We recommend using the W2 preprocessor to catch errors in the set-up files first, before using the GUI. The W2 preprocessor will not catch the line-feed error noted above, however, and will read the files correctly whether in UNIX/Linux or Windows format.

## Command Line Processing

The GUI can also be controlled by command line passing of the working directory and file. In a batch program, or from the command line in a DOS box, you can execute the GUI as follows:

C:\scott\research\corps of engineers\tomcole\w2code\GUI36\w2control\w2control4.exe C:\scott\w2workshop\2009 workshop\waterqual\problem1\w2_con.npt

The first string in quotes executes the GUI (this shows the path to the GUI – your path to the GUI will be different). The command line argument is NOT in quotes. This program was developed in VB6 and does not take quotes around the command line. Note that this is different than the FORTRAN command line argument. Therefore the above command will open the GUI and load the control file automatically from the directory C:\scott\w2workshop\2009 workshop\waterqual\problem1.

## Using the GUI W2Control on Touchscreen Laptops and Monitors

The software, W2Control, is a GUI preprocessor for the W2 model. It works fine on non-touch enabled monitors. But for touch screen monitors, like many of the latest laptops, the opening “treeview” menu does not work because of a software incompatibility with VB6, the source code. W2Control can, however, be used on a touch screen laptop by following these steps:

- Go to Services by typing ‘Services’ in the command line or Cortana line.

- In the list choose Touch keyboard and Handwriting Panel Service.

- Right click your mouse and choose Properties and change ‘Automatic’ to ‘Disabled’.

- Then click STOP in the Service Status to stop the service.

- Click APPLY, and the W2Control will work as expected.

# Post-Processor for CE-QUAL-W2 by DSI, Inc.

Paul Craig at DSI, Inc. has generously provided a software system for evaluating model results. There is a short user’s manual produced by DSI that documents the software utility, and this is a separate PDF file download. This post-processor currently works for Version 3.7 to the current version.

Figure 21. Splash screen for W2_Post.

# Excel Macro Utility for CE-QUAL-W2

This Excel Utility was written by Jeffrey Gregory, P.E., Civil Engineer, USACE, Nashville District, and updated by Scott Wells, Portland State University.

The Excel macro, w2tools_examples.xlsm, can be used to develop and write out input files for the CE-QUAL-W2 model. This can be especially useful if the model user develops an Excel workbook with separate tabs for the different input files for the model.

For example, consider the input file for wind sheltering using the new free format (or csv) formatting option.  You develop a workbook for this as shown below. The coloring in the workbook is optional and is simply an aid in developing your work.

Now open w2tools_examples2.xlsm and select ‘Enables Macros’. Once this file is opened, you will notice that the sheet now has a header termed Add-Ins.  By clicking this header, the user obtains the options shown below: Modify Selection, Export Selection, or Run EXE.

The user then highlights the portion of the file to export or modify, and selects Export Selection and Any selection to csv format. The file is written as a free-format csv file with the file name based on the worksheet name, in this case “wsc.np.”  Many other export file formats can be used that are compatible with CE-QUAL-W2. Also, the worksheets in w2tools_examples.xlsm generally show how to use the different macro features. These sheets are only used as examples of how to use of the Add-In macros.

This toolset also allows you to modify a selection by multiplying, adding, interpolating data gaps, rounding, and transposing. First select the area of the spreadsheet that you wish to modify, then select Add-Ins, Modify Selection, and one of the tools, such as Interpolate (either the X column or the Y column of a 2-column selection).

# W2 V4.2 Bug Fixes, Enhancements, and User Manual Changes

Table 1 contains a list of model bug fixes since the prior release of CE-QUAL-W2 Version 4.1. Note that some of these fixes were documented before the first release of Version 4.2.

Table 1. List of bug fixes and enhancement code changes since CE-QUAL-W2 Version 4.1. Note that shaded areas toggle between release versions.

| # | Code: W2 or PREW2 or GUI | Fix or Enhancement Type | Description of Bug/Enhancement | Date Bug Fixed or Enhancement added |
| --- | --- | --- | --- | --- |
| 1 | W2 | Particle tracking | Fixed a bug when a particle was in the “air” above the water surface when in a river a ‘deep slot’ was added to the river bathymetry to keep it hydrated. Also, fixed a reflection error off the surface for vertical movement of the particle. | 12/26/2018 |
| 2 | W2 | Particle tracking | For file output for finalparticle.csv, adjusted header so that it would be general for any number of monitoring stations. Previously the header was hard-wired to write only 3 monitoring locations in the title. This does not affect prior model output information only the title header. | 1/3/2019 |
| 3 | PREW2 | Pipe invert | The preprocessor gave a ‘false’ error for an invert elevation being below the grid in some rare cases. This has been fixed. The model was not affected. | 1/21/2019 |
| 4 | W2 | TN | There was a bug in the calculation of TN when large concentrations of zooplankton were present. If zooplankton were not simulated or they were a small population, there would be no impact of this TN bug. Bug fixed. | 1/29/2019 |
| 5 | W2 | Epiphyton periphyton burial and conversion to LPOM and sediments | The User Manual showed that the epiphyton parameter EB, burial rate, was in units of m/day. The model implemented this, and also computed the cell layer burial rate as EB*EC (concentration of biomass per cell volume) * Surface area/Volume of a cell or EB*EC/H where H is the layer thickness. In reality though it should be the thickness of the periphyton layer. But that layer thickness is not predicted by the model. Hence, the decision was to change the burial rate to units of 1/day such that the loss of epiphyton by burial would follow a first order decay process (similar to the first order sediment model) as EB*EC. Hence, this burial rate is no longer a function of the grid. Also, there was the conversion of organic matter and nutrients at death to the sediment P, N and C compartments whereas they should go directly to LPOM first. Hence only burial goes directly into the sediment compartments. | 3/5/2019 |
| 6 | W2 | Output files | Code was written to print the initial condition in output files at time t=0 at the start of the simulation. | 3/5/2019 |
| 7 | W2 | Restart with Sediment Diagenesis | Prior to this date, the restart option did not work with sediment diagenesis turned ON. This enhancement has been coded and debugged and is now working. | 7/1/2019 |
| 8 | W2 | Dynamic loading | This is a new feature in CE-QUAL-W2 and applies to linear waterbodies in series. For example, if one has a river system with 9 waterbodies, the model user can break the model into 9 separate models and run them all in parallel where the downstream model dynamically updates its boundary conditions as the upstream model moves forward in time. This has resulted in significant time savings for model runs (90%). | 7/1/2019 |
| 9 | W2 | Sediment Diagenesis | Output files were updated to write csv files that can be plotted easily in Excel. The older output files for sediment diagenesis were hard to graph formatted text output files. All internal writes for warnings are now written to the W2 warning file, w2.wrn, and all errors are written to the w2.err output file. | 7/1/2019 |
| 10 | W2 | Sediment Diagenesis | Bug fixes were made to the scour section of the model. The code had several errors based on the documentation from CEMA. These have been fixed and synchronized with the description in the CEMA sediment diagenesis report. Thanks to Lindsay Bearup at the USBR! Also, the scour model was not set-up for both Chezy and Manning’s friction factors. This code was generalized. | 7/1/2019 |
| 11 | W2 | Output files | Output files were not initialized properly for w2.wrn during a restart. This was fixed allowing the w2.wrn file to be appended to rather than over-written. | 7/1/2019 |
| 12 | W2 | Code updates | Pointers were allocated memory in the ALLOCATE command. This was non-standard Fortran usage and was fixed. There were extra calls to Kinetic_Rates and Temperature_Rates subroutines that were unnecessary during layer addition and in the INIT subroutine. | 7/1/2019 |
| 13 | W2 | TDG | A new algorithm for TDG production at spillways, SYSTDG, was implemented in Version 4.2. This is a more detailed TDG correlation equation compared with the existing algorithms in Version 4.1 and before. The primary application was for the Columbia and Snake River dams. A separate technical memo is included with the User Manual on this new process. | 7/1/2019 |
| 14 | W2 | opt to csv | Flux output files were changed from a file type opt to csv to facilitate graphing in Excel. Structure output files for the auto port selection were changed from opt to csv. | 7/1/2019 |
| 15 | W2 | Contour output | The Tecplot and regular Contour output was not working for cases when branches went inactive. This has been fixed. | 7/1/2019 |
| 16 | W2 | Sediment diagenesis | The input file for sediment diagenesis has been changed from w2_CEMA_input.npt to w2_diagenesis.npt for clarity of naming files. All output files have also been changed to include ‘Diagenesis’ as the first word and eliminating ‘CEMA’. For example, the file CEMALogFile.opt was changed to DiagenesisLogFile.opt. | 7/1/2019 |
| 17 | W2 | Withdrawal | In the withdrawal subroutine, the kt index needed to be defined twice. Below is an example of the fix in two places: (1) ELR = SINA(JB)*DLX(ID)*0.5 ! CB 10/14/11 WSEL = ELWS(ID)-ELR ! CB 10/14/11 kt=ktwb(jw) ! cb 07/24/19 CALL DOWNSTREAM_WITHDRAWAL_ESTIMATE(JSTSPLT(J,1),TEMPTOP,ESTR(JSTSPLT(J,1),TSPLTJB(J))) (2) WSEL = ELWS(IWD(JWD))-ELR ! CB 10/14/11 I=IWD(JWD) kt=ktwb(jjw) ! cb 07/24/19 CALL LATERAL_WITHDRAWAL_ESTIMATE(JSTSPLT(J,1),TEMPTOP,EWD(JSTSPLT(J,1))) | 8/1/2019 |
| 18 | W2 | Branch inactive | The BR_INACTIVE flag that skips output in a branch that was inactive was dimensioned with the wrong variable for the CPL output (Tecplot output). This has been corrected. | 8/27/2019 |
| 19 | W2 | Tecplot Branch | There is a new input file to specify what branches the user wants for Tecplot output, called TecplotBr.csv. This allows the user to only pick some of the branches rather than all of them for Tecplot output. | 8/27/2019 |
| 20 | Prew2 | False warning | A false warning was sometimes activated for pipes for DYNPIPE when in fact was ON or OFF as required. This was fixed. Below is final code fix: IF ((DYNPIPE(JP) /= ' ON') .AND. (DYNPIPE(JP) /= ' OFF')) THEN CALL ERRORS WRITE (ERR,FMTI) 'Pipe DYNAMIC PIPE control [DYNPIPE='//DYNPIPE(JP)(4:8)//'] must be either "ON" or "OFF" for pipe | 8/27/2019 |
| 21 | W2 | str_brX.opt | Structure output file, str_brX.opt, for automatic port selection was changed from a text delimited *.opt file to a comma delimited *.csv file | 8/28/2019 |
| 22 | W2 | Auto Port Selection | The split algorithm for the auto port selection based on temperature now can read an input file of dynamic temperatures rather than relying on multiple rules with different fixed temperatures. | 8/30/2019 |
| 23 | W2 | Tecplot | Tecplot output under the CPL output was not set up for restart properly. Tecplot output files now should work for cases where there are model restarts. | 9/4/2019 |
| 24 | W2 | Water age | The code below for water age correction as a result of evaporation (a very minor adjustment) was incorrect: Old Code: IF(AERATEC == " ON")CALL AERATEMASS IF(EVAPORATION(JW) .AND. WATER_AGE_ACTIVE)THEN ! CORRECT WATER AGE FOR EVAPORATION SR 7/27/2017 DO I=IU,ID JC=NGCS+JG_AGE-1 CSSB(KT,I,JC)=CSSB(KT,I,JC)-EV(I)*CG(KT,I,JC) ENDDO ENDIF New Code: IF(AERATEC == " ON")CALL AERATEMASS IF(EVAPORATION(JW) .AND. WATER_AGE_ACTIVE)THEN ! CORRECT WATER AGE FOR EVAPORATION SR 7/27/2017 DO I=IU,ID JC=NGCS+JG_AGE-1 CSSB(KT,I,JC)=CSSB(KT,I,JC)-EV(I)*CG(KT,I,JG_AGE) ENDDO ENDIF | 10/17/2019 |
| 25 | W2 | Pipe algorithm | The pipe algorithm was revised to reduce issues with pipe algorithm instabilities. There is error trapping to reduce the prevalence of growing instabilities from a water surface slope that becomes unstable. | 11/1/2019 |
| 26 | W2 | W2 fixes | Small fixes that do not affect the model user: Eliminated the unused variable PHISET and changed INTEGER*8 variables in the SYSTDG algorithm to INTEGER*4. Thanks to Stewart Rounds USGS for noting these items. | 11/13/2019 |
| 27 | W2 | Turbidity | Turbidity can be used through the sediment diagenesis model as a derived variable. This will shortly be moved to a derived constituent in W2 and taken out of the sediment diagenesis subroutine. There was a coding error in the turbidity routine that was fixed: !CellTSSValue = C1(K,SegNumI,6) !CellTSSValue = TOTSS(K,SegNumI) !SW 11/15/2019 C1(K,SegNumI,nturb) ! cb 2/18/13 !C1(K,SegNumI,6) = exp(CoeffA_Turb*log(CellTSSValue) + CoeffB_Turb) !C2(K,SegNumI,6) = exp(CoeffA_Turb*log(CellTSSValue) + CoeffB_Turb) !C1(K,SegNumI,nturb) = exp(CoeffA_Turb*log(CellTSSValue) + CoeffB_Turb) ! cb 2/18/13 !C2(K,SegNumI,nturb) = exp(CoeffA_Turb*log(CellTSSValue) + CoeffB_Turb) C1(K,SegNumI,nturb) = exp(CoeffA_Turb*log(TOTSS(K,SegNumI)) + CoeffB_Turb) ! cb 2/18/13 C2(K,SegNumI,nturb) = C1(K,SegNumI,nturb) !exp(CoeffA_Turb*log(CellTSSValue) + CoeffB_Turb) | 11/15/2019 |
| 28 | W2 | Examples and User Manual | The kinetic flux output order for DOEP and DOAR was reversed in a couple of the input files in the model example files (in w2_con.npt) and in one place in the User Manual. These were corrected. | 11/15/2019 |
| 29 | W2 | Branch Active | In the process of setting inflows to upstream inactive branches to branches that were still active, the following code was updated for complex, multi-waterbody systems: [Chris Berger] IF(BR_INACTIVE(JB))THEN ! CONVERT INFLOWS TO TRIBS SET TO THE CUS(1) LOCATION JTT=JTT+1 !ITR(JTT)=CUS(1) ! HARDWIRED TO FIRST BRANCH ITR(JTT)=CUS(jbdn(jw)) ! changed HARDWIRE TO JBDN BRANCH ! cb 11/20/19 QTR(JTT) = QIN(JB) … ! including tributary flows for inactive branches DO JW=1,NWB ! cb 11/20/19 KT = KTWB(JW) DO JB=BS(JW),BE(JW) IF(BR_INACTIVE(JB))then IU = CUS(JB) ID = DS(JB) IF (TRIBUTARIES) THEN DO JT=1,JTT !********** Inflow fractions IF (JB == JBTR(JT)) THEN I = cus(jbdn(jw)) ! placing tributary flows in upstream end of main branch QTRF(KT:KB(I),JT) = 0.0 KTTR(JT) = KT KBTR(JT) = KB(I) KTTR(JT) = MAX(KT,KTTR(JT)) KBTR(JT) = MIN(KB(I),KBTR(JT)) IF (KBTR(JT) < KTTR(JT)) KBTR(JT) = KTTR(JT) BHSUM = 0.0 DO K=KTTR(JT),KBTR(JT) BHSUM = BHSUM+BH2(K,I) END DO DO K=KTTR(JT),KBTR(JT) QTRF(K,JT) = BH2(K,I)/BHSUM END DO DO K=KTTR(JT),KBTR(JT) QSS(K,I) = QSS(K,I)+QTR(JT)*QTRF(K,JT) END DO END IF END DO END IF end if end do end do | 11/22/2019 |
| 30 | W2 | WRN output | The warning output format was improved when there were volume balance errors – thanks to Stewart Rounds, who defies retirement to provide model improvements!! In the subroutine, balances.f90, the new code is shown below: IF (VOLUME_WARNING) THEN WRITE(WRN,'(A,F0.4,/A,I0,A,I0,A,I0,3(:/A,E15.8,A))') & 'COMPUTATIONAL WARNING AT JULIAN DAY = ',JDAY, & 'WATERBODY=', JW, ', BRANCH=', JB, ', KT=', KT, & 'SPATIAL CHANGE =', VOLSBR(JB), ' M^3', & 'TEMPORAL CHANGE =', VOLTBR(JB), ' M^3', & 'VOLUME ERROR =', VOLSBR(JB)-VOLTBR(JB), ' M^3' !SR 11/16/19 WRITE (WRN,*) 'LAYER CHANGE:', LAYERCHANGE(JW) WRITE (WRN,*) 'SZ', SZ(CUS(JB):DS(JB)), 'Z', Z(CUS(JB):DS(JB)), 'H2KT', H2(KT,CUS(JB):DS(JB)), & 'H1KT', H1(KT,CUS(JB):DS(JB)), 'WSE', ELWS(CUS(JB):DS(JB)), 'Q', Q(CUS(JB):DS(JB)), & 'QC', QC(CUS(JB):DS(JB)), 'T1', T1(KT,CUS(JB):DS(JB)), 'T2', T2(KT,CUS(JB):DS(JB)), & 'SUKT', SU(KT,CUS(JB):DS(JB)), 'UKT', U(KT,CUS(JB):DS(JB)), 'QIN', QINSUM(JB), & 'QTR', QTR, 'QWD', QWD !SR 11/16/19 WARNING_OPEN = .TRUE. | 12/2/2019 |
| 31 | W2 | W2 Error Dump | The W2ErrorDump.opt file output format was changed to be easier to analyze. The file was changed to W2ErrorDump.csv to facilitate opening in Excel and for general plotting. The new code is from Stewart Rounds and is found in endsimulation.f90. Old code: !OPEN(W2ERR,FILE='W2Errordump.opt',status='unknown') !WRITE(w2err,*)'JDAY',jday,'SZ',sz,'Z',z,'H2KT',h2(kt,1:imx),'H1KT',h1(kt,1:imx),'BHR1',bhr1(kt,1:imx),'BHR2',bhr2(kt,1:imx),'WSE',elws,'Q',q,'QC',qc,'QERR',qerr,'T1',t1(kt,1:imx),'T2',t2(kt,1:imx),'SUKT',su(kt,1:imx),& ! 'UKT',u(kt,1:imx),'QIN',qin,'QTR',qtr,'QWD',qwd New Code: OPEN (W2ERR,FILE='W2Errordump.csv',status='unknown') WRITE (W2ERR,*) 'JDAY = ', JDAY WRITE (W2ERR,'(A,1000(",",F0.6))') 'QIN:', (QIN(J),J=1,NBR) WRITE (W2ERR,'(A,1000(",",F0.6))') 'QTR:', (QTR(J),J=1,NTRT) WRITE (W2ERR,'(A,1000(",",F0.6))') 'QDT:', (QDTR(J),J=1,NBR) WRITE (W2ERR,'(A,1000(",",F0.6))') 'QWD:', (QWD(J),J=1,NWDT) WRITE (W2ERR,'(/A)') 'SEG,BRANCH,KT,WSE,SZ,Z,Q,QC,QERR,H2KT,H1KT,BHR1,BHR2,T1,T2,SUKT,UKT' DO JW=1,NWB KT = KTWB(JW) DO JB=BS(JW),BE(JW) DO I=US(JB)-1,DS(JB)+1 WRITE (W2ERR,'(I0,",",I0,",",I0,14(",",F0.6))') I, JB, KT, ELWS(I), SZ(I), Z(I), Q(I), QC(I), QERR(I), H2(KT,I), H1(KT,I), BHR1(KT,I), BHR2(KT,I), T1(KT,I), T2(KT,I), SU(KT,I), U(KT,I) END DO END DO END DO | 12/30/2019 |
| 32 | W2 | Multiple Waterbody | Stewart Rounds improved on the code presented in Version 4.2. Instead of writing and reading restart files, the code waited for the updates before proceeding. This led to code changes in many subroutines. Also, the downstream model can now wait for input from multiple upstream files rather than just one. This update is described in the User Manual and is part of the Version 4.2.1 model update because of a new input file format for the multiple waterbody input file. | 1/2/2020 |
| 33 | PREW2 | Preprocessor | A new feature was added to the preprocessor – a new windows pops up with errors if they are present. This avoids having to open the pre.err file in a word processor in case there were errors. | 3/15/2020 |
| 34 | W2 | Pump | Added features were added to the pump algorithm. This allows for the flow through a pump to be controlled by the downstream water level, rather than just the upstream level. If this is the case, then a negative downstream segment number is used. This also changes the logic of ELON and ELOFF. This has been updated in the User Manual also. | 3/15/2020 |
| 35 | W2/PREW2 | Control file | A new control file option was created in Excel using a comma delimited control file, w2_con.csv. This control file is a lot simpler to use than the text file w2_con.npt because (1) strict formatting by spaces is not required, (2) easier to cut and paste large sections withing Excel, (3) variable explanations are available in Column A, (4) the graph.npt file is no longer required and is incorporated into the Excel based input file, and (5) there is a one-button function to write the input file into csv format for reading into the W2 code. This file also has many other advantages in setting up output and more intuitive variable placement (i.e., the order of the old w2_con.npt is not strictly followed.) The only negative is that it breaks the GUI W2_Control, which is not necessary, other than its nice visualization bathymetry tools. But many of those are in the w2Tools post-processor. There is currently no converter from the old format to the new one, but the model executable and preprocessor work with either control file. | 4/16/2020 |
| 36 | W2 | TSR | TSR model output headers were improved. The P, N, and Light limitation for algae headers (APLIM, ANLIM, ALLIM) now show the algae group number rather than a generic header, such as PLIM_ALG1. | 5/3/2020 |
| 37 | Converter | Control file | A control file converter utility and description was added to the download package. This utility converts the control file (w2_con.npt) to a format using Excel. | 5/22/2020 |
| 39 | PreW2 | More checks | Several additional model checks were added to the preprocessor to improve its skill set. | 5/22/2020 6/15/2020 |
| 40 | W2 | Tecplot output | A new input file Tecplotbr.csv is now used to specify which branches are output to the Tecplot contour map. Before, all model branch information was output to the Tecplot file. Now the user can control this more precisely to eliminate a lot of post-processing of the data file. This is described in the User Manual. | 5/22/2020 |
| 41 | W2 | CO2 in atmosphere | As the basis for Version 4.2.2, the CO2 gas saturation was redone. See User Manual Part 2 under TIC. | 8/10/2020 |
| 42 | Converter |  | Fixed a bug in writing out the csv format for the control file in case there were more than one withdrawal. | 8/27/2020 |
| 43 | W2 | SPR ONV | Whenever the SPR was set equal to ONV for a w2_con.csv file, this output was not implemented correctly. For the input file w2_con.npt this worked as expected. | 10/8/2020 |
| 44 | Converter |  | Fixed a bug when NTR was exactly 9 (this bug also affected cases where NWB=9 or NBR=9). Also fixed some of the output formatting. | 11/27/2020 |
| 45 | W2 | Pipe | A dimensioning bug was found when there were no pipes. This did not affect model results with the executable, but did show up using the debugger in the Intel environment. This has been fixed. Many thanks to Jun Ma for noticing that!! | 12/20/2020 |
| 46 | W2/Pre/Converter | Csv input | When converting to the Excel macro utility for the control file and when modeling a system with no tributaries, the columns read for distributed inflows and precipitation may have been read incorrectly. This was adjusted by fixing the converter utility, the preprocessor, and W2 code. | 2/17/2021 |
| 47 | W2 | Kinetic Flux | When there are layer subtractions, the Kinetic flux in the surface layer terms cannot account for the mass. This did not affect the mass balance of the constituents, just the flux variables used for output. Many thanks for Taylor Adams of Hydros for finding that error. In layeraddsubtract.f90: KF(KT,I,KFCN(1:NAF(JW),JW)) = (KF(KT-1,I,KFCN(1:NAF(JW),JW))*VOL(KT-1,I)+ KF(KT,I,KFCN(1:NAF(JW),JW))*VOL(KT,I))/(VOL(KT-1,I)+VOL(KT,I)) ! SW Fix suggested by Taylor Adams Hydros 25Feb2021 ! KF is in units of g/m3/s KFS(KT,I,KFCN(1:NAF(JW),JW)) = KFS(KT-1,I,KFCN(1:NAF(JW),JW))+KFS(KT,I,KFCN(1:NAF(JW),JW)) ! SW Fix suggested by Taylor Adams Hydros 25Feb2021 ! KFS is in units of g KFS=KF*VOL*DT C1(KT-1,I,CN(1:NAC)) = 0.0 | 2/25/2021 |

# W2 Known Code Limitations

The following list shows known bugs and issues with the current release of the code:

| # | Item | Description |
| --- | --- | --- |
| 1 | Water levels in a bowl | If water levels decrease in a waterbody shaped like a bowl, the removal of model layers as the water level decreases will cause the model to bomb if an upstream segment dries up. |
| 2 | Pipes under high head | The pipes algorithm does not handle high-head, high-speed, dynamic flow conditions well when in a pipe, due to numerical instability. |
| 3 | Time step limitation in a complex system model | The time step for stability in a system model is governed by the lowest time step for numerical stability. If you have a very dynamic river with several reservoirs, the time step for the river will control. This can result in very long run times. One can still break apart the model and run the pieces separately using the WDOUT files to provide boundary conditions for downstream waterbodies. |
| 4 | Partitioning | The partitioning coefficient for sorption is currently constant for all organic and inorganic compartments. |
| 5 | Internal weir at a Dam segment | Putting an internal weir at a Dam segment does not affect the outflow from the selective withdrawal structure. One must limit selective withdrawal rather than using an internal weir at the dam segment, recalling that the internal weir works for the right-hand-face of a model layer. |
| 6 | W2 multiple file error check | If the model user accidentally enters duplicate file names for an input file, the w2 executable will bomb because it will try to read the file in more than once. The first use of the file will lock its availability for the second instance. The W2 error message that comes on the screen (traceback error) should mention the file name that has problems. The W2 preprocessor should catch this potential error. |
| 7 | Raising level of spillway/weir above grid | The preprocessor will say there is an error if the user raises the weir, spillway, gate, water level control or any other hydraulic element above the current top-of-the-grid. The w2 code will still run properly though. But more correctly, the model user should increase the DZ of the upper-most layer to a value that would eliminate this problem. Keep in mind that the segment widths from the top layer then extend upward at that same width. |
| 8 | Internal weirs | The internal weir algorithm does not work when all vertical layers of a segment are blocked by the weir. |
| 9 | Multiple dams into one downstream reach | Currently, the code will allow one dam inflow to a downstream branch by a user-specified outflow file. The code, however, does allow multiple dams inflowing to a common downstream branch, if the outflow is specified as a hydraulic structure. |
| 10 | Problems reading file in GUI or in W2 preprocessor of a W2 model | Sometimes the control file or bathymetry file or an input file cannot be read properly. This can be a result of the text editor used to produce the file or of file conversions that occur when transferring files from workstations running Linux or from email. There may be a problem with the end of line character in the file. For Windows files, the standard end of line is a carriage return followed by a line feed: <CR><LF>. For UNIX systems it is usually only a Line Feed <LF>. To convert this from a UNIX system to a Windows system text file, use Notepad++ (a free windows text editor), go to EDIT/EOL Conversion and select Windows. Another issue common in reading text files is that the editor adds ‘tabs’. All tabs must be converted to spaces for the file to be read properly. |

# Appendix A: Differences between CE-QUAL-W2 Versions

## Differences between Version 4.2.2 and Version 4.2.1

Version 4.2.2 has 2 new variables, [CO2PPM] and [CO2YRLY], that were added to the main control file, w2_con.npt and w2_con.csv. The old, fixed format card in w2_con.npt is shown below:

RSO DATE    RSOD    RSOD    RSOD    RSOD    RSOD    RSOD    RSOD    RSOD    RSOD

RSO FREQ    RSOF    RSOF    RSOF    RSOF    RSOF    RSOF    RSOF    RSOF    RSOF

CST COMP     CCC    LIMC     CUF

ON     OFF      10

CST ACTIVE   CAC

TDS           ON

Gen1          ON

ISS1          ON

ISS2          ON

The Version 4.2.2 file, w2_con.npt is shown below with the changes highlighted:

RSO DATE    RSOD    RSOD    RSOD    RSOD    RSOD    RSOD    RSOD    RSOD    RSOD

RSO FREQ    RSOF    RSOF    RSOF    RSOF    RSOF    RSOF    RSOF    RSOF    RSOF

CST COMP     CCC    LIMC     CUF  CO2PPM CO2YRLY

ON      ON      10    400.      ON

CST ACTIVE   CAC

TDS           ON

Gen1          ON

ISS1          ON

ISS2          ON

In the Excel macro version, the new variables are shown below:

## Differences between Version 4.2.1 and Version 4.2

Version 4.2.1 is file compatible except that the multiple_wb.npt format has changed. Also, a new input file, w2_con.csv, can be read by the model. This file is developed using the Excel file w2_con.xslm.

## Differences between Version 4.2 and Version 4.1

Version 4.2 is file compatible with Version 4.1. There are no changes in the main control file. There are new control files though that control new processes: (1) Multiple processor simulation for cascade of waterbodies (multiple_wb.npt), (2) SYSTDG Total Dissolved Gas algorithm for spillways (systdg.npt).

## Differences between Version 4.1 and Version 4.0

Version 4.1 is file compatible with Version 4.0. There are no changes in the main control file. There is only one new input file, ‘particle.csv’, that serves as an input file to the particle tracking algorithm.

## Differences between Version 4.0 and Version 3.72

Version 4 is file compatible with Version 3.72, even though there are new options in the main control file, w2_con.npt, and new input files whose presence or absence is detected by the model. For example, for ICEC control, the options now include ON, ONWB, and OFF, where ONWB is a new option. New input files include a file for sediment diagenesis, W2_CEMA_Input.npt, and a file for the dynamic alkalinity calculation, pH_buffering.npt.

Control file differences are in the Generic Constituent Section of the Code where new variables were added to the control file to allow for phot-degradation and the new N2 state variable for TDG:

GENERIC    CGQ10   CG0DK   CG1DK     CGS   CGLDK   CGKLF     CGS

CG 1     0.00000 0.00000 0.00000 0.00000 0.00000 1.03400 -1.0000     ! TDG

CG 2     0.00000 -1.0000 0.00000 0.00000 0.00000 0.00000 0.00000

CG 3     1.04000 0.00000 1.40000 0.00000 0.00000 0.00000 0.00000

## Differences between Version 3.72 and Version 3.71

These 2 codes are file compatible. Besides a few bug fixes since the last release of Version 3.71, Version 3.72 includes the USGS automatic port selection code. This can be activated by setting SELECTC=’USGS’ in the control file w2_con.npt. In Version 3.71, only ‘ON’ or ‘OFF’ were input variables for SELECTC. If one sets SELECTC=’USGS’, the format of the file w2_selective.npt is also changed from Version 3.71. Details of this and examples are provided in the User’s Manual and on-line.

## Differences between Version 3.71 and Version 3.7

There is only one change in the control file between Version 3.7 and 3.71. There is a new option for outlet structures – dynamic centerline elevation. In the control file, there is an ON/OFF option after declaring the # of structures for each branch:

EDDY VISC    AZC   AZSLC   AZMAX     FBC       E   ARODI STRCKLR BOUNDFR  TKECAL

WB 1         TKE     IMP 1.00000       3 9.53500 0.43100 0.00000 0.00000     IMP

N STRUC     NSTR DYNELEV

BR1           17      ON

BR2            0     OFF

BR3            0     OFF

STR INT    STRIC   STRIC   STRIC   STRIC   STRIC   STRIC   STRIC   STRIC   STRIC

If these fields are missing the model will assume that DYNELEV=OFF.

## Differences between Version 3.7 and Version 3.6

Although there are some cases where a Version 3.7 executable will successfully run Version 3.6 and Version 3.5 files, updates are required to the w2_con.npt file that need to be made. The preprocessor will catch these errors.

Control file changes: w2_con.npt

The main changes to the W2 control file are additional flags to turn ON/OFF new control file options and the addition of new state variables for water quality, BOD-N and BOD-P for each BOD group.

Below is a list of changes in the control file with the card image header for each line changed (highlighted options are new in V3.7). Descriptions of these new features are in the W2 User’s Manual.

- MISCELL

MISCELL     NDAY SELECTC HABTATC ENVIRPC AERATEC INITUWL

100     OFF      ON      ON      ON     OFF

Five new variables, SELECTC, HABITATC, ENVIRPC, AERATEC, and INITUWL, are new control variables that turn ON/OFF the use of automatic selective withdrawal, fish habitat volumes, environmental performance criteria, artificial aeration, and the initial water surface and velocity computations, respectively. If using an old Version 3.6 control file, all of these would default to ‘OFF’ if they were left blank. Also, the model preprocessor would flag these as missing variables.

- DLT CON

DLT CON      NDT  DLTMIN DLTINTR

1 1.00000     OFF

where DLTINTR is a control for interpolating the time step DLTMAX and DLTF rather than a step function.

- BRANCH G

BRANCH G      US      DS     UHS     DHS     UQB     DQB   NLMIN   SLOPE  SLOPEC

Br 1           2      59       0       0       0       0       1     0.0     0.0

where SLOPEC is the hydraulic equivalent slope for a river channel that affects the momentum equation.

- GATE WEIR

GATE WEIR   GTA1    GTB1    GTA2    GTB2  DYNVAR    GTIC

Gate1    1.00000 1.50000 1.00000 1.50000    FLOW      ON

where GTIC is an interpolation control for the specified DYNVAR for the GATE-WEIR.

- Dynamic pipe

PIPES       IUPI    IDPI    EUPI    EDPI     WPI   DLXPI     FPI  FMINPI  LATPIC DYNPIPE

Pi 1          24      28    28.0    27.0     0.5   230.0   0.065     0.1    DOWN      ON

where DYNPIPE controls whether the pipe is controlled by time series of an ON/OFF or partially open gate.

- Dynamic pump

PUMPS 1     IUPU    IDPU     EPU  STRTPU   ENDPU   EONPU  EOFFPU     QPU   WTHLC DYNPUMP

111       0    440.    1.00    366.   441.0   435.0     1.0    DOWN      ON

where DYNPUMP controls the EPU, EONPU, EOFFPU, and QPU over time by reading in a time series file.

- INIT CND

INIT CND   TEMPI    ICEI  WTYPEC   GRIDC

WB 1     -1.0000 0.00000   FRESH    RECT

where GRIDC controls whether the grid is interpreted as rectangular in depth or trapezoidal.

- CST ACTIVE  [Note that this change only appears if NBOD>0]

CST ACTIVE   CAC

TDS           ON

Gen1          ON

Gen2         OFF

Gen3         OFF

Gen4         OFF

Gen5         OFF

ISS1          ON

PO4           ON

NH4           ON

NO3           ON

DSI          OFF

PSI          OFF

FE           OFF

LDOM          ON

RDOM          ON

LPOM          ON

RPOM          ON

1CBOD         ON

2CBOD         ON

3CBOD         ON

4CBOD         ON

5CBOD         ON

6CBOD         ON

7CBOD         ON

8CBOD         ON

9CBOD         ON

10CBOD        ON

1CBODP        ON

2CBODP        ON

3CBODP        ON

4CBODP        ON

5CBODP        ON

6CBODP        ON

7CBODP        ON

8CBODP        ON

9CBODP        ON

10CBODP       ON

1CBODN        ON

2CBODN        ON

3CBODN        ON

4CBODN        ON

5CBODN        ON

6CBODN        ON

7CBODN        ON

8CBODN        ON

9CBODN        ON

10CBODN       ON

ALG1          ON

ALG2          ON

ALG3          ON

DO            ON

TIC           ON

ALK           ON

ZOO1         OFF

LDOM_P        ON

RDOM_P        ON

LPOM_P        ON

RPOM_P        ON

LDOM_N        ON

RDOM_N        ON

LPOM_N        ON

RPOM_N        ON

- CST ICON, CST PRIN, CIN CON,CTR CON, CDT CON and CPR CON

CST ICON   C2IWB   C2IWB   C2IWB   C2IWB   C2IWB   C2IWB   C2IWB   C2IWB   C2IWB

TDS          0.0

AGE          0.0

TRACER       0.0

COL1         0.0

Conduct      0.0

Chlorine     0.0

ISS1         0.0

PO4         0.03

NH4         0.01

NOx          0.3

DSi          0.0

PSi          0.0

TFe          0.0

LDOM         0.1

RDOM         0.1

LPOM         0.1

RPOM         0.1

1CBOD        0.0

2CBOD        0.0

3CBOD        0.0

4CBOD        0.0

5CBOD        0.0

6CBOD        0.0

7CBOD        0.0

8CBOD        0.0

9CBOD        0.0

10CBOD       0.0

1CBODP       0.0

2CBODP       0.0

3CBODP       0.0

4CBODP       0.0

5CBODP       0.0

6CBODP       0.0

7CBODP       0.0

8CBODP       0.0

9CBODP       0.0

10CBODP      0.0

1CBODN       0.0

2CBODN       0.0

3CBODN       0.0

4CBODN       0.0

5CBODN       0.0

6CBODN       0.0

7CBODN       0.0

8CBODN       0.0

9CBODN       0.0

10CBODN      0.0

ALG1         0.1

ALG2         0.1

ALG3         0.1

DO          12.0

TIC          5.0

ALK         19.8

ZOO1         0.0

LDOM_P    0.0005

RDOM_P    0.0005

LPOM_P    0.0005

RPOM_P    0.0005

LDOM_N    0.0080

RDOM_N    0.0080

LPOM_N    0.0080

RPOM_N    0.0080

CST PRIN  CPRWBC  CPRWBC  CPRWBC  CPRWBC  CPRWBC  CPRWBC  CPRWBC  CPRWBC  CPRWBC

TDS           ON

AGE           ON

TRACER        ON

COL1          ON

Conduct       ON

Chlorine      ON

ISS1          ON

PO4           ON

NH4           ON

NOx           ON

DSi          OFF

PSi          OFF

TFe          OFF

LDOM          ON

RDOM          ON

LPOM          ON

RPOM          ON

1CBOD         ON

2CBOD         ON

3CBOD         ON

4CBOD         ON

5CBOD         ON

6CBOD         ON

7CBOD         ON

8CBOD         ON

9CBOD         ON

10CBOD        ON

1CBODP        ON

2CBODP        ON

3CBODP        ON

4CBODP        ON

5CBODP        ON

6CBODP        ON

7CBODP        ON

8CBODP        ON

9CBODP        ON

10CBODP       ON

1CBODN        ON

2CBODN        ON

3CBODN        ON

4CBODN        ON

5CBODN        ON

6CBODN        ON

7CBODN        ON

8CBODN        ON

9CBODN        ON

10CBODN       ON

ALG1          ON

ALG2          ON

ALG3          ON

DO            ON

TIC           ON

ALK           ON

ZOO1         OFF

LDOM_P        ON

RDOM_P        ON

LPOM_P        ON

RPOM_P        ON

LDOM_N        ON

RDOM_N        ON

LPOM_N        ON

RPOM_N        ON

CIN CON   CINBRC  CINBRC  CINBRC  CINBRC  CINBRC  CINBRC  CINBRC  CINBRC  CINBRC

TDS           ON      ON

AGE          OFF     OFF

TRACER       OFF     OFF

COL1         OFF     OFF

Conduct       ON      ON

Chlorine     OFF     OFF

ISS1          ON      ON

PO4           ON      ON

NH4           ON      ON

NOx           ON      ON

DSi          OFF     OFF

PSi          OFF     OFF

TFe          OFF     OFF

LDOM          ON      ON

RDOM          ON      ON

LPOM          ON      ON

RPOM          ON      ON

1CBOD         ON      ON

2CBOD         ON      ON

3CBOD         ON      ON

4CBOD         ON      ON

5CBOD         ON      ON

6CBOD         ON      ON

7CBOD         ON      ON

8CBOD         ON      ON

9CBOD         ON      ON

10CBOD        ON      ON

1CBODP        ON      ON

2CBODP        ON      ON

3CBODP        ON      ON

4CBODP        ON      ON

5CBODP        ON      ON

6CBODP        ON      ON

7CBODP        ON      ON

8CBODP        ON      ON

9CBODP        ON      ON

10CBODP       ON      ON

1CBODN        ON      ON

2CBODN        ON      ON

3CBODN        ON      ON

4CBODN        ON      ON

5CBODN        ON      ON

6CBODN        ON      ON

7CBODN        ON      ON

8CBODN        ON      ON

9CBODN        ON      ON

10CBODN       ON      ON

ALG1          ON      ON

ALG2          ON      ON

ALG3          ON      ON

DO            ON      ON

TIC           ON      ON

ALK           ON      ON

ZOO1         OFF     OFF

LDOM_P        ON      ON

RDOM_P        ON      ON

LPOM_P        ON      ON

RPOM_P        ON      ON

LDOM_N        ON      ON

RDOM_N        ON      ON

LPOM_N        ON      ON

RPOM_N        ON      ON

CTR CON   CTRTRC  CTRTRC  CTRTRC  CTRTRC  CTRTRC  CTRTRC  CTRTRC  CTRTRC  CTRTRC

TDS           ON      ON

AGE          OFF     OFF

TRACER        ON      ON

COL1          ON      ON

Conduct       ON      ON

Chlorine      ON      ON

ISS1          ON      ON

PO4           ON      ON

NH4           ON      ON

NOx           ON      ON

DSi          OFF     OFF

PSi          OFF     OFF

TFe          OFF     OFF

LDOM          ON      ON

RDOM          ON      ON

LPOM          ON      ON

RPOM          ON      ON

1CBOD         ON      ON

2CBOD         ON      ON

3CBOD         ON      ON

4CBOD         ON      ON

5CBOD         ON      ON

6CBOD         ON      ON

7CBOD         ON      ON

8CBOD         ON      ON

9CBOD         ON      ON

10CBOD        ON      ON

1CBODP        ON      ON

2CBODP        ON      ON

3CBODP        ON      ON

4CBODP        ON      ON

5CBODP        ON      ON

6CBODP        ON      ON

7CBODP        ON      ON

8CBODP        ON      ON

9CBODP        ON      ON

10CBODP       ON      ON

1CBODN        ON      ON

2CBODN        ON      ON

3CBODN        ON      ON

4CBODN        ON      ON

5CBODN        ON      ON

6CBODN        ON      ON

7CBODN        ON      ON

8CBODN        ON      ON

9CBODN        ON      ON

10CBODN       ON      ON

ALG1          ON      ON

ALG2          ON      ON

ALG3          ON      ON

DO            ON      ON

TIC           ON      ON

ALK           ON      ON

ZOO1         OFF     OFF

LDOM_P        ON      ON

RDOM_P        ON      ON

LPOM_P        ON      ON

RPOM_P        ON      ON

LDOM_N        ON      ON

RDOM_N        ON      ON

LPOM_N        ON      ON

RPOM_N        ON      ON

CDT CON   CDTBRC  CDTBRC  CDTBRC  CDTBRC  CDTBRC  CDTBRC  CDTBRC  CDTBRC  CDTBRC

TDS           ON      ON

AGE          OFF     OFF

TRACER        ON      ON

COL1          ON      ON

Conduct       ON      ON

Chlorine      ON      ON

ISS1          ON      ON

PO4           ON      ON

NH4           ON      ON

NOx           ON      ON

DSi          OFF     OFF

PSi          OFF     OFF

TFe          OFF     OFF

LDOM          ON      ON

RDOM          ON      ON

LPOM          ON      ON

RPOM          ON      ON

1CBOD         ON      ON

2CBOD         ON      ON

3CBOD         ON      ON

4CBOD         ON      ON

5CBOD         ON      ON

6CBOD         ON      ON

7CBOD         ON      ON

8CBOD         ON      ON

9CBOD         ON      ON

10CBOD        ON      ON

1CBODP        ON      ON

2CBODP        ON      ON

3CBODP        ON      ON

4CBODP        ON      ON

5CBODP        ON      ON

6CBODP        ON      ON

7CBODP        ON      ON

8CBODP        ON      ON

9CBODP        ON      ON

10CBODP       ON      ON

1CBODN        ON      ON

2CBODN        ON      ON

3CBODN        ON      ON

4CBODN        ON      ON

5CBODN        ON      ON

6CBODN        ON      ON

7CBODN        ON      ON

8CBODN        ON      ON

9CBODN        ON      ON

10CBODN       ON      ON

ALG1          ON      ON

ALG2          ON      ON

ALG3          ON      ON

DO            ON      ON

TIC           ON      ON

ALK           ON      ON

ZOO1         OFF     OFF

LDOM_P        ON      ON

RDOM_P        ON      ON

LPOM_P        ON      ON

RPOM_P        ON      ON

LDOM_N        ON      ON

RDOM_N        ON      ON

LPOM_N        ON      ON

RPOM_N        ON      ON

CPR CON   CPRBRC  CPRBRC  CPRBRC  CPRBRC  CPRBRC  CPRBRC  CPRBRC  CPRBRC  CPRBRC

TDS           ON      ON

AGE          OFF     OFF

TRACER        ON      ON

COL1          ON      ON

Conduct       ON      ON

Chlorine      ON      ON

ISS1          ON      ON

PO4           ON      ON

NH4           ON      ON

NOx           ON      ON

DSi          OFF     OFF

PSi          OFF     OFF

TFe          OFF     OFF

LDOM          ON      ON

RDOM          ON      ON

LPOM          ON      ON

RPOM          ON      ON

1CBOD         ON      ON

2CBOD         ON      ON

3CBOD         ON      ON

4CBOD         ON      ON

5CBOD         ON      ON

6CBOD         ON      ON

7CBOD         ON      ON

8CBOD         ON      ON

9CBOD         ON      ON

10CBOD        ON      ON

1CBODP        ON      ON

2CBODP        ON      ON

3CBODP        ON      ON

4CBODP        ON      ON

5CBODP        ON      ON

6CBODP        ON      ON

7CBODP        ON      ON

8CBODP        ON      ON

9CBODP        ON      ON

10CBODP       ON      ON

1CBODN        ON      ON

2CBODN        ON      ON

3CBODN        ON      ON

4CBODN        ON      ON

5CBODN        ON      ON

6CBODN        ON      ON

7CBODN        ON      ON

8CBODN        ON      ON

9CBODN        ON      ON

10CBODN       ON      ON

ALG1          ON      ON

ALG2          ON      ON

ALG3          ON      ON

DO            ON      ON

TIC           ON      ON

ALK           ON      ON

ZOO1         OFF     OFF

LDOM_P        ON      ON

RDOM_P        ON      ON

LPOM_P        ON      ON

RPOM_P        ON      ON

LDOM_N        ON      ON

RDOM_N        ON      ON

LPOM_N        ON      ON

RPOM_N        ON      ON

New control files

Based on the options the user turns ON or OFF, new control files are required. These new control files are named below:

- w2_selective.npt – new variables controlling the selective withdrawal algorithm to select temperature targets

- w2_habitat.npt – new variables controlling fish habitat limits for temperature and dissolved oxygen and surface and segment volume weighted eutrophication state variables

- w2_envirpf.npt – new variables controlling setting environmental performance criteria

- w2_aerate.npt – variables describing use of dissolved oxygen addition to enhance dissolved oxygen levels through diffusers

Details of these new control files are in the CE-QUAL-W2 User Manual.

## Differences between Version 3.6 and Version 3.5

Version 3.6 can be run without changing any of the input files, even though the preprocessor will identify errors in the control file because of missing variables. Below is a highlighted list of locations in the file w2_con.npt where additional variables have been added. There are no other changes in the input files for Version 3.6.

The TKE algorithm has been updated with new algorithms that match experimental tank data for kinetic energy and dissipation. This is based on a master’s degree project by Sam Gould at Portland State University. A new user option is the TKE1 algorithm, in addition to the legacy algorithm TKE. This results in several new input variables on the following line of the w2_con.npt file that are only active if TKE1 is chosen for AZC:

EDDY VISC    AZC   AZSLC   AZMAX     FBC       E   ARODI STRCKLR BOUNDFR  TKECAL

WB 1          W2     IMP 1.00000       3   9.535   0.430    24.0   10.00     IMP

The roughness height of the water for correction of the vertical velocity wind profile is now a user-defined input, z0. Prior to this the model had hardwired the value of z0=0.003 m for wind speed correction at 2m (for evaporation where wind height at 2 m is typical) and z0=0.01 m for wind at 10 m (for shear stress calculations where wind height of 10 m is typical). For consistency, both conversions now use the same value of roughness height. If the user does not specify the value of z0 (for example if he/she leaves the spaces blank for z0 using a V3.5 control file), the code uses 0.001 m.

HYD COEF      AX      DX    CBHE    TSED      FI   TSEDF   FRICC      Z0

WB 1     1.00000 1.00000 0.30000 11.5000 0.01000 1.00000    MANN   0.001

A new option for output is in the format required for TECPLOT. For TECPLOT animation, there is only a flag in the CPL output line. This allows for easy model animation of the variables U, W, T, RHO, and all active constituents at the frequency specified by the CPL file as a function of distance and elevation.

CPL PLOT    CPLC    NCPL TECPLOT

WB 1          ON       1      ON

A new variable for determining the fraction of NO3-N that is diffused into the sediments that becomes organic matter, or SED-N, was introduced. According to one study, only about 37% of NO3-N that diffuses into the sediments becomes incorporated into organic matter in the sediments. The rest is denitrified.

NITRATE    NO3DK    NO3S FNO3SED

Wb 1        0.05     0.0    0.37

Wb 2        0.05     0.0    0.37

In V3.5, the model computed an average decay coefficient of the sediments based on what was deposited. The user now has the option to dynamically compute that decay rate or to have it fixed and controlled by the model user. A new variable was introduced called DYNSEDK, which is either ON/OFF to allow or not allow dynamic computation of the sediment decay rate.

SEDIMENT    SEDC   PRNSC   SEDCI    SEDK    SEDS    FSOD    FSED   SEDBR DYNSEDK

Wb 1          ON      ON     0.0     0.1     0.0     1.0     1.0   0.001     OFF

Wb 2          ON      ON     0.0     0.1     0.0     1.0     1.0   0.001     OFF

The user can now specify the # of processors to use on the host computer. Most users find that setting NPROC=2 gets the best results. Sometimes setting this greater than 2 results in slower model performance. Also, the CLOSEC control closes the windows dialog box after the model completes its simulation. This is helpful when using the Windows version of the release code in batch simulations. These are specified in the control file as follows:

GRID         NWB     NBR     IMX     KMX   NPROC  CLOSEC

1       4      66     117       2      ON

## Differences between Version 3.2 and Version 3.5

The differences in V3.5 and V3.2 input files are found in the control file: w2_con.npt and in the graph.npt file. All other files are the same between the two versions.

w2_con.npt

Below is an example of parts of the control file from V3.5 where all new variables are highlighted. Most of these changes have to do with the new zooplankton, macrophyte, and state variables added to the model. See the User Manual for a list of changes between V3.2 and V3.5 in the version history. Also, there were some deletions from the V3.2 w2_con.npt file. These are shown below.

New variables added to the control file are highlighted

.

.

IN/OUTFL     NTR     NST     NIW     NWD     NGT     NSP     NPI     NPU

1       1       0       0       0       0       0       0

CONSTITU     NGC     NSS     NAL     NEP    NBOD     NMC     NZP

5       1       1       1       5       0       1

MISCELL     NDAY

100

.

.

CST COMP     CCC    LIMC     CUF

ON      ON      10

CST ACTIVE   CAC

TDS          OFF

Gen1          ON

Gen2         OFF

Gen3         OFF

Gen4         OFF

Gen5         OFF

ISS1         OFF

PO4          OFF

NH4          OFF

NO3          OFF

DSI          OFF

PSI          OFF

FE           OFF

LDOM         OFF

RDOM         OFF

LPOM         OFF

RPOM         OFF

BOD1         OFF

BOD2         OFF

BOD3         OFF

BOD4         OFF

BOD5         OFF

ALG1         OFF

DO           OFF

TIC          OFF

ALK          OFF

ZOO1         OFF

LDOM_P       OFF

RDOM_P       OFF

LPOM_P       OFF

RPOM_P       OFF

LDOM_N       OFF

RDOM_N       OFF

LPOM_N       OFF

RPOM_N       OFF

CST DERI   CDWBC   CDWBC   CDWBC   CDWBC   CDWBC   CDWBC   CDWBC   CDWBC   CDWBC

DOC          OFF

POC          OFF

TOC          OFF

DON          OFF

PON          OFF

TON          OFF

TKN          OFF

TN           OFF

DOP          OFF

POP          OFF

TOP          OFF

TP           OFF

APR          OFF

CHLA         OFF

ATOT         OFF

%DO          OFF

TSS          OFF

TISS         OFF

CBOD         OFF

pH           OFF

CO2          OFF

HCO3         OFF

CO3          OFF

CST FLUX   CFWBC   CFWBC   CFWBC   CFWBC   CFWBC   CFWBC   CFWBC   CFWBC   CFWBC

TISSIN       OFF

TISSOUT      OFF

PO4AR        OFF

PO4AG        OFF

PO4AP        OFF

PO4ER        OFF

PO4EG        OFF

PO4EP        OFF

PO4POM       OFF

PO4DOM       OFF

PO4OM        OFF

PO4SED       OFF

PO4SOD       OFF

PO4SET       OFF

NH4NITR      OFF

NH4AR        OFF

NH4AG        OFF

NH4AP        OFF

NH4ER        OFF

NH4EG        OFF

NH4EP        OFF

NH4POM       OFF

NH4DOM       OFF

NH4OM        OFF

NH4SED       OFF

NH4SOD       OFF

NO3DEN       OFF

NO3AG        OFF

NO3EG        OFF

NO3SED       OFF

DSIAG        OFF

DSIEG        OFF

DSIPIS       OFF

DSISED       OFF

DSISOD       OFF

DSISET       OFF

PSIAM        OFF

PSINET       OFF

PSIDK        OFF

FESET        OFF

FESED        OFF

LDOMDK       OFF

LRDOM        OFF

RDOMDK       OFF

LDOMAP       OFF

LDOMEP       OFF

LPOMDK       OFF

LRPOM        OFF

RPOMDK       OFF

LPOMAP       OFF

LPOMEP       OFF

LPOMSET      OFF

RPOMSET      OFF

CBODDK       OFF

DOAP         OFF

DOAR         OFF

DOEP         OFF

DOER         OFF

DOPOM        OFF

DODOM        OFF

DOOM         OFF

DONITR       OFF

DOCBOD       OFF

DOREAR       OFF

DOSED        OFF

DOSOD        OFF

TICAG        OFF

TICEG        OFF

SEDDK        OFF

SEDAS        OFF

SEDLPOM      OFF

SEDSET       OFF

SODDK        OFF

CST ICON   C2IWB   C2IWB   C2IWB   C2IWB   C2IWB   C2IWB   C2IWB   C2IWB   C2IWB

TDS      0.00000

Gen1     0.00000

Gen2     0.00000

Gen3     0.00000

Gen4     0.00000

Gen5     0.00000

ISS1     0.00000

PO4      0.03000

NH4      0.01000

NO3      0.30000

DSI      0.00000

PSI      0.00000

FE       0.00000

LDOM     0.10000

RDOM     0.10000

LPOM     0.10000

RPOM     0.10000

BOD1     0.00000

BOD2     0.00000

BOD3     0.00000

BOD4     0.00000

BOD5     0.00000

ALG1     0.10000

DO       12.0000

TIC      5.00000

ALK      19.8000

ZOO1      0.1000

LDOM_P    0.0005

RDOM_P    0.0005

LPOM_P    0.0005

RPOM_P    0.0005

LDOM_N    0.0080

RDOM_N    0.0080

LPOM_N    0.0080

RPOM_N    0.0080

CST PRIN  CPRWBC  CPRWBC  CPRWBC  CPRWBC  CPRWBC  CPRWBC  CPRWBC  CPRWBC  CPRWBC

TDS          OFF

Gen1          ON

Gen2         OFF

Gen3         OFF

Gen4         OFF

Gen5         OFF

ISS1         OFF

PO4          OFF

NH4          OFF

NO3          OFF

DSI          OFF

PSI          OFF

FE           OFF

LDOM         OFF

RDOM         OFF

LPOM         OFF

RPOM         OFF

BOD1         OFF

BOD2         OFF

BOD3         OFF

BOD4         OFF

BOD5         OFF

ALG1         OFF

DO           OFF

TIC          OFF

ALK          OFF

ZOO1         OFF

LDOM_P       OFF

RDOM_P       OFF

LPOM_P       OFF

RPOM_P       OFF

LDOM_N       OFF

RDOM_N       OFF

LPOM_N       OFF

RPOM_N       OFF

CIN CON   CINBRC  CINBRC  CINBRC  CINBRC  CINBRC  CINBRC  CINBRC  CINBRC  CINBRC

TDS           ON

Gen1         OFF

Gen2          ON

Gen3          ON

Gen4          ON

Gen5          ON

ISS1          ON

PO4           ON

NH4           ON

NO3           ON

DSI          OFF

PSI          OFF

FE           OFF

LDOM          ON

RDOM          ON

LPOM          ON

RPOM          ON

BOD1          ON

BOD2          ON

BOD3          ON

BOD4          ON

BOD5          ON

ALG1          ON

DO            ON

TIC           ON

ALK           ON

ZOO1         OFF

LDOM_P       OFF

RDOM_P       OFF

LPOM_P       OFF

RPOM_P       OFF

LDOM_N       OFF

RDOM_N       OFF

LPOM_N       OFF

RPOM_N       OFF

CTR CON   CTRTRC  CTRTRC  CTRTRC  CTRTRC  CTRTRC  CTRTRC  CTRTRC  CTRTRC  CTRTRC

TDS           ON      ON

Gen1         OFF     OFF

Gen2          ON      ON

Gen3          ON      ON

Gen4          ON      ON

Gen5          ON      ON

ISS1          ON      ON

PO4           ON      ON

NH4           ON      ON

NO3           ON      ON

DSI          OFF     OFF

PSI          OFF     OFF

FE           OFF     OFF

LDOM          ON      ON

RDOM          ON      ON

LPOM          ON      ON

RPOM          ON      ON

BOD1          ON      ON

BOD2          ON      ON

BOD3          ON      ON

BOD4          ON      ON

BOD5          ON      ON

ALG1          ON      ON

DO            ON      ON

TIC           ON      ON

ALK           ON      ON

ZOO1         OFF     OFF

LDOM_P       OFF     OFF

RDOM_P       OFF     OFF

LPOM_P       OFF     OFF

RPOM_P       OFF     OFF

LDOM_N       OFF     OFF

RDOM_N       OFF     OFF

LPOM_N       OFF     OFF

RPOM_N       OFF     OFF

CDT CON   CDTBRC  CDTBRC  CDTBRC  CDTBRC  CDTBRC  CDTBRC  CDTBRC  CDTBRC  CDTBRC

TDS           ON

Gen1         OFF

Gen2          ON

Gen3          ON

Gen4          ON

Gen5          ON

ISS1          ON

PO4           ON

NH4           ON

NO3           ON

DSI          OFF

PSI          OFF

FE           OFF

LDOM          ON

RDOM          ON

LPOM          ON

RPOM          ON

BOD1          ON

BOD2          ON

BOD3          ON

BOD4          ON

BOD5          ON

ALG1          ON

DO            ON

TIC           ON

ALK           ON

ZOO1         OFF

LDOM_P       OFF

RDOM_P       OFF

LPOM_P       OFF

RPOM_P       OFF

LDOM_N       OFF

RDOM_N       OFF

LPOM_N       OFF

RPOM_N       OFF

CPR CON   CPRBRC  CPRBRC  CPRBRC  CPRBRC  CPRBRC  CPRBRC  CPRBRC  CPRBRC  CPRBRC

TDS           ON

Gen1         OFF

Gen2          ON

Gen3          ON

Gen4          ON

Gen5          ON

ISS1          ON

PO4           ON

NH4           ON

NO3           ON

DSI          OFF

PSI          OFF

FE           OFF

LDOM          ON

RDOM          ON

LPOM          ON

RPOM          ON

BOD1          ON

BOD2          ON

BOD3          ON

BOD4          ON

BOD5          ON

ALG1          ON

DO            ON

TIC           ON

ALK           ON

ZOO1         OFF

LDOM_P       OFF

RDOM_P       OFF

LPOM_P       OFF

RPOM_P       OFF

LDOM_N       OFF

RDOM_N       OFF

LPOM_N       OFF

RPOM_N       OFF

EX COEF    EXH2O    EXSS    EXOM    BETA     EXC    EXIC

WB 1     0.45000 0.01000 0.40000 0.45000     OFF     OFF

ALG EX       EXA     EXA     EXA     EXA     EXA     EXA

0.10000

ZOO EX       EXZ     EXZ     EXZ     EXZ     EXZ     EXZ

0.2     0.2     0.2

MACRO EX     EXM     EXM     EXM     EXM     EXM     EXM

0.0100

GENERIC    CGQ10   CG0DK   CG1DK     CGS

CG 1     0.00000 -1.0000 0.00000 0.00000

CG 2     0.00000 0.00000 0.00000 0.00000

CG 3     1.04000 0.00000 0.50000 0.00000

CG 4     0.00000 0.00000 0.00000 0.00000

CG 5     0.00000 0.00000 0.00000 0.00000

S SOLIDS     SSS   SEDRC   TAUCR

SS1      1.50000     OFF    0.00

ALGAL RATE    AG      AR      AE      AM      AS    AHSP    AHSN   AHSSI    ASAT

ALG1     2.00000 0.12000 0.02000 0.05000 0.04000 0.00500 0.00500 0.00000 50.0000

ALGAL TEMP   AT1     AT2     AT3     AT4     AK1     AK2     AK3     AK4

ALG1     5.00000 12.0000 20.0000 30.0000 0.10000 0.99000 0.99000 0.10000

ALG STOI    ALGP    ALGN    ALGC   ALGSI   ACHLA   ALPOM   ANEQN    ANPR

ALG1     0.00500 0.08000 0.45000 0.00000 65.0000 0.80000       1 0.00100

EPIPHYTE    EPIC    EPIC    EPIC    EPIC    EPIC    EPIC    EPIC    EPIC    EPIC

EPI1         OFF

EPI PRIN    EPRC    EPRC    EPRC    EPRC    EPRC    EPRC    EPRC    EPRC    EPRC

EPI1         OFF

EPI INIT   EPICI   EPICI   EPICI   EPICI   EPICI   EPICI   EPICI   EPICI   EPICI

EPI1     10.0000

EPI RATE      EG      ER      EE      EM      EB    EHSP    EHSN   EHSSI

EPI1     2.00000 0.05000 0.02000 0.05000 0.01000 0.00200 0.00200 0.00000

EPI HALF    ESAT     EHS   ENEQN    ENPR

EPI1     50.0000 40.0000       2 0.00200

EPI TEMP     ET1     ET2     ET3     ET4     EK1     EK2     EK3     EK4

EPI1     2.00000 5.00000 20.0000 30.0000 0.10000 0.99000 0.99000 0.10000

EPI STOI      EP      EN      EC     ESI   ECHLA    EPOM

EPI1     0.00500 0.08000 0.45000 0.00000 65.0000 0.80000

ZOOP RATE     ZG      ZR      ZM    ZEFF   PREFP  ZOOMIN    ZS2P

Zoo1        1.50    0.10   0.010    0.50    0.50  0.0100    0.30

ZOOP ALGP  PREFA   PREFA   PREFA   PREFA   PREFA   PREFA   PREFA   PREFA   PREFA

Zoo1        1.00    0.50    0.50

ZOOP ZOOP  PREFZ   PREFZ   PREFZ   PREFZ   PREFZ   PREFZ   PREFZ   PREFZ   PREFZ

Zoo1        0.00    0.00    0.00

ZOOP TEMP    ZT1     ZT2     ZT3     ZT4     ZK1     ZK2     ZK3     ZK4

0.0    15.0    20.0    36.0     0.1     0.9    0.98   0.100

ZOOP STOI     ZP      ZN      ZC

0.01500 0.08000 0.45000

MACROPHYT MACWBC  MACWBC  MACWBC  MACWBC  MACWBC  MACWBC  MACWBC  MACWBC  MACWBC

Mac1          ON     OFF     OFF

MAC PRINT MPRWBC  MPRWBC  MPRWBC  MPRWBC  MPRWBC  MPRWBC  MPRWBC  MPRWBC  MPRWBC

Mac1          ON     OFF     OFF

MAC INI  MACWBCI MACWBCI MACWBCI MACWBCI MACWBCI MACWBCI MACWBCI MACWBCI MACWBCI

Mac1     0.00000	 0.1     0.5

MAC RATE      MG      MR      MM    MSAT    MHSP    MHSN    MHSC    MPOM  LRPMAC

Mac 1       0.30    0.05    0.05    30.0     0.0     0.0     0.0     0.9     0.2

MAC SED     PSED    NSED

MAC 1        0.5     0.5

MAC DIST    MBMP    MMAX

Mac 1       40.0   500.0

MAC DRAG  CDSTEM     DWV    DMSA   ANORM

Mac 1        2.0     7e4    8.00    0.80

MAC TEMP     MT1     MT2     MT3     MT4     MK1     MK2     MK3     MK4

Mac 1        7.0    15.0    24.0    34.0     0.1    0.99    0.99    0.01

MAC STOICH    MP      MN      MC

Mac 1      0.005    0.08    0.45

DOM       LDOMDK  RDOMDK   LRDDK

WB 1     0.10000 0.00100 0.00100

POM       LPOMDK  RPOMDK   LRPDK    POMS

WB 1     0.08000 0.00100 0.00100 0.10000

OM STOIC    ORGP    ORGN    ORGC   ORGSI

WB 1     0.00500 0.08000 0.45000 0.18000

OM RATE     OMT1    OMT2    OMK1    OMK2

WB 1     4.00000 30.0000 0.10000 0.99000

CBOD        KBOD    TBOD    RBOD   CBODS

BOD 1    0.04180 1.01470 1.00000     0.0

BOD 2    0.13020 1.01470 1.00000     0.0

BOD 3    0.04690 1.01470 1.00000     0.0

BOD 4    0.08800 1.01470 1.00000     0.0

BOD 5    0.05000 1.01470 1.00000     0.0

CBOD STOIC  BODP    BODN    BODC

BOD 1    0.00500 0.08000 0.45000

BOD 2    0.00500 0.08000 0.45000

BOD 3    0.00500 0.08000 0.45000

BOD 4    0.00500 0.08000 0.45000

BOD 5    0.00500 0.08000 0.45000

PHOSPHOR    PO4R   PARTP

WB 1     0.00100 0.00000

AMMONIUM    NH4R   NH4DK

WB 1     0.00100 0.50000

NH4 RATE   NH4T1   NH4T2   NH4K1   NH4K2

WB 1     5.00000 25.0000 0.10000 0.99000

NITRATE    NO3DK    NO3S

WB 1     0.05000 0.00000

NO3 RATE   NO3T1   NO3T2   NO3K1   NO3K2

WB 1     5.00000 25.0000 0.10000 0.99000

SILICA      DSIR    PSIS   PSIDK  PARTSI

WB 1     0.10000 0.00000 0.30000 0.20000

IRON         FER     FES

WB 1     0.10000 0.00000

SED CO2     CO2R

WB 1     0.10000

STOICH 1   O2NH4    O2OM

WB 1     4.57000 1.40000

STOICH 2    O2AR    O2AG

ALG1     1.10000 1.40000

STOICH 3    O2ER    O2EG

EPI1     1.10000 1.40000

STOICH 4    O2ZR

ZOO1     1.10000

STOICH 5    O2MR    O2MG

MAC1         1.1     1.4

O2 LIMIT     KDO

0.10000

SEDIMENT    SEDC  SEDPRC   SEDCI    SEDK    SEDS    FSOD    FSED   SEDBR

WB 1          ON      ON 0.00000 0.10000     0.1 1.00000 1.00000     0.2

SOD RATE   SODT1   SODT2   SODK1   SODK2

WB 1     4.00000 30.0000 0.10000 0.99000

S DEMAND     SOD     SOD     SOD     SOD     SOD     SOD     SOD     SOD     SOD

0.6     0.6     0.6     0.6     0.6     0.6     0.6     0.6     0.6

0.6     0.6     0.6     0.6     0.6     0.6     0.6     0.6     0.6

0.6     0.6     0.6     0.6     0.6     0.6     0.6     0.6     0.6

0.6     0.6     0.6     0.6     0.6     0.6     0.6     0.6     0.6

0.6

REAERATION  TYPE    EQN#   COEF1   COEF2   COEF3   COEF4

WB1         LAKE       6

Lines removed from the V3.2 control file: These are a result of eliminating the pumpback and line printer settings.

See below the deleted part of the V3.2 control file:

DST TRIB    DTRC

Br 1          ON

Br 2          ON

Br 3         OFF

Br 4         OFF

Br 5         OFF

PUMPBACK     JBG     KTG     KBG     JBP     KTP     KBP

0

PRINTER      LJC

IV

HYD PRINT HPRWBC  HPRWBC  HPRWBC  HPRWBC  HPRWBC  HPRWBC  HPRWBC  HPRWBC  HPRWBC

NVIOL        OFF     OFF

U             ON      ON

Graph.npt file changes. These changes are a result of the new state variables in W2 and are highlighted below.

Hydrodynamic, constituent, and derived constituent names, formats, multipliers, and array viewer controls

....................HNAME...................     FMTH   HMULT    HMIN    HMAX   HPLTC       #

Timestep violations [NVIOL]                     (I10)     1.0    -1.0     1.0     OFF       1

Horizontal velocity [U], m/s                (1PE10.1)     1.0  -.1000    0.15     OFF       2

Vertical velocity [W], m/s                  (1PE10.1)     1.0  -.1E-6   -0.01     OFF       3

Temperature [T1], <o/>C                       (F10.2)     1.0   -10.0   -26.0      ON       4

Density [RHO], g/m^3                          (F10.3)     1.0   997.0  1005.0     OFF       5

Vertical eddy viscosity [AZ], m^2/s           (F10.3)     1.0  -1E-08    0.01     OFF       6

Velocity shear stress [SHEAR], 1/s^2          (F10.3)     1.0  -1E-08    0.01     OFF       7

Internal shear [ST], m^3/s                    (F10.3)     1.0  -1E-08    0.01     OFF       8

Bottom shear [SB], m^3/s                      (F10.3)     1.0  -1E-08    0.01     OFF       9

Longitudinal momentum [ADMX], m^3/s           (F10.3)     1.0  -1E-08    0.01     OFF      10

Longitudinal momentum [DM], m^3/s             (F10.3)     1.0  -1E-08    0.01     OFF      11

Horizontal density gradient [HDG], m^3/s      (F10.3)     1.0  -1E-08    0.01     OFF      12

Vertical momentum [ADMZ], m^3/s               (F10.3)     1.0  -1E-08    0.01     OFF      13

Horizontal pressure gradient [HPG], m^3/s     (F10.3)     1.0  -1E-08    10.0     OFF      14

Gravity term channel slope [GRAV], m^3/s      (F10.3)     1.0     0.0     0.0     OFF      15

....................CNAME....................    FMTC   CMULT    CMIN    CMAX   CPLTC       #

TDS, g/m^3                                    (F10.3)     1.0    -1.0   200.0     OFF       1

Age, days                                     (F10.3)     1.0    -1.0  -200.0      ON       2

Tracer, g/m^3                                 (F10.3)     1.0 -20.000   100.0     OFF       3

Bacteria, col/100ml                           (F10.3)     1.0 -20.000   100.0     OFF       4

Conductivity, mhos                            (F10.3)     1.0 -20.000   100.0     OFF       5

Chloride, mg/l                                (F10.3)     1.0 -20.000   100.0     OFF       6

ISS, g/m^3                                    (F10.3)     1.0 -20.000   100.0     OFF       7

Phosphate, g/m^3                              (F10.3)  1000.0    -1.0   500.0     OFF       8

Ammonium, g/m^3                               (F10.3)  1000.0 -0.1000   300.0     OFF       9

Nitrate-Nitrite, g/m^3                        (F10.3)     1.0 -0.1000     5.0     OFF      10

Dissolved silica, g/m^3                       (F10.3)     1.0    -1.0    10.0     OFF      11

Particulate silica, g/m^3                     (F10.3)     1.0 -0.2000    15.0     OFF      12

Total iron, g/m^3                             (F10.3)     1.0 -0.1000     2.0     OFF      13

Labile DOM, g/m^3                             (F10.3)     1.0 -0.1000    -3.0     OFF      14

Refractory DOM, g/m^3                         (F10.3)     1.0 -0.1000    -4.0     OFF      15

Labile POM, g/m^3                             (F10.3)     1.0 -0.1000    -3.0     OFF      16

Refractory POM, g/m^3                         (F10.3)     1.0 -0.1000    -4.0     OFF      17

CBOD1, g/m^3                                  (F10.3)     1.0 -0.0100     3.0     OFF      18

CBOD2, g/m^3                                  (F10.3)     1.0 -0.0100     3.0     OFF      19

CBOD3, g/m^3                                  (F10.3)     1.0 -0.0100     3.0     OFF      20

CBOD4, g/m^3                                  (F10.3)     1.0 -0.0100     3.0     OFF      21

CBOD5, g/m^3                                  (F10.3)     1.0 -0.0100     3.0     OFF      22

Algae, g/m^3                                  (F10.3)     1.0 -0.0100     3.0     OFF      23

Dissolved oxygen, g/m^3                       (F10.3)     1.0 -0.0100    -1.0     OFF      24

Inorganic carbon, g/m^3                       (F10.3)     1.0 -0.0100     3.0     OFF      25

Alkalinity, g/m^3                             (F10.3)     1.0 -0.0100     3.0     OFF      26

zooplankton1, mg/m^3                          (g10.3)  1000.0 -0.0100     1.0     OFF      27

LDOM P, mg/m^3                                (g10.3)  1000.0     0.0     1.0     OFF      28

RDOM P, mg/m^3                                (g10.3)  1000.0     0.0     1.0     OFF      29

LPOM P, mg/m^3                                (g10.3)  1000.0     0.0     1.0     OFF      30

RPOM P, mg/m^3                                (g10.3)  1000.0     0.0     1.0     OFF      31

LDOM N, mg/m^3                                (g10.3)  1000.0     0.0     1.0     OFF      32

RDOM N, mg/m^3                                (g10.3)  1000.0     0.0     1.0     OFF      33

LPOM N, mg/m^3                                (g10.3)  1000.0     0.0     1.0     OFF      34

RPOM N, mg/m^3                                (g10.3)  1000.0     0.0     1.0     OFF      35

....................CDNAME...................   FMTCD  CDMULT   CDMIN   CDMAX  CDPLTC       #

Dissolved organic carbon, g/m^3               (F10.3)     1.0    -1.0    25.0     OFF       1

Particulate organic carbon, g/m^3             (F10.3)     1.0    -1.0    50.0     OFF       2

Total organic carbon, g/m^3                   (F10.3)     1.0    -1.0    25.0     OFF       3

Dissolved organic nitrogen, g/m^3             (F10.3)     1.0    -1.0    25.0     OFF       4

Particulate organic nitrogen, g/m^3           (F10.3)     1.0    -1.0    25.0     OFF       5

Total organic nitrogen, g/m^3                 (F10.3)     1.0    -1.0    50.0     OFF       6

Total Kheldahl Nitrogen, g/m^3                (F10.3)     1.0    -1.0    15.0     OFF       7

Total nitrogen, g/m^3                         (F10.3)     1.0    -1.0    15.0     OFF       8

Dissolved organic phosphorus, mg/m^3          (F10.3)  1000.0    -1.0    25.0     OFF       9

Particulate organic phosphorus, mg/m^3        (F10.3)  1000.0    -1.0    -1.0     OFF      10

Total organic phosphorus, mg/m^3              (F10.3)  1000.0    -1.0     5.0     OFF      11

Total phosphorus, mg/m^3                      (F10.3)  1000.0    -1.0    20.0     OFF      12

Algal production, g/m^2/day                   (F10.3)     1.0    -1.0     5.0     OFF      13

Chlorophyll a, mg/m^3                         (F10.3)     1.0    -5.0   145.0     OFF      14

Total algae, g/m^3                            (F10.3)     1.0    -1.0    60.0     OFF      15

Oxygen % Gas Saturation                       (F10.3)     1.0    -1.0    50.0     OFF      16

Total suspended Solids, g/m^3                 (F10.3)     1.0    -1.0     5.0     OFF      17

Total Inorganic Suspended Solids,g/m^3        (F10.3)     1.0    -1.0    20.0     OFF      18

Carbonaceous Ultimate BOD, g/m^3              (F10.3)     1.0     5.0     9.0     OFF      19

pH                                            (F10.3)     1.0    -1.0    10.0     OFF      20

CO2                                           (F10.3)     1.0    -1.0    10.0     OFF      21

HCO3                                          (F10.3)     1.0    -1.0    10.0     OFF      22

CO3                                           (F10.3)     0.0     0.0     0.0     OFF      23

## Differences between Version 3.1 and Version 3.2

There are minor differences in two input files between the two versions: w2_con.npt and the graph.npt file. All other files are the same between the two versions.

w2_con.npt

The only section in which there is a slight difference in the control file is where the inorganic suspended solids group settling velocities are entered. In Version 3.1, this section resembles the following:

ALG EX       EXA     EXA     EXA     EXA     EXA     EXA

0.10000

GENERIC    CGQ10   CG0DK   CG1DK     CGS

CG 1     0.00000 -1.0000 0.00000 0.00000

CG 2     0.00000 0.00000 0.00000 0.00000

CG 3     1.04000 0.00000 0.50000 0.00000

CG 4     0.00000 0.00000 0.00000 0.00000

CG 5     0.00000 0.00000 0.00000 0.00000

S SOLIDS     SSS     SSS     SSS     SSS     SSS     SSS     SSS     SSS     SSS

1.50000

ALGAL RATE    AG      AR      AE      AM      AS    AHSP    AHSN   AHSSI    ASAT

ALG1     2.00000 0.12000 0.02000 0.05000 0.04000 0.00500 0.00500 0.00000 50.0000

In Version 3.2, there is now a sediment resuspension capability for wind-driven resuspension along the shores of lakes and reservoirs. The Version 3.2 control file has the following lines in this same section of the control file:

ALG EX       EXA     EXA     EXA     EXA     EXA     EXA

0.10000

GENERIC    CGQ10   CG0DK   CG1DK     CGS

CG 1     0.00000 -1.0000 0.00000 0.00000

CG 2     0.00000 0.00000 0.00000 0.00000

CG 3     1.04000 0.00000 0.50000 0.00000

CG 4     0.00000 0.00000 0.00000 0.00000

CG 5     0.00000 0.00000 0.00000 0.00000

S SOLIDS     SSS   SEDRC   TAUCR

SS1      1.50000     OFF    0.00

ALGAL RATE    AG      AR      AE      AM      AS    AHSP    AHSN   AHSSI    ASAT

ALG1     2.00000 0.12000 0.02000 0.05000 0.04000 0.00500 0.00500 0.00000 50.0000

For Version 3.2, SSS is the settling velocity for particle group 1, SEDRC is the control which turns ON or OFF sediment resuspension, and TAUCR is the critical shear stress at which resuspension occurs. For Version 3.2, each line represents one SS group, while in Version 3.1, each group settling velocity is in the next eight columns moving across the page.

graph.npt

The graph file controls output formatting and the graphing parameters used in Array Viewer (only for the PC platform). The files have been rearranged significantly. A Version 3.1 graph file is shown below:

Constituent, hydrodynamic, and derived constituent names, formats, multipliers, and array viewer controls

....................CNAME....................   CMULT    CMIN    CMAX   CPLTC       #

TDS g/m^3 or Salinity kg/m^3                  1.00000 -1.0000 200.000     OFF       1

Generic Constituent,g/m^3, #1                 1.00000 -1.0000 -200.00      ON       2

Generic Constituent,g/m^3, #2                 1.00000 -1.0000 1000.00     OFF       3

Generic Constituent,g/m^3, #3                 1.00000 -1.0000 5.00000     OFF       4

Generic Constituent,g/m^3, #4                 1.00000 -1.0000 -300.00     OFF       5

Generic Constituent,g/m^3, #5                 1.00000 -1.0000 -3.0000     OFF       6

Suspended solids,g/m^3, #1                    1.00000 -1.0000 15.0000     OFF       7

Phosphate, g/m^3                              1000.00 -1.0000 -50.000     OFF       8

Ammonium,  g/m^3                              1000.00 -0.1000 -300.00     OFF       9

Nitrate-Nitrite, g/m^3                        1.00000 -0.1000 -5.0000     OFF      10

Dissolved silica, g/m^3                       1.00000 -1.0000 10.0000     OFF      11

Particulate silica, g/m^3                     1.00000 -0.2000 15.0000     OFF      12

Total iron, g/m^3                             1.00000 -0.1000 2.00000     OFF      13

Labile DOM, g/m^3                             1.00000 -0.1000 -3.0000     OFF      14

Refractory DOM, g/m^3                         1.00000 -0.1000 4.00000     OFF      15

Labile POM, g/m^3                             1.00000 -0.1000 3.00000     OFF      16

Refractory POM, g/m^3                         1.00000 -0.1000 4.00000     OFF      17

CBOD, g/m^3,     #1                           1.00000 -0.1000 10.0000     OFF      18

CBOD, g/m^3,     #2                           1.00000 -0.1000 10.0000     OFF      19

CBOD, g/m^3,     #3                           1.00000 -0.1000 10.0000     OFF      20

CBOD, g/m^3,     #4                           1.00000 -0.1000 10.0000     OFF      21

CBOD, g/m^3,     #5                           1.00000 -0.1000 10.0000     OFF      22

Algae,  g/m^3,      #1                        1.00000 -0.0100 -3.0000     OFF      23

Dissolved oxygen, g/m^3                       1.00000 -2.0000 15.0000     OFF      24

Inorganic carbon, g/m^3                       1.00000 -1.0000 10.0000     OFF      25

Alkalinity, g/m^3                             1.00000 -1.0000 200.000     OFF      26

....................HNAME...................     HFMT    HMIN    HMAX   HPLTC       #

Timestep violations [NVIOL]                   (F10.0) -1.0000  100000     OFF       1

Horizontal velocity [U], m/s                (1PE10.1) -0.0100 0.10000      ON       2

Vertical velocity [W], m/s                  (1PE10.1)-.10E-06 0.01000     OFF       3

Temperature [T1], <o/>C                       (F10.2) -2.0000 -30.000      ON       4

Density [RHO], g/m^3                          (F10.2) 997.000 1005.00     OFF       5

Vertical eddy viscosity [AZ], m^2/s         (1PE10.1)  -1E-08 0.00100     OFF       6

Velocity shear stress [SHEAR], 1/s^2        (1PE10.1)  -1E-08 0.01000     OFF       7

Internal shear [ST], m^3/s                  (1PE10.1)  -1E-08 0.01000     OFF       8

Bottom shear [SB], m^3/s                    (1PE10.1)  -1E-08 0.01000     OFF       9

Longitudinal momentum [ADMX], m^3/s         (1PE10.1)  -1E-08 0.01000     OFF      10

Longitudinal momentum [DM], m^3/s           (1PE10.1)  -1E-08 0.01000     OFF      11

Horizontal density gradient [HDG], m^3/s    (1PE10.1)  -1E-08 0.01000     OFF      12

Vertical momentum [ADMZ], m^3/s             (1PE10.1)  -1E-08 0.01000     OFF      13

Horizontal pressure gradient [HPG], m^3/s   (1PE10.1)  -1E-08 0.01000     OFF      14

Gravity term channel slope [GRAV], m^3/s    (1PE10.1)  -1E-08 10.0000     OFF      15

....................CDNAME...................  CDMULT   CDMIN   CDMAX  CDPLTC       #

Dissolved organic carbon, g/m^3               1.00000 -1.0000 3.00000     OFF       1

Particulate organic carbon, g/m^3             1.00000 -1.0000 25.0000     OFF       2

Total organic carbon, g/m^3                   1.00000 -1.0000 50.0000     OFF       3

Dissolved organic nitrogen, g/m^3             1.00000 -1.0000 25.0000     OFF       4

Particulate organic nitrogen, g/m^3           1.00000 -1.0000 25.0000     OFF       5

Total organic nitrogen, g/m^3                 1.00000 -1.0000 25.0000     OFF       6

Total Kheldahl Nitrogen, g/m^3                1.00000 -1.0000 5.00000     OFF       7

Total nitrogen, g/m^3                         1.00000 -1.0000 50.0000     OFF       8

Dissolved organic phosphorus, mg/m^3          1000.00 -1.0000 15.0000     OFF       9

Particulate organic phosphorus, mg/m^3        1000.00 -1.0000 15.0000     OFF      10

Total organic phosphorus, mg/m^3              1000.00 -1.0000 25.0000     OFF      11

Total phosphorus, mg/m^3                      1000.00 -1.0000 -1.0000     OFF      12

Algal production, g/m^2/day                   1.00000 -1.0000 5.00000     OFF      13

Chlorophyll a, mg/m^3                         1000.00 -1.0000 -70.000     OFF      14

Total algae, g/m^3                            1.00000 -1.0000 5.00000     OFF      15

Oxygen % Gas Saturation                       1.00000 -5.0000 145.000     OFF      16

Total suspended Solids, g/m^3                 1.00000 -1.0000 60.0000     OFF      17

Total Inorganic Suspended Solids,g/m^3        1.00000 -1.0000 50.0000     OFF      18

Carbonaceous Ultimate BOD, g/m^3              1.00000 -1.0000 20.0000     OFF      19

pH                                            1.00000 6.00000 9.00000     OFF      20

CO2                                           1.00000 -1.0000 10.0000     OFF      21

HCO3                                          1.00000 -1.0000 10.0000     OFF      22

CO3                                           1.00000 -1.0000 10.0000     OFF      23

An example of the same graph file but for Version 3.2 is shown below:

Hydrodynamic, constituent, and derived constituent names, formats, multipliers, and array viewer controls

....................HNAME...................     FMTH   HMULT    HMIN    HMAX   HPLTC       #

Timestep violations [NVIOL]                     (I10)     1.0    -1.0     1.0     OFF       1

Horizontal velocity [U], m/s                  (Z10.8)     1.0  -.1000    0.15      ON       2

Vertical velocity [W], m/s                    (Z10.8)     1.0  -.1E-6   -0.01     OFF       3

Temperature [T1], <o/>C                       (Z10.8)     1.0   -10.0   -26.0      ON       4

Density [RHO], g/m^3                          (Z10.8)     1.0   997.0  1005.0     OFF       5

Vertical eddy viscosity [AZ], m^2/s           (Z10.8)     1.0  -1E-08    0.01     OFF       6

Velocity shear stress [SHEAR], 1/s^2          (Z10.8)     1.0  -1E-08    0.01     OFF       7

Internal shear [ST], m^3/s                    (Z10.8)     1.0  -1E-08    0.01     OFF       8

Bottom shear [SB], m^3/s                      (Z10.8)     1.0  -1E-08    0.01     OFF       9

Longitudinal momentum [ADMX], m^3/s           (Z10.8)     1.0  -1E-08    0.01     OFF      10

Longitudinal momentum [DM], m^3/s             (Z10.8)     1.0  -1E-08    0.01     OFF      11

Horizontal density gradient [HDG], m^3/s      (Z10.8)     1.0  -1E-08    0.01     OFF      12

Vertical momentum [ADMZ], m^3/s               (Z10.8)     1.0  -1E-08    0.01     OFF      13

Horizontal pressure gradient [HPG], m^3/s     (Z10.8)     1.0  -1E-08    10.0     OFF      14

Gravity term channel slope [GRAV], m^3/s      (Z10.8)     1.0     0.0     0.0     OFF      15

....................CNAME....................    FMTC   CMULT    CMIN    CMAX   CPLTC       #

TDS, g/m^3                                    (Z10.8)     1.0    -1.0   200.0     OFF       1

Age, days                                     (Z10.8)     1.0    -1.0  -200.0      ON       2

Tracer, g/m^3                                 (Z10.8)     1.0 -20.000   100.0     OFF       3

Bacteria, col/100ml                           (Z10.8)     1.0 -20.000   100.0     OFF       4

Conductivity, mhos                            (Z10.8)     1.0 -20.000   100.0     OFF       5

Chloride, mg/l                                (Z10.8)     1.0 -20.000   100.0     OFF       6

ISS, g/m^3                                    (Z10.8)     1.0 -20.000   100.0     OFF       7

Phosphate, g/m^3                              (Z10.8)  1000.0    -1.0   500.0     OFF       8

Ammonium, g/m^3                               (Z10.8)  1000.0 -0.1000   300.0     OFF       9

Nitrate-Nitrite, g/m^3                        (Z10.8)     1.0 -0.1000     5.0     OFF      10

Dissolved silica, g/m^3                       (Z10.8)     1.0    -1.0    10.0     OFF      11

Particulate silica, g/m^3                     (Z10.8)     1.0 -0.2000    15.0     OFF      12

Total iron, g/m^3                             (Z10.8)     1.0 -0.1000     2.0     OFF      13

Labile DOM, g/m^3                             (Z10.8)     1.0 -0.1000    -3.0     OFF      14

Refractory DOM, g/m^3                         (Z10.8)     1.0 -0.1000    -4.0     OFF      15

Labile POM, g/m^3                             (Z10.8)     1.0 -0.1000    -3.0     OFF      16

Refractory POM, g/m^3                         (Z10.8)     1.0 -0.1000    -4.0     OFF      17

CBOD1, g/m^3                                  (Z10.8)     1.0 -0.0100     3.0     OFF      18

CBOD2, g/m^3                                  (Z10.8)     1.0 -0.0100     3.0     OFF      19

CBOD3, g/m^3                                  (Z10.8)     1.0 -0.0100     3.0     OFF      20

CBOD4, g/m^3                                  (Z10.8)     1.0 -0.0100     3.0     OFF      21

CBOD5, g/m^3                                  (Z10.8)     1.0 -0.0100     3.0     OFF      22

Algae, g/m^3                                  (Z10.8)     1.0 -0.0100     3.0     OFF      23

Dissolved oxygen, g/m^3                       (Z10.8)     1.0 -0.0100    -1.0     OFF      24

Inorganic carbon, g/m^3                       (Z10.8)     1.0 -0.0100     3.0     OFF      25

Alkalinity, g/m^3                             (Z10.8)     1.0 -0.0100     3.0     OFF      26

....................CDNAME...................   FMTCD  CDMULT   CDMIN   CDMAX  CDPLTC       #

Dissolved organic carbon, g/m^3               (F10.3)     1.0    -1.0    25.0     OFF       1

Particulate organic carbon, g/m^3             (F10.3)     1.0    -1.0    50.0     OFF       2

Total organic carbon, g/m^3                   (F10.3)     1.0    -1.0    25.0     OFF       3

Dissolved organic nitrogen, g/m^3             (F10.3)     1.0    -1.0    25.0     OFF       4

Particulate organic nitrogen, g/m^3           (F10.3)     1.0    -1.0    25.0     OFF       5

Total organic nitrogen, g/m^3                 (F10.3)     1.0    -1.0    50.0     OFF       6

Total Kheldahl Nitrogen, g/m^3                (F10.3)     1.0    -1.0    15.0     OFF       7

Total nitrogen, g/m^3                         (F10.3)     1.0    -1.0    15.0     OFF       8

Dissolved organic phosphorus, mg/m^3          (F10.3)  1000.0    -1.0    25.0     OFF       9

Particulate organic phosphorus, mg/m^3        (F10.3)  1000.0    -1.0    -1.0     OFF      10

Total organic phosphorus, mg/m^3              (F10.3)  1000.0    -1.0     5.0     OFF      11

Total phosphorus, mg/m^3                      (F10.3)  1000.0    -1.0    20.0     OFF      12

Algal production, g/m^2/day                   (F10.3)     1.0    -1.0     5.0     OFF      13

Chlorophyll a, mg/m^3                         (F10.3)     1.0    -5.0   145.0     OFF      14

Total algae, g/m^3                            (F10.3)     1.0    -1.0    60.0     OFF      15

Oxygen % Gas Saturation                       (F10.3)     1.0    -1.0    50.0     OFF      16

Total suspended Solids, g/m^3                 (F10.3)     1.0    -1.0     5.0     OFF      17

Total Inorganic Suspended Solids,g/m^3        (F10.3)     1.0    -1.0    20.0     OFF      18

Carbonaceous Ultimate BOD, g/m^3              (F10.3)     1.0     5.0     9.0     OFF      19

pH                                            (F10.3)     1.0    -1.0    10.0     OFF      20

CO2                                           (F10.3)     1.0    -1.0    10.0     OFF      21

HCO3                                          (F10.3)     1.0    -1.0    10.0     OFF      22

CO3                                           (F10.3)     0.0     0.0     0.0     OFF      23

In Version 3.2, the user has format control of all output variables, as well as MULT control (see User Manual). In Version 3.1, some groups had one but not the other. Also, in Version 3.2, the groups (HNAME, CNAME, CDNAME) were reordered.

# Appendix B: BUG FIXES AND ENHANCEMENTS BETWEEN VERSIONS

There have been many updates and bug fixes between model versions. Although some model updates have not been documented, we have endeavored to be diligent in outlining code updates between model versions since Version 3.7. We have included below a series of tables with code fixes for multiple versions of CE-QUAL-W2 as a reference to earlier versions.

## W2 V4.1 Bug Fixes, Enhancements, and User Manual Changes

Table 2. Bug fixes and enhancements for Version 4.1.

| # | Code: W2 or PREW2 or GUI | Fix or Enhancement Type | Description of Bug/Enhancement | Date Fixed or Enhancement added |
| --- | --- | --- | --- | --- |
| 1 | PREW2 | Additional checks | Additional checks were added to the preprocessor for sediment decay temperature coefficients and stoichiometric coefficients. Concentration summaries in downstream and upstream head boundary conditions were added to the pre.opt file. | 5/19/2017 |
| 2 | W2 | DO Saturation | The equation for computing dissolved oxygen saturation was a function of elevation and temperature. If the user set the water body type to SALT, the TDS or salinity was used to correct the dissolved oxygen saturation. The TDS correction for dissolved oxygen saturation was also added to the freshwater computation. The new code is highlighted below: SATO = EXP(7.7117-1.31403*(LOG(T+45.93)))*P IF (SALT_WATER) THEN SATO = EXP(LOG(SATO)-SAL*(1.7674E-2-1.0754E1/(T+273.15)+2.1407E3/(T+273.15)**2)) ! SAL is in ppt ELSEIF(SAL > 100.)THEN SATO = EXP(LOG(SATO)-(SAL/1000.)*(1.7674E-2-1.0754E1/(T+273.15)+2.1407E3/(T+273.15)**2)) ! SAL is in mg/l ENDIF | 5/21/2017 |
| 3 | W2 | Sediment diagenesis output | Another line was added to the sediment diagenesis input file for the frequency of output. Prior to this, it used the TSR output frequency and wrote out duplicate results if there was more than one waterbody. | 5/25/2017 |
| 4 | W2 | Initialize variables | DLVOL, VOLTBR, EVBR, and QSUM were added to the initialized variables in INIT.F90. This only affects the Fortran compiler when it is in debug model. In the release executable, all variables are initialized to zero even if not explicitly set to zero. | 7/24/2017 |
| 5 | W2 | Sediment diagenesis code updates | We have deleted unused variables and array initializations. This has improved the speed of running the model with sediment diagenesis. These code areas are as follows: !SP CEMA if(sediment_diagenesis)then If(CEMARelatedCode .and. IncludeBedConsolidation)Call ComputeCEMARelatedSourceSinks ! If(CEMARelatedCode .and. IncludeCEMASedDiagenesis)Call ComputeCEMADiagenesisSourceSinks SW 6/27/2017 end if !End SP CEMA !SP CEMA !if(sediment_diagenesis)then ! If(CEMARelatedCode .and. IncludeBedConsolidation)TSS = 0.0 ! SW 7/27/2017 !end if !End SP CEMA !SP CEMA !if(sediment_diagenesis)then ! CEMATSSCopy = TSS !end if !End SP CEMA | 7/24/2017 |
| 6 | PREW2 | Sediment diagenesis | Additional error checking for the sediment diagenesis model was added to the preprocessor. In this case, whenever SOD was not set to zero, an error is displayed. | 7/24/2017 |
| 7 | W2 | Assorted code improvements | Stewart Rounds of the USGS suggested a few minor updates: extra right-parentheses eliminated in a format description for time series output (the Intel compiler allowed them!), WARNING_OPEN and ERROR_OPEN = .TRUE added in several cases where output is written to these files, and eliminated a situation where the derived output file at a withdrawal point was not written out if the file is empty. Also, for water age, evaporation should not concentrate the ‘age.’ Hence, code was added to recognize water age and to eliminate the effect of evaporation on water age. | 7/27/2017 |
| 8 | W2 | Branch active or inactive | In the W2 model, if a model branch became dehydrated, the model would not continue running. In order to allow for wide varieties of water levels, users would often have to add numerous deep fictitious layers to keep a branch hydrated. Now, the model can handle branches becoming active or inactive automatically. Code was added to allow branches to become active as they fill up or to become inactive if they lose their water. Also, any branch inflows or tributaries entering inactive branches are automatically moved to the current active segment of the nearest hydrated branch. | 7/27/2017 |
| 9 | W2 | RPOMN | Stewart Rounds, USGS, found this one. The if test below used NRPOMP rather than NRPOMN. Usually both NRPOMN and NRPOMP are both ‘ON,’ so for most applications, this should not affect the model user. Old Code: IF(CAC(NRPOMP) == ' ON')THEN IF(RPOM(K,I).GT.0.0)THEN ORGNRP(K,I)=RPOMN(K,I)/RPOM(K,I) New Code: IF(CAC(NRPOMN) == ' ON')THEN IF(RPOM(K,I).GT.0.0)THEN ORGNRP(K,I)=RPOMN(K,I)/RPOM(K,I) | 8/2/2017 |
| 10 | W2 | Screen Dialog Box | Under some unique conditions, exiting the W2 dialog box reinitializes some of the output files. Additional code was inserted to STOP program execution after closing the dialog box. | 8/23/2017 |
| 11 | W2 | Sediment Diagenesis | Flux rates for P, NH3, and NO3 were added to the MASSBAL output file from sediment diagenesis, so that a complete N and P balance can be evaluated for a waterbody. | 8/31/2017 |
| 12 | W2 | Particle Tracking | Particle tracking algorithm and documentation in a separate report have been added to the model release | 8/31/2017 |
| 13 | W2 | Opt to csv file | Changed flowbal.opt and massbal.opt to flowbal.csv and massbal.csv in order to facilitate opening in Excel. | 8/31/2017 |
| 14 | W2 | Gate file | The gate file was not converted to a csv format in the earlier Version 4.0 code. The 4.1 code was updated to include csv gate files. | 9/26/2017 |
| 15 | W2 | Shading | Code added to allow for a canopy shading in addition to dynamic shading. The DYNSH in the shade input file can now be a negative number between -1 and 0. This activates dynamic shading, and taking the absolute value of this number will reduce the short wave solar radiation by a fixed fraction, as if some of the channel has a canopy. Of course, canopy cover is more complex than this small correction, since it also affects long-wave radiation transfer. New code is highlighted below: SN = MIN (HT*ABS (SIN (ABS (PHI0(I)-AZ00)))/TAN (A0)-EDGE,BI(KT,I)) SFACT = SRED*SN/BI(KT,I) 100 CONTINUE SHADE(I) = MAX (0.0,1-SFACT) SHADE(I) = MIN(ABS(SHADEI(I)),SHADE(I)) ! SW 10/2/2017 Allows for fixed canopy cover over top of channel - only used if shade is less than SHADEI; only valid for -0.99 and 0.0. Hence, if DYNSH (or SHADEI) in the code is -0.9 and the dynamic shading algorithm computed the shade factor as 0.95 (which is a 5% reduction in short wave solar), the code uses 0.9, or a 10% reduction in short-wave solar. If the dynamic shade algorithm computes a shade greater than the fixed rate, the minimum of these is used. | 10/3/2017 |
| 16 | W2 | Sediment Diagenesis | Changed back to the original segment width at the bottom for sediment diagenesis, so that this algorithm replicates the original CEMA sediment diagenesis algorithm. CellArea, as a dimensioned variable, computed only once rather than for each cell at each time step. Also, a control variable was added to turn ON/OFF Bubbles calculation. This saves much computational time, and until the Bubbles subroutine is vetted, we do not recommend its use. Also, several code fixes were made in the sediment diagenesis module for mistakes in the original algorithm. | 10/3/2017, 10/22/2017 |
| 17 | W2 | User Manual | The User Manual was updated, fixing minor errors and typos and adding discussion of new features in Version 4.1. This is Revision 1 of the 4.1 Manual, but includes updates and fixed typos from the Version 4.0 Manual and explanations of new features. | 10/3/2017 |
| 18 | W2 | Kinetic fluxes for ADD Layer | A bug was corrected in the Kinetic Flux layer addition code as shown below: KFS(KT,I,KFCN(1:NAF(JW),JW)) = KFS(KT+1,I,KFCN(1:NAF(JW),JW)) !KF(KT+1,I,KFCN(1:NAF(JW),JW)) CODE ERROR FIX SW 10/24/2017 This does not affect fluxes in the TSR file, only the cumulative fluxes (KFS) during an add layer event for the surface layer only. | 10/24/2017 |
| 19 | W2 | Header | Changed header for flux for DO reaeration from just source to source/sink, which can occur during super-saturation. KFNAME(64) = 'DO reaeration - source/sink, kg/day ' | 10/24/2017 |
| 20 | W2 | Fluxes | In the file, kfl_wbX.opt where X is the waterbody number, the fluxes are presented in the same format as a snapshot file. The headers showed fluxes in kg/d but they were in kg. This has been corrected. The fluxes in kg/d in the file kflux_wbX.opt were already in the correct units of kg/d. Also, added the following code, since KT would have been from the prior waterbody rather than the current waterbody: DO JW=1,NWB KT = KTWB(JW) ! SW 10/25/2017 IF (FLUX(JW)) CALL KINETIC_FLUXES END DO | 10/24/2017 |
| 21 | W2 | Pumps | Changed some of the logic for pumps to avoid settings for older values influencing the current settings. The following code was added to hydroinout.f90: ILAT = 0 JWW = NWD withdrawals = jww > 0 if(nwdt>nwd)qwd(nwd+1:nwdt)=0.0 ! SW 10/30/2017 JTT = NTR tributaries = jtt > 0 if(ntrt>ntr)qtr(ntr+1:ntrt)=0.0 ! SW 10/30/2017 JSS = NSTR | 10/30/2017 |
| 22 | W2 | TDG at spillway | Since the new implementation of the TDG algorithm, the withdrawal.f90 algorithm at spillways had not been updated properly. This fix applies to the computation of TDG at spillways and gates. The old and new code are shown below: !if(tdgon)then ! cb 11/6/17 !cdavg(js,jb,16) = (cavg(js,jb,ndo)/exp(7.7117-1.31403*(log(tavg(js,jb)+45.93)))*palt(id))*100.0 dosat=exp(7.7117-1.31403*(log(tavg(js,jb)+45.93)))*palt(id) cdavg(js,jb,16)=(cavg(js,jb,ndo)/dosat)*100.0 IF(ngctdg /= 0)THEN EA = DEXP(2.3026D0*(7.5D0*TDEW(JW)/(TDEW(JW)+237.3D0)+0.6609D0))*0.001316 ! in mm Hg 0.0098692atm=7.5006151mmHg !cdavg(js,jb,NDC) = (cavg(js,jb,NGN2)/(1.5568D06*0.79*(PALT(ID)-EA)*(1.8816D-5 - 4.116D-7 * Tavg(js,jb) + 4.6D-9 * Tavg(js,jb)**2)))*100.0 ! SW 10/27/15 n2sat=1.5568D06*0.79*(PALT(ID)-EA)*(1.8816D-5 - 4.116D-7 * Tavg(js,jb) + 4.6D-9 * Tavg(js,jb)**2) cdavg(js,jb,NDC) = 100.*(0.79*(cavg(js,jb,NGN2)/n2sat) + 0.21*(cavg(js,jb,ndo)/dosat)) ENDIF !end if | 11/8/2017 |
| 23 | W2 Control | Updated GUI | A new version of W2Control was made to account for a large number of small refinements in the control file to bring it up to Version 4.1. Otherwise, some of these changes had to be implemented by editing the text file, w2_con.npt. Also, guidance was added to the release notes on how to use the GUI with a touch-screen laptop or desktop in cases where the touch screen does not work with the GUI. | 3/10/2018 |
| 24 | W2 | Csv file input for shade file | Updated reading in csv file for shade file to be compatible with preprocessor. Sometimes one needs to add an extra column of commas in a csv input file. New code: IF(INFORMAT=='$')THEN READ(SHD,'(/)') DO I=1,IMX READ (SHD,*)J,SHADEI(I) ! SW 3/14/2018 ADDED TO BE COMPATIBLE WITH PREPROCESSOR IF(SHADEI(I)<0.0)THEN BACKSPACE(SHD) READ (SHD,*) J,SHADEI(I),TTLB(I),TTRB(I),CLLB(I),CLRB(I),SRLB1(I),SRLB2(I),SRRB1(I),SRRB2(I),(TOPO(I,J),J=1,IANG),SRFJD1(I),SRFJD2(I) ENDIF ENDDO | 3/14/2018 |
| 25 | W2 | TSR file output | When the model user sets the elevation as a negative value, the model outputs variables at that layer only. When the water level went below that layer, the output was fixed at the old value of the variable until the water level rose into the layer. To eliminate issues with misinterpreting or having to edit out constant values, whenever the water level is below the layer, a -99 is now written out showing that no water is specified in the layer. | 4/5/2018 |
| 26 | PREW2 | More checks | Added checks for NaN in input files for meteorological files and flow, temperature and concentration files for inflows, distributed tributaries, precipitation, and tributaries. Previously, the preprocessor read input files even with NaN without reporting an error, since this is a proper numerical value. | 4/10/2018 |
| 27 | W2 | WDO output | The Withdrawal files are often used for downstream models. The withdrawal output frequency is in days. In order to make this more precise numerically, the output frequency can also be entered in hours and seconds. The problem was that 1 hour is 0.04167 days, and due to round off error for long term runs of many years, the hourly frequency output would not be at the same hour. The variable WDOC now can be ON/OFF/ONS/ONH, where ONS means the output frequency is in sec and ONH means the output frequency is in hours. The User Manual, GUI, and Preprocessor have been updated. | 4/10/2018 |
| 28 | Water balance | Water balance | A new water balance utility was released that is a console application and can be used in batch file processing. It also has more features than the old water balance utility, allowing the use of multiple waterbodies in the calculation of flows. There is a new file directory for this application in the download section as well as executables and a User Manual for this utility. | 4/10/2018 |
| 29 | PREW2 | More checks | Several more preprocessor enhancements were added to the preprocessor to check for Unix file types where an EOL is a <CR> and not the windows <CR><LF>. Also a few more descriptions were added to the error output to clarify where a problem may be. | 4/18/2018 |
| 30 | Water balance | Water balance | Some refinements to the water balance utility console application were made. | 6/5/2018 |
| 31 | W2 | Sediment Resuspension | The sediment resuspension algorithm computed sediment resuspension in all model layers except the surface layer. The fix below adds resuspension for the surface layer (highlighted code is new). IF (SEDIMENT_RESUSPENSION(J)) THEN FETCH = FETCHD(I,JB) IF (COS(PHI(JW)-PHI0(I)) < 0.0) FETCH = FETCHU(I,JB) FETCH = MAX(FETCH,BI(KT,I),DLX(I)) U2 = WIND(JW)*WSC(I)*WIND(JW)*WSC(I)+NONZERO COEF1 = 0.53 *(G*DEPTHB(KT,I)/U2)**0.75 COEF2 = 0.0125*(G*FETCH/U2)**0.42 COEF3 = 0.833* (G*DEPTHB(KT,I)/U2)**0.375 COEF4 = 0.077* (G*FETCH/U2)**0.25 HS = 0.283 *U2/G*0.283*TANH(COEF1)*TANH(COEF2/TANH(COEF1)) !TS = 2.0*PI*U2/G*1.2* TANH(COEF3)*TANH(COEF4/TANH(COEF3)) TS = 2.0*PI*sqrt(U2)/G*1.2* TANH(COEF3)*TANH(COEF4/TANH(COEF3)) ! cb 7/15/14 L0 = G*TS*TS/(2.0*PI) L1 = L0 ! SW 6/28/2018 Allow for resuspension of surface layer L = L0*TANH(2.0*PI*DEPTHB(KT,I)/L1) DO WHILE (ABS(L-L1) > 0.001) L1 = L L = L0*TANH(2.0*PI*DEPTHB(KT,I)/L1) END DO COEF = MIN(710.0,2.0*PI*DEPTHB(KT,I)/L) UORB = PI*HS/TS*100.0/SINH(COEF) TAU = 0.003*UORB*UORB IF (TAU-TAUCR(J) > 0.0) EPSILON = MAX(0.0,0.008/49.0*(TAU-TAUCR(J))**3*10000.0/DLT) SSR = EPSILON*DLX(I)*(BI(KT,I)-BI(KT+1,I))/VOL(KT,I) END IF SSSS(KT,I,J) = -SSS(J)*SS(KT,I,J)*BI(KT,I)/BH2(KT,I)+SSR | 6/28/2018 |
| 32 | W2 | Sediment Flux | ‘RECURSIVE’ added to the subroutine definition as it calls itself, and recent updates to the Intel compiler flagged this as an error. RECURSIVE Subroutine CEMASedimentDiagenesis Use MAIN Use GLOBAL Use GEOMC Use SCREENC | 6/28/2018 |
| 33 | PREW2 | Added checks | Assorted minor updates to error checks | 7/1/2018 |
| 34 | PREW2 | Consolidation | Update to checking code in sediment diagenesis for consolidation. New code: Read(5000,*)MessageTemp, LayerAddThkFrac Read(5000,*)MessageTemp, NumConsolidRegns if(IncludeBedConsolidation .and. NumConsolidRegns == 0)then ! CB 7/9/2018 changed .or. to .and. CALL ERRORS | 7/9/2018 |
| 35 | W2 | Sediment | The sediment model allows for first order sediment decay in the layers above the bottom layer. If a modeler set as an initial condition a finite value for the initial concentration of sediment, then the amount in layer kb would be there and not transfer eventually to the sediment diagenesis layer as downward sloughing occurs. Hence, if the model user sets an initial concentration of the first order sediment model and has turned on sediment diagenesis, then the code sets the value of sediment in layer kb to zero. The code fixe for SED is shown below: SED(KT,I) = SED(KT,I)/H2(KT,I) If(CEMARelatedCode .and. IncludeCEMASedDiagenesis)then ! cb 07/23/18 SED(KT+1:KB(I)-1,I) = SED(KT+1:KB(I)-1,I)/H2(KT+1:KB(I)-1,I) sed(kb(i),i)=0.0 else SED(KT+1:KB(I),I) = SED(KT+1:KB(I),I)/H2(KT+1:KB(I),I) end if END DO Similar code fixes were applied to SEDP, SEDC and SEDN. | 7/23/2018 |
| 36 | W2 | TSR output | When a branch became inactive, the TSR output file continued to write output information even though the segment was not hydrated. Logic was added to skip output for a TSR file if the segment was inactive or if the fixed layer was above the surface layer. DO J=1,NIKTSR I = ITSR(J) ! find out if segment is inactive OR cell is inactive for fixed layer - do not write out tsr file ! SW 7/24/2018 IF(BR_INACTIVE(JBTSR(J)))CYCLE IF(CUS(JBTSR(J)) > I)CYCLE DO JW=1,NWB IF (I >= US(BS(JW))-1 .AND. I <= DS(BE(JW))+1) EXIT END DO IF(ETSR(J) < 0)THEN ! SW 7/24/2018 IF(INT(ABS(ETSR(J))) < KTWB(JW))CYCLE ENDIF | 7/24/2018 |
| 37 | W2 | WRN | When a branch becomes ACTIVE, it is now written out to the WRN file (this was previously written out to the SNP file and only when INACTIVE to the WRN file). | 7/24/2018 |
| 38 | W2 | SO4 in Sediment | The code was updated to compute SO4 in both the aerobic and anaerobic layers and to allow diffusion from the water column to the sediment. This was previously not implemented. Also, updates were made to the bed consolidation routine in the sediment diagenesis model. | 8/1/2018 |
| 39 | W2, PREW2 | Constriction | This is a model enhancement: The model user can specify a maximum width between segments by specifying that in a new input file called ‘constriction.csv’. This does not affect the volume of the segments – it only affects the right hand side face width. Whereas an internal weir blocks all the flow, this allows for a reduced area and eliminates the need to insert a short segment of small width that can cause instability and lowered time step issues. The new input file is constiction.csv and has the following format: Line 1: Comment Line 2: # of constrictions Line 3: Comment Line 4: [Repeated by # of constrictions] Segment # of constriction, Maximum width in m of constriction # of Constrictions 1 Seg # Max width, m 10 115 The W2 code looks for constriction.csv – if it is found, then it reads the file and applies this to the right hand side width between two segments. Updates were made to the User Manual, Preprocessor and W2 Executables | 8/3/2018 |
| 40 | W2 | Restart | If there were WDO output files, the RESTART option may not have worked properly. A code fix was implemented for a RESTART with WDO output files. | 8/3/2018 |
| 41 | W2 | Envir Perf | The environmental performance output was updated to include the descriptive terms: Sum of fractions and Average. The User Manual was updated. | 8/6/2018 |
| 42 | W2 | Particle Transport | Tracking the time history of temperature, velocity and depth of particles was added to the particle transport algorithm. The file particle.csv now includes information on histogram output for each particle released into the waterbody. The User Manual and Preprocessor were also updated for this new feature. | 8/6/2018 |
| 43 | W2 | Layer subtract | When the W2 model adds a deep slot in shallow rivers, the variable AVHR in layeraddsub.f90 was defined for an inactive segment incorrectly. This code was cleaned up. New code is shown below: DEPTHM(KTWB(JW),I)=(H1(KTWB(JW),I)-(EL(KBI(I)+1,I)-EL(KB(I)+1,I)))*0.5 ! SW 1/23/06 IF(I<=DS(JB))THEN ! SW 8/6/2018 AVHR(KT,I)=(H1(KT,I)-(EL(KBI(I)+1,I)-EL(KB(I)+1,I))) +(H1(KT,I+1)-(EL(KBI(I)+1,I+1)-EL(KB(I)+1,I+1))& -H1(KT,I)+(EL(KBI(I)+1,I)-EL(KB(I)+1,I)))/(0.5*(DLX(I)+DLX(I+1)))*0.5*DLX(I) ELSE AVHR(KT,I)=AVHR(KT,I-1) ENDIF | 8/6/2018 |
| 44 | W2 | Input Bathymetry | In the bathymetry input file, if the model user gives the water surface (or FRIC, or PHI0, or DLX) of inactive segments a 0 value, the code in the first time step may set the KT layer incorrectly because of using ELWS of an inactive segment. The code, though, fixes it on the 2nd time step. This does not really affect the model run as it gets started, other than an unnecessary change in the surface layer designation. If the model user gave the inactive segments the same values as the active ones, the code ran normally from the first time step. In order to standardize this and allow the model user to keep inactive segment ELWS, DLX, FRIC, and PHI0 set to zero, these values are initialized with the neighboring segment value. READ (BTH(JW),'(//(10F8.0))') (DLX(I), I=US(BS(JW))-1,DS(BE(JW))+1) READ (BTH(JW),'(//(10F8.0))') (ELWS(I), I=US(BS(JW))-1,DS(BE(JW))+1) READ (BTH(JW),'(//(10F8.0))') (PHI0(I), I=US(BS(JW))-1,DS(BE(JW))+1) READ (BTH(JW),'(//(10F8.0))') (FRIC(I), I=US(BS(JW))-1,DS(BE(JW))+1) READ (BTH(JW),'(//(10F8.0))') (H(K,JW), K=1,KMX) DO I=US(BS(JW))-1,DS(BE(JW))+1 READ (BTH(JW),'(//(10F8.0))') (B(K,I), K=1,KMX) H2(:,I) = H(:,JW) END DO endif ! Set water surface of inactive segments to those active cells next to them SW 8/6/2018 DO JB=BS(JW),BE(JW) ELWS(US(JB)-1)=ELWS(US(JB)) ELWS(DS(JB)+1)=ELWS(DS(JB)) DLX(US(JB)-1)=DLX(US(JB)) DLX(DS(JB)+1)=DLX(DS(JB)) PHI0(US(JB)-1)=PHI0(US(JB)) PHI0(DS(JB)+1)=PHI0(DS(JB)) FRIC(US(JB)-1)=FRIC(US(JB)) FRIC(DS(JB)+1)=FRIC(DS(JB)) END DO | 8/6/2018 |
| 45 | W2 | Particle | Added the ability of particles to be passed to other branches by connections of gates, pumps or spillways between upstream and downstream branches aligned linearly along a river. | 8/7/2018 |
| 46 | W2 | Flow Balance Output | The flow balance output file, flowbalance.csv, output was affected by Branches becoming active or inactive. Code was revised in balances.f90 to allow for the flow balances to properly account for cumulative flow sources/sinks. IF (VOLUME_BALANCE(JW)) THEN DO JB=BS(JW),BE(JW) IF(.NOT.BR_INACTIVE(JB))THEN ! SW 8/8/2018 VOLSBR(JB) = VOLSBR(JB)+DLVOL(JB) VOLTBR(JB) = VOLEV(JB)+VOLPR(JB)+VOLTRB(JB)+VOLDT(JB)+VOLWD(JB)+VOLUH(JB)+VOLDH(JB)+VOLIN(JB)+VOLOUT(JB)+VOLICE(JB) if(sediment_diagenesis)then If(CEMARelatedCode .and. IncludeBedConsolidation)Then VOLTBR(JB) = VOLTBR(JB)+ VOLCEMA(JB) End If ENDIF IF (ENERGY_BALANCE(JW)) THEN ESR(JW) = 0.0 ETR(JW) = 0.0 DO JB=BS(JW),BE(JW) IF(BR_INACTIVE(JB))CYCLE ! SW 8/8/2018 IF (MASS_BALANCE(JW)) THEN DO JB=BS(JW),BE(JW) IF(BR_INACTIVE(JB))CYCLE ! SW 8/8/2018 DO JC=1,NAC | 8/8/2018 |
| 47 | W2 | Particle Transport | In the particle tracking algorithm, the depth statistics of are now based on the particle depth. Before this, the depth statistics of a particle were based on water depth, not particle depth. | 8/24/2018 |
| 48 | W2 | Long Profile Output | The volume weighted temperature was added to the longitudinal profile output. The User Manual was also updated. WRITE(NUNIT,'(1000(A,","))')'Seg#','ElevWaterSurf(m)','Q(m3/s)','SurfaceTemp(oC)','Depth(m)','Width(m)','VolWeighTemp(oC)' ,(CNAME2(CN(JC)),JC=1,NAC),(CDNAME2(CDN(JD,JW)),JD=1,NACD(JW)) ELSE WRITE(NUNIT,'(1000(A,","))')'Seg#','ElevWaterSurf(m)','Q(m3/s)','SurfaceTemp(oC)','Depth(m)','Width(m)','VolWeighTemp(oC)' ENDIF DO JJ=1,NWB K=KTWB(JJ) DO JB=BS(JJ),BE(JJ) DO I=CUS(JB),DS(JB) ! TEMP VOL WEIGHTED AVERAGE SW 8/30/2018 TVOLAVG=0.0 ! SW 8/30/2018 VOLTOT=0.0 DO KK=KTWB(JW),KB(I) VOLTOT=VOLTOT+VOL(KK,I) TVOLAVG=TVOLAVG+T1(KK,I)*VOL(KK,I) ENDDO IF(KB(I)>=KTWB(JW))tvolavg=tvolavg/voltot IF(CONSTITUENTS)THEN WRITE(NUNIT,'(I5,",",100(F12.3,","))')I,ELWS(I),QC(I),T2(K,I),DEPTHB(KB(I),I),B(KTI(I),I),TVOLAVG,(C2(K,I,CN(JAC))*CMULT(CN(JAC)),JAC=1,NAC),(CD(K,I,CDN(JD,JW))*CDMULT(CDN(JD,JW)),JD=1,NACD(JW)) ELSE WRITE(NUNIT,'(I5,",",100(F12.3,","))')I,ELWS(I),QC(I),T2(K,I),DEPTHB(KB(I),I),B(KTI(I),I),TVOLAVG | 8/30/2018 |
| 49 | W2 | SPR output | The spreadsheet output file now has an option to output volume weighted values at the locations and times specified. If the user enters ‘ONV’ in the control file in the line SPR PLOT SPRC NSPR NISPR WB 1 ONV 1 2 the output will include the regular SPR output and a separate file with the suffix ‘_volw.csv’ for the volume- weighted output. The User Manual has also been updated with the new feature. | 9/28/2018 |
| 50 | W2 | User Manual | Updated the LPOM, LPOM-P, and LPOM-N source sink equations and LPOM source sink stencil to agree with the model code. These were old and did not reflect the addition of zooplankton ingestion and excretion/mortality. | 10/9/2018 |
| 51 | W2 | Particle Tracking | Added an enhancement to add monitoring segments that record when the particle passes by the monitor or sensor. This would be useful for comparing to a dye study in a river, for example. | 11/14/2018 |
| 52 | W2 | SPR output | A volume weighted output bug for item 49 was fixed. | 11/14/2018 |
| 53 | W2 | TSR flux output units | If flux output was ON, the instantaneous flux values for TSR output were in the incorrect units. These units were corrected to kg/day. Thanks to Binglei Gong at Anchor! No other flux files units were affected. | 12/10/2018 |
| 54 | W2 | Volume Balance | The volume balance when segments were added and subtracted because of a shallow location in the grid caused the code to incorrectly compute the volume balance (if VBC is ON). This was corrected. | 12/18/2018 |
| 55 | W2 | Internal solar radiation and angle of sun for shading | In the internal solar radiation algorithm, if one used reference years many years from your simulation year, or made a simulation over 20 years, the solar maximum each year drifts. If you read in solar radiation, there are no issues, or make runs from 1-10 years, the drift is probably minimal. This mainly affects those who use a reference year far from the start date or run simulations over 20 years or longer while using the internal short wave solar algorithm. Fix from Dan Turner at USACE and Stewart Rounds USGS – many kudos!! | 12/18/2018 |
| 56 | W2 | Energy and Mass balance | For a simulation with an initial BRANCH INACTIVE, the energy and mass balance when ON (EBC and MBC set to ON) starts off incorrectly. In some cases where there were deeper holes followed by shallows, the layer subtraction algorithm incorrectly computed the energy and constituent mass balance when segments were subtracted. Also, some changes were made for the heat balance and mass balance for constituents when branches were changed from active to inactive. | 12/18/2018 |
| 57 | W2 | Particle tracking | Fixed a bug when a particle was in the “air” above the water surface when in a river a ‘deep slot’ was added to the river bathymetry to keep it hydrated. Also, fixed a reflection error off the surface for vertical movement of the particle. | 12/26/2018 |
| 58 | W2 | Particle tracking | For file output for finalparticle.csv, adjusted header so that it is general for any number of monitoring stations. Previously the header was hard-wired to write only 3 monitoring locations in the title. This does not affect prior model output information, but only the title header. | 1/3/2019 |
| 59 | PREW2 | Pipe invert | The preprocessor gave a ‘false’ error for an invert elevation being below the grid in rare cases. This has been fixed. The model was not affected. | 1/21/2019 |
| 60 | W2 | TN | There was a bug in the calculation of TN when large concentrations of zooplankton were present. If zooplankton were not simulated or they were a small population, there would be no impact of this TN bug. Bug fixed. | 1/29/2019 |
| 61 | W2 | Epiphyton periphyton burial and conversion to LPOM and sediments | The User Manual showed that the epiphyton parameter EB, burial rate, was in units of m/day. The model implemented this and also computed the cell layer burial rate as EB*EC(concentration of biomass per cell volume)*Surface area/Volume of a cell or EB*EC/H, where H is the layer thickness. In reality, though, it should be the thickness of the periphyton layer. But that layer thickness is not predicted by the model. Hence, the burial rate was changed to units of 1/day, such that the loss of epiphyton by burial would follow a first order decay process (similar to the first order sediment model) as EB*EC. Hence, this burial rate is no longer a function of the grid. Also, there was the conversion of organic matter and nutrients at death to the sediment P, N and C compartments whereas they should go directly to LPOM first. Hence only burial goes directly into the sediment compartments. | 3/5/2019 |
| 62 | W2 | Output files | Code was written to print the initial condition in output files at time t=0 at the start of the simulation. | 3/5/2019 |
| 63 | W2 | Restart with Sediment Diagenesis | Prior to this date, the restart option did not work with sediment diagenesis turned ON. This enhancement has been coded and debugged and is now working. | 3/5/2019 |
| 64 | W2 | Dynamic loading | This is a new feature in CE-QUAL-W2, and applies to linear waterbodies in series. For example, if one has a river system with nine waterbodies, the model user can break the model into nine separate models and run them all in parallel, where the downstream model dynamically updates its boundary conditions as the upstream model moves forward in time. This has resulted in significant time savings for model runs (90%). Documentation for this will be provided shortly. | 3/5/2019 |
| 65 | W2 | User Manual | The existing User Manual will be frozen in its current version until a newly updated user manual is released. Hence, the dynamic loading described above will be described in the new User Manual. | 3/5/2019 |

## W2 V4.0 Bug Fixes, Enhancements, and User Manual Changes

Table 3. Bug fixes and enhancements for Version 4.0.

| # | Code: W2 or PREW2 or GUI | Fix or Enhancement Type | Description of Bug/Enhancement | Date Fixed or Enhancement added |
| --- | --- | --- | --- | --- |
| 1 | PREW2 | Additional model checks | Additional model checks were added for Profile and Spreadsheet output model segments | 6/7/16 |
| 2 | User Manual | Updated | User Manual Rev 6 was released with many minor updates and better explanatory text | 6/7/16 |
| 3 | W2 | Restart | Fixed restart to work for epiphyton and macrophytes. This was broken in case a model user used RESTART. Fixed Restart for mass balance for nutrients output in the file massbal.opt. | 6/7/16 |
| 4 | W2 | Location of compiler info file | Fixed location of W2 compiler information in case of using command line aware directory. File was written to the location of the model executable rather than the command line aware directory. | 6/7/16 |
| 5 | Water balance | Update for Version 4 | The water balance utility uses a model tsr file for reading in water level over time. Since the Version 4 file format was updated with comma delimited output files, the water balance utility has been updated. This utility is not compatible with earlier versions. | 6/10/16 |
| 6 | W2 | Sediment Diagenesis | Initialized the sediment width (sedcellwidth) in subroutine CEMASedimentDiagenesis. | 6/11/16 |
| 7 | W2 | Screen output | The text fields in the Windows dialog box may ‘overflow’ if you have more than 160 tributaries. The field size was increased to avoid this possibility. Old code: CHARACTER(1000) :: TEXT1 New code: CHARACTER(1700) :: TEXT1 | 6/24/16 |
| 8 | W2 | Profile output | The longitudinal profile output added depth at a segment as part of the longitudinal output. User Manual updated also. | 7/11/2016 |
| 9 | W2 | Profile output | Changed file name of longitudinal file output from integer of the Julian day to Julian day in F8.2 format in case of multiple outputs on one day | 7/16/2016 |
| 10 | W2 | TSR output | Changed TSR file so that the first 11 lines of header are eliminated to facilitate graphing. Also, the name of the filetype in the control file is now read and used for the output file. Hence, using the TSR FILENAME of ‘tsr.csv’ will produce csv files that are immediately opened in Excel for viewing again making it easier for post-processing. | 8/1/2016 |
| 11 | PRE | Met file checks | The preprocessor has been enhanced with more model file checks. This program now has summaries of meteorological data (min, max, average) for each waterbody in the pre.opt file as well as further logical checks on values of these averages. These summaries are another check on the correctness of the input met data file. A typical result in pre.opt is shown below: Meteorological Data Input Summary Parameter Waterbody Average Value Maximum Minimum TAIR(C) 1 10.553 37.780 -11.940 TDEW(C) 1 6.935 19.500 -17.670 WIND(m/s) 1 1.337 12.440 0.000 PHI(rad) 1 3.426 6.280 0.000 CLOUD(0-10) 1 7.367 9.720 0.000 SRO(W/m2) 1 0.000 0.000 0.000 TAIR(C) 2 10.553 37.780 -11.940 TDEW(C) 2 6.935 19.500 -17.670 WIND(m/s) 2 1.337 12.440 0.000 PHI(rad) 2 3.426 6.280 0.000 CLOUD(0-10) 2 7.367 9.720 0.000 SRO(W/m2) 2 0.000 0.000 0.000 | 10/30/16 |
| 12 | PRE | Distributed concentration checks | Added checks for average, min, and max inflow concentrations for all distributed tributaries. These are written out to the pre.opt file | 11/1/16 |
| 13 | PRE | LPR input | For LPR file inputs for temperature, the preprocessor reports an error when using LPR input. The code incorrectly used KT rather than KTWB(JW). [This also affects V3.7 preprocessor.] | 11/9/2016 |
| 14 | W2 | Model update | The model executables were updated from Intel Fortran Compiler #14 to Intel Fortran compiler #17. Also, the flag to initialize all variables to zero was enforced. There are many variables in the new sediment diagenesis model that need to be explicitly set to zero. These initializations will be made in the code in the future so that setting this flag will be unnecessary. | 11/17/2016 |
| 15 | W2 and PRE | Code updates | A couple code updates were made as a result of using the Intel Fortran Version 17 compiler. The new compiler did not like some of the older implementations. These were minor updates. | 11/22/2016 |
| 16 | W2 | Output | Improved clarity of output headers for flux outputs, including units of kg/d in all header titles | 11/28/2016 |
| 17 | W2 | Output header | The order of the flux headings in the file kflux_jwX.opt were switched. The header showed DOAP, DOAR, DOEP, DOER, but it should have been DOAP, DOEP, DOAR, DOER. This is determined from the order in the example problem control files and the User Manual. The example problems and User Manual have all been updated. | 11/28/2016 |
| 18 | W2 | Example problems | Updated example problems using FIX #10 above where tsr.opt filename was changed to tsr.csv allowing tsr files to open directly in Excel. | 11/28/2016 |
| 19 | W2 | Algae-Si | The flux of Si from dying algae was incorrectly computed. This bug has existed since Version 3.0, when the algorithm was first added to W2. Below is the code fix: ENTRY PARTICULATE_SILICA PSIAM(:,IU:ID) = 0.0 DO I=IU,ID DO K=KT,KB(I) DO JA=1,NAL IF(ALG_CALC(JA))THEN PSIAM(K,I) = PSIAM(K,I)+AMR(K,I,JA)*ALG(K,I,JA)*ASI(JA) ! PSI(K,I) HA-Z 12/2016 ENDIF | 1,2/5/2016 |
| 20 | W2 | WDO output | Enhancement: The Withdrawal output file name WDOFN was unused in the main program. Now the model reads this file and uses the file type for all WDO output files. Previously, this was hard-wired to ‘opt’ output. Now, if the user sets WDOFN to wdo.csv, all the files will be written with the ‘csv’ file type, facilitating opening in Excel. The files are already in comma delimited format. | 12/8/2016 |
| 21 | W2 | DLT INTER | There was a problem computing the interpolated value of DLTMAX and DLTF when the first value of DLTD was earlier than the start date of the model. This bug was fixed. | 12/9/2016 |
| 22 | User Manual | Updates | Assorted typos fixed, and better explanatory text, definitions and units of model parameters added. This is REV8. | 1/6/2017, 2/10/2017 |
| 23 | W2 | Output format | Output format changed for Bioenergetics output file | 1/6/2017 |
| 24 | W2 | TECPLOT output | Added derived variables to TECPLOT output files (See Contour Plot in User Manual). User Manual updated. | 1/17/2017 |
| 25 | PREW2 | ENVIRPC | Checks added for the ENVIRPC input file in the preprocessor. | 2/16/2017 |
| 25 | W2 | ENVIRPC | Fixed several minor bugs in the ENVIRPC subroutine and added an enhancement to perform a histogram analysis of water depth. The User Manual was updated to reflect this new enhancement as well as the new csv output format and file names. The example problems were updated with new w2_envirprf.npt files. | 2/16/2017 |
| 26 | W2 | Head BC input files | For head boundary condition input files (both upstream and downstream), the W2 code was updated to include new file formats for these boundary conditions (BCs). They include the older format, a new csv format and a new csv format in case conditions are not stratified at the BC. The User Manual was updated to show these new file formats. | 3/3/2017 |
| 27 | PREW2 | Head BC checks | With the new file format for head BCs in #26 above, the preprocessor was updated to check these new input file formats. Also, additional checks were added to the head BCs. | 3/3/2017 |
| 28 | Water balance | Bug fixes/updates | The water balance utility was updated because of the new input format of TSR output files. See fix #10. Also, a bug was fixed in this code that affected cases when the water level was above the top of the grid. | 3/17/2017 |
| 29 | W2 | CPL Tecplot | The CPL Tecplot output sometimes did not update the month in the contour plot text files. This has been fixed – thanks to Jung Ma, Hubei University of Technology in Wuhan, for finding it! | 4/4/2017 |
| 30 | Water balance | Water balance manual | The water balance manual was updated for Version 4. | 4/14/2017 |
| 31 | W2 | TSR output | Refined the TSR output so that flux terms that were not specified are no longer written out. This cleans up the TSR output and reduces the active number of flux variables when sediment diagenesis is not on. | 4/15/2017 |
| 32 | W2 | Derived variables | Fixed a code regression for derived variable TDG when the user stopped the code and pressed restart | 4/15/2017 |
| 33 | W2 | Withdrawal output | When a user pressed restart and he/she specified withdrawal output files, the restarted files ignored the filetype of the WDO specification in the control file and used ‘opt’. | 4/15/2017 |

## W2 V3.7 Bug Fixes, Enhancements and User Manual Changes

Table 4. Bug fixes and enhancements for Version 3.7.

| # | Code: W2 or PREW2 or GUI | Fix or Enhancement Type | Description of Bug/Enhancement | Date Bug Fixed or Enhancement added |
| --- | --- | --- | --- | --- |
| 1 | W2 | Fish habitat limits | Changed temperature and DO criteria from t2(k,i)<fishtemph(ii).and.t2(k,i)>fishtempl(ii).and.o2(k,i)>fishdo(ii) to t2(k,i)<=fishtemph(ii).and.t2(k,i)>fishtempl(ii).and.o2(k,i)>=fishdo(ii) This update is reflected in the manual. Hence the high temperature limit and the dissolved oxygen minimum is less than or equal to given value rather than less than. | 8/7/2012 |
| 2 | W2 | Structure, gate, pump, pipe, withdrawal output files | Added code to ensure that if flow is ‘0’ in an outlet structure, that the corresponding temperature and concentration in the outlet file is written as ‘-99.0’. Previously this was not fully implemented in the code. Code such as this was inserted in several places in the subroutine outputa2.f90: IF(QGT(JS)==0.0)THEN TAVGW(JWD)=-99.0 CAVGW(JWD,:)=-99.0 CDAVGW(JWD,:)=-99.0 ENDIF | 8/13/2012 |
| 3 | PREW2 | Format updates | Several output updates were made for warnings and errors | 8/16/2012 |
| 4 | Resource files for W2 | Compiling files | Updated some corrupted resource files that were used to compile the source code. Also, zipped up source code and compiler settings together so that file locations are correct for using the Intel compiler. | 9/12/2012 |
| 5 | W2 and PREW2 | Read csv files | By inserting the character ‘$’ as the first character of the first line, the following files can now be read in free-format or csv format: met, lpr, vpr, wsc, met, cin, ctr, cdtr, cpre, qot, and qwd. This is described in a Word document that accompanies the download package. The preprocessor has also been updated for file checks. This is part of the Version 3.71 update. | 9/12/2012 |
| 6 | W2 | Read input file | An input format bug was fixed for a system with more than 9 waterbodies. DO JD=1,NDC !READ (CON,'(A8,(:9A8))') CDNAME2(JD),(CDWBC(JD,JW), JW=1,NWB) READ (CON,'(A8,(:9A8):/(8X,(:9A8)))') CDNAME2(JD),(CDWBC(JD,JW), JW=1,NWB) !cb 9/13/12 END DO READ (CON,'(/)') ! DO JF=1,NFL do jf=1,73 ! Fix this later !READ (CON,'(A8,(:9A8))') KFNAME2(JF),(KFWBC(JF,JW), JW=1,NWB) READ (CON,'(A8,(:9A8):/(8X,(:9A8)))') KFNAME2(JF),(KFWBC(JF,JW), JW=1,NWB) !cb 9/13/12 END DO This had the effect of turning OFF output for derived constituents for waterbody 10. | 9/13/2012 |
| 7 | GUI | Time series elevation | The GUI read in values of ETSR as integers rather than real numbers. This was fixed. | 10/30/12 |
| 8 | W2 | Spillways Lateral | Lateral spillways when connected to other model segments were sometimes not connecting as a tributary to the downstream segment. This has been fixed. | 10/30/12 |
| 9 | W2 | W2Tools output | In place of the Vector Plot Output (VPL), a new output was added that allows use of the W2Tools post-processing package. This is part of the Version 3.71 update. | 10/30/12 |
| 10 | W2 | User Manual | The User Manual has been updated with the new model features as shown in 5 and 9 above. In addition, a separate user manual file shows how to use the w2tools post-processor. This is in the directory for W2tools. This is the version 3.71 update. | 10/30/12 |
| 11 | W2 | Water quality and temperature | A new calculation technique was added that eliminates calling the Tri-diagonal subroutine. These were built into the temperature and water quality subroutines. This change results in improvements in computational speed of from less than 5% to over 20% for water quality models with lots of water quality state variables. | 10/30/2012 |
| 12 | PREW2 | More checks | Added more error trapping for input files. This is an effort for the error trapping to occur before the code bombs. Fixed a couple of regression errors as a result of this fix. | 11/2/2012, 11/5/2012 |
| 13 | Excel macro utility |  | Added an Excel macro utility to aid in writing out input files to CE-QUAL-W2 | 11/5/2012 |
| 14 | W2 | Withdrawal subroutine | Fixed an IF test that used the wrong variable in the dynamic port allocation algorithm. Also added code to allow the code to test for temperatures at the outlet levels specified. Deleted line of code is underlined, followed by the fix. DO J=1,NUMTSPLT !REODERING OUTLETS SO THAT HIGHEST ELEVATION STRUCTURE ON TOP (ASSUMING 2 SPLIT OUTLETS) ! IF(TCNTR(J) == ' ST')THEN IF(TSPLTCNTR(J) == ' ST')THEN ! cb 11/11/12 IF(ESTR(JSTSPLTT(J,1),TSPLTJB(J)) < ESTR(JSTSPLTT(J,2),TSPLTJB(J)))THEN JSTSPLT(J,1)=JSTSPLTT(J,2) JSTSPLT(J,2)=JSTSPLTT(J,1) END IF ! ELSE IF(TCNTR(J) == ' WD')THEN ELSE IF(TSPLTCNTR(J) == ' WD')THEN ! cb 11/11/12 IF(EWD(JSTSPLTT(J,1)) < EWD(JSTSPLTT(J,2)))THEN … IF(TSPLTJB(J) == JB .AND. TSPLTCNTR(J) == ' ST')THEN QALL=0.0 DO JJ=1,NOUTS(J) QALL=QALL+QSTR(JSTSPLT(J,JJ),TSPLTJB(J)) ! SUM UP ALL THE FLOWS ELR = SINA(JB)*DLX(DS(JB))*0.5 DO K=KTWB(JW),KB(DS(JB)) IF (EL(K,DS(JB))-ELR < ESTR(JSTSPLT(J,JJ),TSPLTJB(J))) EXIT !SW 10/17/01 END DO KSTR = K-1 KSTRSPLT(JJ) = MIN(KSTR,KB(DS(JB))) ENDDO DO JJ=1,NOUTS(J) ! cb 11/11/12 dividing total flow between outlets for temperature test QSTR(JSTSPLT(J,JJ),TSPLTJB(J)) = qall/real(nouts(j)) ENDDO | 11/13/12 |
| 15 | W2 | Reading in names of WQ variables | In case a user does not enter the units in graph.npt, the code improperly parses the WQ variable name. In this case the output name is a blank. To avoid this issue, extra code was added to preserve the variable name even if no units were added to the graph.npt list. L1 = SCAN (CNAME(JC),',')+2 IF(L1 == 2)L1=43 ! SW 12/3/2012 Implies no comma found L2 = SCAN (CNAME(JC)(L1:43),' ')+L1 IF(L2 > 43)L2=43 ! SW 12/3/2012 CUNIT(JC) = CNAME(JC)(L1:L2) CNAME1(JC) = CNAME(JC)(1:L1-3) CNAME3(JC) = CNAME1(JC) DO WHILE (L3 < L1-3) | 12/3/2012 |
| 16 | PREW2 | SEDS and SEDK | The variable names were switched in reading the control file in the preprocessor perhaps leading to incorrect warnings/errors being tagged. The proper order was restored: !READ (CON,'(/A8/(8X,2A8,6F8.0,A8))', ERR=400) AID, (SEDC(JW), PRNSC(JW), SEDCI(JW), seds(jw), SEDDK(JW), FSOD(JW), & ! FSED(JW), sedbr(jw), DYNSEDK(JW), JW=1,NWB) ! SW 6/1/07 READ (CON,'(/A8/(8X,2A8,6F8.0,A8))', ERR=400) AID, (SEDC(JW), PRNSC(JW), SEDCI(JW), SEDDK(JW),seds(jw), FSOD(JW), & FSED(JW), sedbr(jw), DYNSEDK(JW), JW=1,NWB) ! cb 12/30/12 | 12/30/12 |
| 17 | Excel macro utility w2tool | Integer/Long variables | Some loose ends were corrected in the Visual Basic code built into the Excel macros. | 1/2/2013 |
| 18 | W2 | TDG output | A series of code changes were made to fix some issues that arose for computing the impact of a structure on downstream TDG. These fixes were made in subroutines Withdrawal, outputa2w2tools, w2modules, and hydroinout. These affected calculation of output of dissolved gas concentration for output files for spillways or gates that had dissolved gas equation. | 1/23/2013 |
| 19 | W2 | Reading in dynamic extinction coefficient | For temperature only studies, the model did not update the dynamic light extinction coefficient correctly. This has been fixed by the added code below: DO JW=1,NWB IF (READ_EXTINCTION(JW))GAMMA(:,US(BS(JW)):DS(BE(JW))) = EXH2O(JW) ! SW 1/28/13 KT = KTWB(JW) IF (.NOT. NO_HEAT(JW)) THEN | 1/28/2013 |
| 20 | W2 | Input format when 9 WBs | A specific input read error occurred when nine waterbodies were present as a result of an earlier bug fix: The new read statements occur in 2 places: READ (CON,'(A8,9A8,/(:8X,9A8)))') CDNAME2(JD),(CDWBC(JD,JW), JW=1,NWB) !cb 9/13/12 sw 2/18/13 READ (CON,'(A8,9A8,/(:8X,9A8)))') KFNAME2(JF),(KFWBC(JF,JW), JW=1,NWB) !cb 9/13/12 sw2/18/13 | 2/18/13 |
| 21 | PREW2 | More checks added | Added Additional checks to warn users of gaps in meteorological data when interpolation may be inappropriate. | 2/20/2013 |
| 22 | W2 User Manual | Updated | Updated User Manual – many small additions and edits – REV3. | 2/20/2013 |
| 23 | PREW2 | Improved an error check | Updated an error check for choosing inactive segments for ISNP output. | 3/21/2013 |
| 24 | PREW2 | More checks added | Added checks for inflow temperature and tributary temperatures. | 3/28/2013 |
| 25 | W2 | Initial WL Calculation | Changed SLOPE to SLOPEC in init—u-elws.f90 routine since the normal depth should be based on SLOPEC. END IF FUNCVALUE=FLOW-XAREA*HRAD**0.6667*SLOPEC(JB)**0.5/FMANN ! SW 4/5/2013 RETURN END SUBROUTINE MANNINGS_EQN Also changed KB(I)-1 to KB(I)+1 for ELWS: IF(ABS(DX).LT.XACC .OR. FMID.EQ.0.)THEN ELWS(I)=RTBIS+EL(KB(I)+1,I) ! SW 4/5/13 RETURN And changed KTTOP from REAL to an INTEGER: REAL :: XAREA, WSURF ! 4/5/13 SW INTEGER :: KTTOP ! 4/5/13 SW | 4/5/2013 |
| 25 | W2 | Output for pumps, spillways, gates | If the LAT option was chosen, the output files index for JWD was incorrect. This may have affected output temperatures and concentrations. | 5/17/2013 |
| 26 | PRE-W2 | Mass loading calculation | There were cases where the preprocessor bombed while calculating the mass loading for output to the pre.opt file. This error has been fixed. | 6/21/2013 |
| 27 | W2 | Assorted code updates | Minor format errors (that were ignored by compiler), update to code comments, and faster code initializations to speed up model performance were performed in several subroutines: input_PAR.f90, temperature_PAR.f90, transport_PAR.f90, update.f90, and w2_37_win.f90. An example of an initialization code speed up from temperature_PAR.f90: New code: DO K=KT,KB(I) AT(K,I) = 0.0D0; CT(K,I) = 0.0D0; VT(K,I) = 0.0D0 ! SW CODE SPEEDUP 6/15/13 ENDDO Old code: AT(:,I) = 0.0D0; CT(:,I) = 0.0D0; VT(:,I) = 0.0D0 | 6/21/2013 |
| 28 | W2 tools Excel macro | Update | More robust tools release | 6/21/2013 |
| 29 | PRE-W2 | Label error | A label error for one spillway error was fixed. It mistakenly used ‘gate’. | 7/2/2013 |
| 30 | W2 | CPL output | A slight change in output format for the ‘raw’ cpl output file format was made. No change was made in the tecplot output format. DO I=CUS(JB),DS(JB) WRITE (CPL(JW),'(A38/(9(F10.3,2X)))') CDNAME(CDN(JD,JW)),(CD(K,I,CDN(JD,JW))*CDMULT(CDN(JD,JW)), K=KTWB(JW),KB(I)) ! cb 6/28/13 end do !WRITE (CPL(JW),'(A38/(9(F10.3,2X)))') CDNAME(CDN(JD,JW)),((CD(K,I,CDN(JD,JW))*CDMULT(CDN(JD,JW)), & ! SW 8/12/06 !K=KTWB(JW),KB(I)),I=CUS(JB),DS(JB)) ! CB 1/03/05 | 7/31/13 |
| 31 | W2 | Read input file | A regression error that cropped up when there were 9 or greater than 10 waterbodies has been fixed. This had to do with reading in derived and flux variables in the control file. DO JD=1,NDC If(nwb < 10)READ (CON,'(A8,(:9A8))') CDNAME2(JD),(CDWBC(JD,JW), JW=1,NWB) If(nwb >= 10) READ (CON,'(A8,9A8,/(:8X,9A8))') CDNAME2(JD),(CDWBC(JD,JW), JW=1,NWB) !cb 9/13/12 sw 2/18/13 6/16/13 END DO READ (CON,'(/)') ! DO JF=1,NFL do jf=1,73 ! Fix this later If(nwb < 10)READ (CON,'(A8,(:9A8))') KFNAME2(JF),(KFWBC(JF,JW), JW=1,NWB) If(nwb >= 10) READ (CON,'(A8,9A8,/(:8X,9A8))') KFNAME2(JF),(KFWBC(JF,JW), JW=1,NWB) !cb 9/13/12 sw2/18/13 6/16/13 | 8/13/13 |
| 32 | W2 | New compiler | Upgraded to the Intel XE 13.1.3.198 compiler. New W2 executables for 32 bit and 64 bit. | 8/13/13 |
| 33 | W2 | INIT WL | An error was fixed in the initial water level computation program for rivers. The code below should have the subscript JB instead of J. DO JJW=1,NWB DO JJB=BS(JJW),BE(JJW) IF(DHS(JB) > US(JJB) .AND. DHS(J) < DS(JJB))THEN JBD=JJB END IF END DO | 8/20/13 |
| 34 | W2 | INIT WL | There was an index error with gates in the initial water level computation. Old code: IF(ELWS(ID) < WSUP)THEN IF(ELWS(IDSP(JS)) > WSUP) WSUP = ELWS(IDSP(JS)) ! CHECKING TO SEE IF DOWNSTREAM WS ELEVATION ISN'T ALREADY 'HIGH' ELWS(ID)=WSUP New code: IF(ELWS(IDGT(JG)) > WSUP) WSUP = ELWS(IDGT(JG)) ! CHECKING TO SEE IF DOWNSTREAM WS ELEVATION ISN'T ALREADY 'HIGH' WX 8/21/13 | 8/21/2013 |
| 35 | W2 | GATE | Cleaned up some code in the gate algorithm. Old code: IF (A2GT(JG) /= 0.0 .AND. IDGT(JG) /= 0.0) THEN New code: IF (A2GT(JG) /= 0.0 .AND. IDGT(JG) /= 0) THEN | 8/21/2013 |
| 36 | W2 | TSS computation | Updated the computation for the derived variable TSS to include zooplankton and the particulate form of CBOD. A formula was added to the User Manual reflecting this change. New code: IF(CBODS(IBOD)>0.0)TOTSS(K,I) = TOTSS(K,I)+CBOD(K,I,IBOD)/O2OM(JW) ! SW 9/5/13 Added particulate CBOD to TSS computation TOTSS(K,I) = TOTSS(K,I)+ZOO(K,I,JZ) ! SW 9/5/13 Added zooplankton to TSS computation | 9/6/2013 |
| 37 | W2 | Spillway-LAT | When a spillway was defined with IDSP=0 and LAT, a tributary was defined incorrectly. New code: IF (IDSP(JS) /= 0)then ! cb 9/11/13 JTT = JTT+1 QTR(JTT) = QSP(JS) ITR(JTT) = IDSP(JS) PLACE_QTR(JTT) = PDSPC(JS) == ' DENSITY' SPECIFY_QTR(JTT) = PDSPC(JS) == ' SPECIFY' IF (SPECIFY_QTR(JTT)) THEN ELTRT(JTT) = ETDSP(JS) ELTRB(JTT) = EBDSP(JS) END IF JBTR(JTT) = JBD end if ! cb 9/11/13 | 9/11/2013 |
| 38 | W2 | 32 bit exe on XP | Recompiled with new settings from Visual Studio 2012 to (hopefully) run on XP systems with 32 bit OS. | 9/11/2013 |
| 39 | W2 | End Simulation | Added new close open files in the end_simulation subroutine. This is merely cleaning up the code to be consistent in closing all open files when a ‘Stop’ is executed. This should have no effect on the end user. Part of this new code is shown below: IF(SELECTC == ' ON')then ! SW 9/25/13 New Section on closing files ifile=1949 do jb=1,nbr if(nstr(jb) > 0)then ifile=ifile+1 close(ifile) endif enddo if(nwd > 0)then ifile=ifile+1 close(ifile) endif endif IF (DOWNSTREAM_OUTFLOW) THEN JFILE=0 DO JWD=1,NIWDO CLOSE(WDO(JWD,1)) CLOSE(WDO(JWD,2)) IF (CONSTITUENTS) THEN CLOSE (WDO(JWD,3)) END IF IF (DERIVED_CALC) THEN CLOSE(WDO(JWD,4)) END IF | 9/25/13 |
| 40 | W2 | Pumps – Lateral | Fixed several sections of code in the PUMP algorithm in the hydroinout.f90 routine. Under some conditions, such as specifying “Lateral,” the PUMP algorithm may not have moved the water from the upstream to the downstream segment correctly. This has been fixed and tested. Part of the code changes are shown below: IF (LATERAL_PUMP(JP)) THEN ELW = EL(KTWB(JWU),IUPU(JP))-Z(IUPU(JP))*COSA(JBU) ! JWW = JWW+1 ! SW 9/25/13 ! JBWD(JWW) = JBU ! IWD(JWW) = IUPU(JP) ELSE ELW = EL(KTWB(JWU),IUPU(JP))-Z(IUPU(JP))*COSA(JBU)-SINA(JBU)*DLX(IUPU(JP))*0.5 ! JSS(JBU) = JSS(JBU)+1 ! SW 9/25/13 END IF … IF (PUMPON(JP)) THEN IF (LATERAL_PUMP(JP)) THEN JLAT = 1 JWW = JWW+1 ! SW 9/25/13 … CALL LATERAL_WITHDRAWAL ! (JWW) DO K=KTW(JWW),KBW(JWW) QSS(K,I) = QSS(K,I)-QSW(K,JWW) END DO IF (IDPU(JP) /= 0) THEN ! MOVED CODE SW 9/25/13 JTT = JTT+1 … ELSE JSS(JBU) = JSS(JBU)+1 ! SW 9/25/13 KTSW(JSS(JBU),JBU) = KTPU(JP) … | 9/25/13 |
| 41 | W2 | Clean up memory issues | Cleaned up a series of minor memory issues. This should have no impacts on current model runs. These were usually uninitialized memory. Code changes include the following: READ (CON,'(/)') KFNAME2=' ' ! SW 9/27/13 INITIALIZE ENTIRE ARRAY KFWBC =' ' ! SW 9/27/13 INITIALIZE ENTIRE ARRAY READ (CON,'(//(:8X,9I8))') (KBWD(JW), JW=1,NWD); TRC= ' ' ! SW 9/27/13 INITIALIZATION SINCE ALLOCATION IS TO NTRT READ (CON,'(//(:8X,9A8))') (TRC(JT), JT=1,NTR) EHSN(JE), EHSSI(JE), JE=1,NEPT) !JE=1,NEP) SW 9/27/13 READ (CON,'(//(8X,2F8.0,I8,F8.0))') (ESAT(JE), EHS(JE), ENEQN(JE), ENPR(JE), JE=1,NEPT) !JE=1,NEP) SW 9/27/13 READ (CON,'(//(8X,8F8.0))') (ET1(JE), ET2(JE), ET3(JE), ET4(JE), EK1(JE), EK2(JE), & EK3(JE), EK4(JE), JE=1,NEPT) !JE=1,NEP) SW 9/27/13 READ (CON,'(//(8X,6F8.0))') (EP(JE), EN(JE), EC(JE), ESI(JE), ECHLA(JE), EPOM(JE), JE=1,NEPT) !JE=1,NEP) SW 9/27/13 READ (CON,'(//8X,A8,I8,A8)') RSOC, NRSO, RSIC; RSOD=0.0 ! SW 9/27/13 INITIALIZE SINCE ALLOCATED AS NOD BUT ONLY NRSO USED READ (CON,'(//(:8X,9F8.0))') (RSOD(J), J=1,NRSO) READ (CON,'(//8X,I8,F8.0,a8)') NDLT, DLTMIN, DLTINTER; DLTD=0.0 ! SW 9/28/13 INITIALIZE ARRAY TO NOD SINCE ONLY NDLT ASSIGNED READ (CON,'(//(:8X,9F8.0))') (DLTD(J), J =1,NDLT) SINKC(1:NSTR(JB),JB) = SINKCT(1:NSTR(JB),JB) POINT_SINK(1:NSTR(JB),JB) = SINKC(1:NSTR(JB),JB) == ' POINT' ! SW 9/27/13 END DO ! POINT_SINK = SINKC == ' POINT' COLDEP=ELWS(I)-COLB ! MACT(J,KT,I)=MACT(J,KT+1,I) IF(MACROPHYTE_ON)MACT(J,KT,I)=MACT(J,KT+1,I) ! SW 9/28/13 ! SDKV(:,US(JB):DS(JB))=SDK(JW) SDKV(:,US(JB)-1:DS(JB)+1)=SDK(JW) ! SW 9/28/13 | 9/27/13 |
| 42 | W2 | CPL output | Code added to eliminate writing out the habitat index to the CPL file for Tecplot when HABITATC is OFF. IF(I /= DS(JB)+1)THEN IF(HABTATC == ' ON')THEN WRITE (CPL(JW),9999) X1(I),ELWS(I),U(K,I),-W(K,I),T1(K,I),RHO(K,I),HAB(K,I),(C2(K,I,CN(JC)),JC=1,NAC) ELSE WRITE (CPL(JW),9999) X1(I),ELWS(I),U(K,I),-W(K,I),T1(K,I),RHO(K,I),(C2(K,I,CN(JC)),JC=1,NAC) ENDIF ELSE XDUM=-99.0 WRITE (CPL(JW),9999) X1(I),ELWS(I),XDUM,XDUM,XDUM,XDUM,XDUM,(XDUM, JJ=1,NAC) ENDIF DO K=KTWB(JW),KMX-1 IF(I /= DS(JB)+1 .AND. K <= KB(I))THEN IF(HABTATC == ' ON')THEN WRITE (CPL(JW),9999) X1(I),ELWS(I)-DEPTHM(K,I),U(K,I),-W(K,I),T1(K,I),RHO(K,I),HAB(K,I),(C2(K,I,CN(JC)),JC=1,NAC) ELSE WRITE (CPL(JW),9999) X1(I),ELWS(I)-DEPTHM(K,I),U(K,I),-W(K,I),T1(K,I),RHO(K,I),(C2(K,I,CN(JC)),JC=1,NAC) ENDIF IF(K == KB(I))THEN IF(HABTATC == ' ON')THEN WRITE (CPL(JW),9999) X1(I),ELWS(I)-DEPTHB(K,I),U(K,I), -W(K,I),T1(K,I),RHO(K,I),HAB(K,I),(C2(K,I,CN(JC)),JC=1,NAC) ELSE WRITE (CPL(JW),9999) X1(I),ELWS(I)-DEPTHB(K,I),U(K,I), -W(K,I),T1(K,I),RHO(K,I),(C2(K,I,CN(JC)),JC=1,NAC) ENDIF WRITE (CPL(JW), *)'TITLE="CE-QUAL-W2"' IF(HABTATC == ' ON')THEN WRITE (CPL(JW),19233)(CNAME2(CN(JN)),JN=1,NAC) ELSE WRITE (CPL(JW),19234)(CNAME2(CN(JN)),JN=1,NAC) ENDIF ! sw 9/28/13 19233 FORMAT('VARIABLES="Distance, m","Elevation, m","U","W","T","RHO", "HABITAT" ',<NAC>(',"',A8,'"')) 19234 FORMAT('VARIABLES="Distance, m","Elevation, m","U","W","T","RHO" ',<NAC>(',"',A8,'"')) ! sw 9/28/13 | 9/28/13 |
| 43 | W2 | SPECIFY TRIB | In specifying the elevation between top and bottom for an inflow tributary, the code put the inflow one layer below where it should have been in many cases. This has been fixed by the additional code shown below: IF (SPECIFY_QTR(JT)) THEN KTTR(JT) = 2 ! DO WHILE (EL(KTTR(JT),I) > ELTRT(JT)) DO WHILE (EL(KTTR(JT),I) > ELTRT(JT) .and. EL(KTTR(JT)+1,I) > ELTRT(JT)) ! SW 10/3/13 KTTR(JT) = KTTR(JT)+1 END DO | 10/3/2013 |
| 44 | W2 | CWO or CWDO output | Fixed a format overflow in writing out concentrations in a withdrawal output file. IF (QWDO(J) /= 0.0) CWDO(CN(JC),J) = CWDO(CN(JC),J)/QWDO(J) WRITE (CWDOC(CN(JC)),'(F8.3)') CWDO(CN(JC),J) ! SW 9/23/13 Changed format from G8.3 to F8.3 to avoid format overflow CWDOC(CN(JC)) = ADJUSTR(CWDOC(CN(JC))) IF (QWDO(J) /= 0.0) CDWDO(CDN(JD,JW),J) = CDWDO(CDN(JD,JW),J)/QWDO(J) WRITE (CDWDOC(CDN(JD,JW)),'(F8.3)') CDWDO(CDN(JD,JW),J) ! SW 9/23/13 Changed format from G8.3 to F8.3 to avoid format overflow CDWDOC(CDN(JD,JW)) = ADJUSTR(CDWDOC(CDN(JD,JW))) | 10/4/2013 |
| 45 | W2 and PREW2 | Inflow, Tributary, Distributary and Shade inputs | Added csv file format as a new file input format for flow and temperature files for inflows, tributaries, and distributed tributaries. Also, the shade file is now in csv file format. This enhancement includes updates to the preprocessor and W2 codes. Also, several minor bug fixes were made on the Preprocessor. | 7/15/14 |
| 46 | W2 | Resuspension of inorganic solids | A resuspension formula was corrected. See the code change below: HS = 0.283 *U2/G*0.283*TANH(COEF1)*TANH(COEF2/TANH(COEF1)) !TS = 2.0*PI*U2/G*1.2* TANH(COEF3)*TANH(COEF4/TANH(COEF3)) TS = 2.0*PI*sqrt(U2)/G*1.2* TANH(COEF3)*TANH(COEF4/TANH(COEF3)) ! cb 5/9/14 | 7/15/14 |
| 47 | W2 | Tecplot output | When the user sets CPL output for Tecplot, the output format when HABITAC=OFF was incorrect. This has been fixed. | 7/15/14 |
| 48 | PREW2 | Warnings | Fixed a name inconsistency for developing warnings for input concentrations ! IF (NAME /= 'Residence time' .AND. NAME /= 'Water age') THEN IF (NAME /= 'Residence time' .AND. NAME /= 'AGE') THEN ! SW 7/15/14 CALL WARNINGS | 7/15/14 |
| 49 | W2 | TSR filename | The filename in w2_con.npt for TSR is used for the output filenames. In order to account for complex paths that include more than one ‘.’, the following change was made with the BACK=.TRUE. command, which checks from the right-hand-side rather than left-hand-side of the character string ! L1 = SCAN(TSRFN,'.') L1 = SCAN(TSRFN,'.',BACK=.TRUE.) | 8/22/14 |
| 50 | PREW2 | Hydraulic structure warnings | Added many new hydraulic structure warnings (gates, spillways, pumps, pipes, internal weirs) for cases where KBSTR was less than KB and fixed a few error messages for these structure checks. | 9/10/14 |
| 51 | W2 | TSR output | The time series file added the surface heat flux terms (net, short wave solar net, long wave radiation net, back radiation heat flux, evaporation heat flux, conductive heat flux) to the output. The manual was also updated. | 1/15/15 |
| 52 | W2 | Interpolation of wind direction | In some cases, the wind direction interpolation was incorrect. Code was added to reduce the wind direction angle to less than 2*pi before the interpolation is performed and to consider another possible interpolation case. Thanks to Wenwei Xu for pointing this out. New code is shown below: ! CONVERT PHIO AND PHINX TO LESS THAN 2*PI SW 2/13/15 DO WHILE(PHIO(JW)>2.*PI) PHIO(JW)=PHIO(JW)-2.*PI ENDDO DO WHILE(PHINX(JW)>2.*PI) PHINX(JW)=PHINX(JW)-2.*PI ENDDO IF (PHIO(JW)-PHINX(JW) > PI) THEN PHI(JW) = (1.0-RATIO)*(PHINX(JW)+2.0*PI)+RATIO*PHIO(JW) ELSEIF (PHIO(JW)-PHINX(JW) < -PI) THEN ! WX 2/13/15 PHI(JW) = (1.0-RATIO)*PHINX(JW)+RATIO*(PHIO(JW) +2.0*PI) ! WX 2/13/15 ELSE PHI(JW) = (1.0-RATIO)*PHINX(JW)+RATIO*PHIO(JW) END IF | 2/13/15 |
| 53 | W2 | Withdrawal | Stewart Rounds: Extra check to avoid divide by zero in withdrawal algorithm (this or similar code occurs in 4 subroutines in withdrawal.f90) IF ((ELSTR-HSWB) > EL(KBOT+1,ID)) THEN DLRHOB = ABS(RHO(KSTR,ID)-RHO(KBOT,ID)) ELSE IF ((EL(KBOT+1,ID)-ELR) == ELSTR) THEN !SR 03/24/13 DLRHOB = NONZERO !SR 03/24/13 ELSE DLRHOB = ABS(RHO(KSTR,ID)-RHO(KBOT,ID))*HSWB/(ELSTR-(EL(KBOT+1,ID)-ELR)) | 4/9/2015 |
| 54 | W2 | SELECTC | The USGS has developed a new automatic port selection algorithm. In the control file, w2_con.npt, one can use the new algorithm by setting SELECTC=’USGS’. The old algorithm is used when this is set to SELECTC= ‘ON.’ There is new documentation in the User Manual for this new algorithm. | 4/9/2015 |
| 55 | W2 | Restart output | Added code to write out a restart file (rso.opt) at the end of a run if restart_output is ON. | 4/9/15 |
| 56 | W2 Examples | Added example problems | Added new example problem for the Spokane River using new csv file inputs and four example problems for using the USGS auto-port algorithm | 4/9/15 |
| 57 | W2 | Restart for file volume_wbX.opt | The file handler was not closed properly for volume_wbX.opt. Fixed it with additional code in endsimualtion.f90: if(nwd > 0)then ifile=ifile+1 close(ifile) endif do jw=1,nwb ! sw 4/20/15 ifile=ifile+1 ! sw 4/20/15 close(ifile) ! sw 4/20/15 enddo ! sw 4/20/15 | 4/20/15 |
| 58 | W2 | W2selective.npt | Changed input format for critical temperatures for the output file volume_wbX.opt from a maximum of 10 waterbodies to 100. READ(1010,'(8X,100F8.0)')(TEMPCRIT(JW,J),JW=1,NWB) ! NOTE MAX OF 100 WATERBODIES sw 4/20/15 | 4/20/15 |
| 59 | W2 | Resuspension of SS | Changed DO loop index in suspended solids resuspension in water_quality.f90 from DO K=KT-1,KB(I)-1 to DO K=KT+1,KB(I)-1 ! cb 9/29/14 | 5/14/2015 |
| 60 | W2 | Wind at 2 m | The W2 model computes the wind at a 2 m height based on the used defined measurement height of the wind for evaporation computations. The formula for computing this variable was using a step function of the wind data rather than interpolation of the wind data when the user chose to interpolate meteorological data. For meteorological input data at short time intervals this is a very minor change. For meteorological data at large time intervals (e.g., a day), this could affect the amount of evaporation. Hence, the calculation below was moved from the TVDS routine to the main routine so that the interpolated wind would be used. WIND2(I)=WIND(JW)*WSC(I)*DLOG(2.0D0/Z0(JW))/DLOG(WINDH(JW)/Z0(JW)) | 5/21/15 |
| 61 | W2 | TSR output | The TSR file output now also includes a volume weighted vertical average temperature for the segment that the TSR file is located. The manual has been updated also. | 6/1/15 |
| 62 | W2 | Writing over output files | In some intermittent cases, when the dialog box closes, the model reinitializes some of the output files (effectively deleting the output). The following line of code was adding at the beginning of the main W2 code to prevent this: !** Task 1: Inputs ** !*********************************************************************************************************************************** INTEGER(4) length,istatus character*255 dirc ! call omp_set_num_threads(4) ! set # of processors to NPROC Moved to INPUT subroutine IF(END_RUN)STOP ! SW 6/26/15 Added code to prevent a thread from reinitializing output files as dialog box is closing...intermittant error | 6/26/15 |
| 63 | W2 | Output order for kinetic fluxes | The output columns for DOAR and DOER were switched in the output file kflux_jw*.opt. The model code was changed to fix this. ! DOAR => KF(:,:,56); DOEP => KF(:,:,57); DOER => KF(:,:,58); DOPOM => KF(:,:,59); DODOM => KF(:,:,60) DOEP => KF(:,:,56); DOAR => KF(:,:,57); DOER => KF(:,:,58); DOPOM => KF(:,:,59); DODOM => KF(:,:,60) ! cb 9/16/2015 | 9/16/15 |

## W2 V3.6 Bug Fixes, Enhancements, and User Manual Changes

Table 5. Bug fixes and enhancements for Version 3.6.

| # | Code: W2 or PREW2 or GUI | Fix or Enhancement Type | Description of Bug/Enhancement | Date Bug Fixed or Enhancement added |
| --- | --- | --- | --- | --- |
| 1 | W2 | TKE1 model | The variable STRICK was incorrectly allocated as an INTEGER rather than REAL. | 10/11/2008 |
| 2 | W2 | PIPE | Code was streamlined in the subroutine ZBRENT where calls were made directly to CDFUNC rather than through the dummy function FUNC | 10/11/2008 |
| 3 | W2 Manual | Z0 | The User Manual had Z0 in an incorrect line in the control file (w2_con.npt). The write up and example control file in the User Manual were corrected. | 10/28/2008 |
| 4 | W2 | Longitudinal profile input | The W2 program did not read initial constituent concentrations in the longitudinal profile file when CCC was 'OFF'. This has been fixed. | 12/4/2008 |
| 5 | W2 | TECPLOT output | When using TECPLOT output for multiple waterbodies, the output format did not allow loading the information into TECPLOT. Fixed. | 1/26/2009 |
| 6 | W2 | Epiphyton input | For entering vertical profile data for periphyton, there was an index error: Old code: IF (VERT_EPIPHYTON(JW,JE)) EPD(:,I,JE) = EPIVP(K,JW,JE) New code: IF (VERT_EPIPHYTON(JW,JE)) EPD(:,I,JE) = EPIVP(:,JW,JE) | 5/21/2009 |
| 7 | PreW2 | Constituent loads | An enhancement was added to the Preprocessor to compute loads in kg/day for all inflow, tributary and distributed tributaries. Also, these are summed up for the model application. These are shown in the file pre.opt. These are approximate loads since the concentration data are used to set the frequency of loading update. Flow rates at the time of the concentration input data are used to compute load. | 5/21/2009 |
| 8 | W2 | Gas transfer at spillways | A couple code fixes in the hydroinout.f90 subroutine: CGAS needed to be initialized in some cases to CGAS=C2(K,ID,CN(JC)) prior to calling the subroutine TOTAL_DISSOLVED_GAS for use in the Butts and Evans (1983) equation: CGAS=C2(K,ID,CN(JC)) ! MM 5/21/2009 Change logic in several lines from IF(CAC(NDO) == ‘ ON’ to IF(CAC(NDO) == ‘ ON’ .and. CN(JC)==NDO to IF (CN(JC)==NDO .AND. CAC(NDO) == ' ON' .AND. GASSPC(JS) == ' ON' .AND. QSP(JS) > 0.0) THEN ! MM 5/21/2009 | 5/21/2009 |
| 9 | W2 | Reaeration from dams | An error was found in the formulae from Butts and Evans (1983). Old code: DB = SAT-C DA = DB*(1.0+0.38*AGASGT(N)*BGASGT(N)*CGASGT(N)*(1.0-0.11*CGASGT(N))*(1.0+0.046*T)) C = SAT-DA New code: DA = SAT-C ! MM 5/21/2009 DA: Deficit upstream DB = DA/(1.0+0.38*AGASSP(N)*BGASSP(N)*CGASSP(N)*(1.0-0.11*CGASSP(N))*(1.0+0.046*T)) ! DB: deficit downstream C = SAT-DB | 5/21/2009 |
| 10 | W2 | Order of flux parameters | The order of flux parameters in the User Manual and output were incorrect. The control file has them in this order RPOMSET CBODDK DOAP DOAR DOEP DOER DOPOM DODOM DOOM whereas the code assumed this order: RPOMSET CBODDK DOAP DOEP DOAR DOER DOPOM DODOM DOOM This has been corrected. The User Manual and control file order is now reflected in the W2 code. | 6/2/2009 |
| 11 | Pre | False errors for inflow location | The preprocessor sometimes gave false errors in the pre.err for tributary, internal weirs, pipes, and other hydraulic features saying that the pipe or tributary was below the elevation of the bottom of the segment. The W2 model ran fine even with this error message given in the preprocessor. This has been fixed. Example of OLD code: IF (EBTR(JT) < EL(KB(ITR(JT)+1),ITR(JT))) THEN CALL ERRORS WRITE (ERR,FMTFI) 'Inflow placement bottom elevation [EBTR=',EBTR(JT),'] < bottom active cell elevation for tributary ',JT NEW CODE: IF (EBTR(JT) < EL(KB(ITR(JT))+1,ITR(JT))) THEN CALL ERRORS WRITE (ERR,FMTFI) 'Inflow placement bottom elevation [EBTR=',EBTR(JT),'] < bottom active cell elevation for tributary ',JT | 6/18/09 |
| 12 | Pre | Additional error checking | Additional error checking was added to help debug an error in the bathymetry file when the problem was in the branch connectivity specifically BS and BE. Also, a false error was given when the temperature had an isothermal initial condition, constituents were OFF, and an initial concentration was set to -2. This was fixed. | 6/22/09 |
| 13 | Pre | Command line processing and working directory displayed for windows | In the windows version of the preprocessor, the user can now supply a command line argument that sets the working directory of the code. Hence, one does not need to copy the preprocessor into every directory. In a batch file, for example, one can execute the following command: preW2_ivf.exe "C:\scott\w2workshop\2009 workshop\waterqual\problem3" The preprocessor now uses the supplied directory (in double quotes) as the working directory for all the files. The command line argument has one blank space between the end of the executable and the first quote. Also, the working directory is now displayed at the top of the window. Additional checks were also added for checking the grid linkage. | 9/12/09 |
| 14 | W2 | # of processors | The model user can now control the # of physical processors the model uses. At this point, dual-processor model runs have shown an improvement of about 20% over a single processor. But QUAD processors are usually slower. It is recommended that NPROC be set to 2 in the control file. The user can experiment on his/her own system. If this is not set by the user or is left blank, the model still runs, but sets it to 2 processors. GRID NWB NBR IMX KMX NPROC CLOSEC 1 1 23 22 2 ON | 9/12/09 |
| 15 | W2 | Command line processing for windows | In the windows version of the w2 model, the user can now supply a command line argument that sets the working directory of the code. Hence, one does not need to copy the model executable into every directory. In a batch file, for example, one can execute the following command: W2_ivf.exe "C:\scott\w2workshop\2009 workshop\waterqual\problem3" The w2 model now uses the supplied directory (in double quotes) as the working directory for all the files. The command line argument has one blank space between the end of the executable and the first quote. The working directory is displayed in a text box in the window. | 9/12/09 |
| 16 | W2 | W2 window closed at end of successful execution | At the end of a windows run, the windows dialog box waits for the user to press ‘close’ to exit the window. This allows the user to examine the final run parameters. In the w2_con.npt file there is now an option to close this window when the run has completed. If this option is not set, then the dialog box will stay until the user clicks close. This allows for efficient batch processing of the model, especially if used in conjunction with the command line processing mentioned in #15. GRID NWB NBR IMX KMX NPROC CLOSEC 1 1 23 22 0 ON When CLOSEC is set to ON, then the dialog box will disappear once the run finishes. If it is set to OFF, then the dialog box will remain until the user clicks ‘close.’ | 9/12/09 |
| 17 | User Manual | Updates | Updates and changes to the control file (#13-#16) were reflected in an updated User Manual. | 9/12/09 |
| 18 | GUI | Updates | The GUI was updated with the following: - New control file parameters NPROC and CLOSEC were added (see #14 and 16). There is also a SELECTC that will be used in V3.7 that has been included – ignore it for now. - The GUI can also be controlled by command line passing of the working directory and file. In a batch program or from the command line in a DOS box, you can execute the GUI as follows: "C:\scott\research\corps of engineers\tomcole\w2code\GUI36\w2control\w2control36.exe" C:\scott\w2workshop\2009 workshop\waterqual\problem1\w2_con.npt The first string in quotes executes the GUI. The command line argument is NOT in quotes. This program was developed in VB6 and does not take quotes around the command line. Note that this is different than the FORTRAN command line argument. So the above command will open the GUI and load the control file automatically. - A text box now shows the file path and name of the file that you are working on - Earlier in file open, all *.npt files were shown. Since only w2_con.npt files are loaded into the GUI, only the w2_con.npt file was shown for opening. | 9/12/09 |
| 19 | W2 | Gates, spillways, pipes | Whenever DOWN was specified for a gate, spillway or pump, the model estimated the water level at the end of the segment, rather than using the branch center water level. This is important in sloping river systems where a long segment may have a water surface elevation drop between the segment center and the edge. In the past this was computed assuming the slope of the channel. This was updated to estimate the water surface elevation using linear interpolation rather than the grid slope. Below is an example of the code fix – in this case for GATES: Old code: ELIU=ELWS(IUGT(JG))-SINA(JBUGT(JG))*DLX(IUGT(JG))*0.5 New code: ELIU= ELWS(IUGT(JG)) + (ELWS(IUGT(JG))-ELWS(IUGT(JG)-1))/(0.5*(DLX(IUGT(JG))+DLX(IUGT(JG)-1)))*DLX(IUGT(JG))*0.5 | 9/25/09 |
| 20 | W2 | New executable | A new executable was made using a new release of Intel Version 11 compiler that corrected problems with Windows 7 applications. | 9/25/09 |
| 21 | W2 | ICE cover algorithm | There were some logic errors in the ice cover algorithm. These were corrected below: !************** Ice thickness ICETH(I) = ICETH(I)+ICETHU+ICETH1+ICETH2 IF (ICETH(I) < ICE_TOL) ICETH(I) = 0.0 IF (WINTER .AND. (.NOT. ICE_IN(JB))) THEN IF (.NOT. ALLOW_ICE(I)) ICETH(I) = 0.0 END IF ICE(I) = ICETH(I) > 0.0 IF (ICE(I))THEN ! 3/27/08 SW ICESW(I) = 0.0 ELSE ICESW(I) = 1.0 ENDIF ICETHU = 0.0 ICETH1 = 0.0 ICETH2 = 0.0 IF (ICETH(I) < ICE_TOL .AND. ICETH(I) > 0.0) ICETH(I) = ICE_TOL ELSE IF(TERM_BY_TERM(JW))CALL EQUILIBRIUM_TEMPERATURE ! SW 10/20/09 Must call this first otherwise ET and CSHE are 0 HIA = 0.2367*CSHE(I)/5.65E-8 ! JM 11/08 convert SI units of m/s to English (btu/ft2/d/F) and then back to SI W/m2/C ! ICETH(I) = MAX(0.0,ICETH(I)+DLT*((RIMT-ET(I))/(ICETH(I)/RK1+1.0/HIA)-(T2(KT,I)-RIMT))/RHOIRL1) ! OLD CODE ICETH(I) = MAX(0.0,ICETH(I)+DLT*((RIMT-ET(I))/(ICETH(I)/RK1+1.0/HIA)-HWI(JW)*(T2(KT,I)-RIMT))/RHOIRL1) ! SW 10/20/09 Revised missing HWI(JW) ICE(I) = ICETH(I) > 0.0 ICESW(I) = 1.0 IF (ICE(I)) THEN ! TFLUX = 2.392E-7*(RIMT-T2(KT,I))*BI(KT,I)*DLX(I) ! OLD CODE TFLUX = 2.392E-7*HWI(JW)*(RIMT-T2(KT,I))*BI(KT,I)*DLX(I) ! SW 10/20/09 Revised missing HWI(JW) TSS(KT,I) = TSS(KT,I) +TFLUX TSSICE(JB) = TSSICE(JB)+TFLUX*DLT ICESW(I) = 0.0 END IF END IF END DO END IF END IF | 10/20/09 |
| 22 | W2 | Gates output in QWD file | The following bug was found in defining in which branch a gate was located. This affected the output for the withdrawals at a location where gates were not tied to other branches. Old code: JWUGT(JG) = JW IF (IDGT(JG) > 0) THEN DO JB=1,NBR IF (IDGT(JG) >= US(JB) .AND. IDGT(JG) <= DS(JB)) EXIT END DO JBDGT(JG) = JB DO JW=1,NWB IF (JB >= BS(JW) .AND. JB <= BE(JW)) EXIT END DO JWDGT(JG) = JW else ! BUG FIX 9/27/07 jbdgt(jp)=1 jwdgt(jp)=1 END IF New code: JWUGT(JG) = JW IF (IDGT(JG) > 0) THEN DO JB=1,NBR IF (IDGT(JG) >= US(JB) .AND. IDGT(JG) <= DS(JB)) EXIT END DO JBDGT(JG) = JB DO JW=1,NWB IF (JB >= BS(JW) .AND. JB <= BE(JW)) EXIT END DO JWDGT(JG) = JW else ! BUG FIX 9/27/07 jbdgt(jg)=1 ! SW 3/24/10 jwdgt(jg)=1 ! SW 3/24/10 END IF | 3/24/10 |
| 23 | PreW2 | Reading of WSC | Reading in of the WSC file was limited to only 100 dates in the preprocessor. This limitation was fixed by the code shown below: ! DO J=1,100 28995 continue ! cb 3/26/10 READ (NPT,'(10F8.0:/(8X,9F8.0))',END=29000) SDAY,(WSC(I),I=1,IMX) IF (SDAY <= SDAYO) THEN CALL ERRORS WRITE (ERR,'(3(A,F0.3))') 'Julian date ',SDAY,' <= previous date of ',SDAYO,' in '//WSCFN END IF DO I=1,IMX IF(WSC(I) <= 0.0)THEN CALL ERRORS WRITE (ERR,'(A,F0.3,A,I4,A)') 'Julian date ',SDAY,': WSC AT SEG(I)=',I,' <= 0.0 in '//WSCFN ENDIF IF (WSC(I) > 2.0) THEN CALL WARNINGS WRITE (WRN,'(A,F0.3,A,I4,A)') 'Julian day ',SDAY,': WSC(I) AT SEG(I)=',I,' > 2.0 in '//WSCFN END IF IF (WSC(I) > 0.0 .and. wsc(i) < 0.5) THEN CALL WARNINGS WRITE (WRN,'(A,F0.3,A,I4,A)') 'Julian day ',SDAY,': WSC(I) AT SEG(I)=',I,' < 0.5 in '//WSCFN END IF ENDDO SDAYO=SDAY ! ENDDO go to 28995 ! cb 3/26/10 | 3/26/10 |
| 24 | PreW2 | Check on LAT or DOWN | Added an enhancement to do a check in case a spillway, pipe, pump, or gate was specified as ‘DOWN.’ In all cases where ‘DOWN’ is specified, the segment where the hydraulic structure originates must be at the end of a branch. Additional logic was added to check for this in all the hydraulic structures. | 3/26/10 |
| 25 | W2 Manual | Light extinction, ice | Added more text to the section on computation of light extinction and inserted a missing reference. Revised an equation for clarity in ICE algorithm and added more explanation on how to estimate HICE. | 4/13/2010 |
| 26 | W2 Manual | Precipitation input file | The units of precipitation are in m/s. The example precipitation input file was changed to more realistic values. | 4/14/2010 |
| 27 | W2 | ICE | Added code to account for the need to compute long wave radiation in case user chose the equilibrium temperature approach. Fixed subscript error in ice melt computation. Also, made the variable TICE double precision since it is assumed double precision in the call to Surface_terms. New code: IF (ICE(I)) THEN TICE = TAIR(JW) DEL = 2.0 J = 1 if(tair(jw).ge.5.0)then ! SW 4/19/10 RANLW(JW) = 5.31E-13*(273.15+TAIR(JW))**6*(1.0+0.0017*CLOUD(JW)**2)*0.97 else RANLW(JW) = 5.62E-8*(273.15+TAIR(JW))**4*(1.-0.261*exp(-7.77E-4*TAIR(JW)**2))*(1.0+0.0017*CLOUD(JW)**2)*0.97 endif RN1=SRON(JW)/(REFL*RHOWCP)*SHADE(I)*(1.0-ALBEDO(JW))*BETAI(JW)+RANLW(JW) ! SW 4/19/10 DO WHILE (DEL > 1.0 .AND. J < 500) CALL SURFACE_TERMS (TICE) RN(I) = RN1-RB(I)-RE(I)-RC(I) ! 4/19/10 ! RN(I) = SRON(JW)/(REFL*RHOWCP)*SHADE(I)*(1.0-ALBEDO(JW))*BETAI(JW)+RANLW(JW)-RB(I)-RE(JW)-RC(I) ! OLD CODE DEL = RN(I)+RK1*(RIMT-TICE)/ICETH(I) IF (ABS(DEL) > 1.0) TICE = TICE+DEL/500.0 J = J+1 END DO | 4/19/10 |
| 28 | W2 | Evaporation | Units for EV in the SNP file were given in m/s but were actually m^3/s | 4/21/10 |
| 29 | W2 | Ice | In the ice melt algorithm, SRON should not have been divided by RHOCP in computing RN1, and DEL in the DO WHILE loop should have been ABS(DEL) rather than DEL: RN1=SRON(JW)/REFL*SHADE(I)*(1.0-ALBEDO(JW))*BETAI(JW)+RANLW(JW) ! SW 4/19/10 eliminate spurious divsion of SRO by RHOCP DO WHILE (ABS(DEL) > 1.0 .AND. J < 500) ! SW 4/21/10 Should have been ABS of DEL CALL SURFACE_TERMS (TICE) | 4/21/2010 |
| 30 | PRE | Constituent Loading | The output from the preprocessor in the pre.opt file for constituent loading was in kg rather than the output header of kg/day. The output was updated to kg/day by adding the following lines of code: cdtload(incdt(1:NACdt(Jb),Jb),jb)=cdtload(incdt(1:NACdt(Jb),Jb),jb)/(jday-tstart) ! CB 5/10/10 Change units to kg/day ctrload(trcn(1:NACtr(Jt),Jt),jt)=ctrload(trcn(1:NACtr(Jt),Jt),jt)/(JDAY-TSTART) !CB 5/11/10 convert to units of kg/day | 5/10/10 |
| 31 | W2 | Gate, spillways, pipes | When the user specified that the flow is DOWN, in the case of reverse flow, the model did not assign the flow correctly if the user had no other tributaries or withdrawals specified in the control file. For this rare event, additional code was written to account for this fact. Also, a logic error was discovered in reverse flow for spillways and gates. This was corrected. New code added to hydroinout.f90: JWW = NWD withdrawals = jww > 0 ! 6/4/10 SW JTT = NTR tributaries = jtt > 0 ! 6/4/10 SW JSS = NSTR IF (SPILLWAY) THEN … END IF tributaries = jtt > 0 ! 6/4/10 SW withdrawals = jww > 0 ! 6/4/10 SW DO JW=1,NWB KT = KTWB(JW) DO JB=BS(JW),BE(JW) New code in gate-spill-pipe.f90: For spillway: IF (ISUB == 0) THEN DLEL = ELIU-ESP(JS) IF (ELID > ESP(JS)) DLEL = ELIU-ELID ! SW 6/7/10 IF (DLEL < 0.0) THEN DLEL = -DLEL For gates: IF (A2GT(JG) == 0.0 .AND. G2GT(JG) /= 0.0) DLEL = ELIU-G2GT(JG) IF (ELID > EGT(JG)) DLEL = ELIU-ELID ! SW 6/7/10 IF (DLEL < 0.0) THEN | 6/4/10 |
| 32 | W2 | Branch intersections with multiple waterbodies | In cases where there are branch intersections between waterbodies, it was possible that the variables KBI and KB were incorrectly set. Here is the fix: Move the statement defining KBI in the subroutine init-geom.f90 to the place shown below (delete the earlier reference): IF (B(K,ID+1) == 0.0) B(K,ID+1) = B(K-1,ID+1) IF (IEXIT == 1) EXIT END IF END IF END IF END DO END DO ! SW 1/23/06 END DO ! SW 1/23/06 bnew=b ! SW 1/23/06 KBI = KB ! SW 10/30/2010 !**** Upstream active segment and single layer ! 1/23/06 entire section moved SW DO JW=1,NWB KT = KTWB(JW) DO JB=BS(JW),BE(JW) | 10/30/2010 |
| 33 | W2 | SS Resuspension | The code index was incorrect in the loop for computing resuspension. This led in some compilers to an infinite loop. The corrected code is shown below: SSSS(KT,I,J) = -SSS(J)*SS(KT,I,J)*BI(KT,I)/BH2(KT,I)+SSR ! DO K=KT-1,KB(I)-1 DO K=KT,KB(I)-1 ! JP 2/3/12 IF (SEDIMENT_RESUSPENSION(J)) THEN Thanks to James Pasley for this bug report/fix. | 2/3/2012 |

## W2 V3.5 Bug Fixes, Enhancements, and User Manual Changes

Table 6. Bug fixes and enhancements for Version 3.5.

| # | Code: W2 or PREW2 or GUI | Fix or Enhancement Type | Description of Bug/Enhancement | Date Bug Fixed or Enhancement Added |
| --- | --- | --- | --- | --- |
| 1 | W2 | Zooplankton-algae | Sign error in the zooplankton grazing on algae term. | 8/23/06 |
| 2 | W2 | Input/output | Format for I/O was changed to allow better decimal precision of output. | 8/23/06 |
| 3 | W2 | Sediment settling rate | The sediment settling rate was accidentally used for POM settling. This was fixed. The old and new code lines are shown below: Old Code: sedsum = sedsum+seds(JW)*(LPOM(K,I)*lpomdk(jw)+RPOM(K,I)*rpomdk(jw))*BI(K,I)/BH2(K,I)*(1.0-BI(K+1,I)/BI(K,I)) New Code: sedsum = sedsum+poms(JW)*(LPOM(K,I)*lpomdk(jw)+RPOM(K,I)*rpomdk(jw))*BI(K,I)/BH2(K,I)*(1.0-BI(K+1,I)/BI(K,I)) ! cb 10/22/06 This was an issue in the SEDIMENT, SEDIMENT C, SEDIMENT P, SEDIMENT N, and SEDIMENT DECAY RATE subroutines. | 10/26/06 |
| 4 | W2 | Sediment burial | An algorithm was added for sediment burial. This is now a new parameter in the sediment part of the control file. An updated user manual description is forthcoming. The sediment burial rate SEDB (day-1) can be specified in the “SEDIMENT” card section of the control file. A different burial rate can be specified for each water body. OLD/NEW line (example): ! SED(K,I) = MAX(SED(K,I)+(LPOMEP(K,I)+SEDAS(K,I)+SEDOMS(K,I)+SEDNS(K,I)-SEDD(K,I))*DLT,0.0) SED(K,I) = MAX(SED(K,I)+(sedem+SEDAS(K,I)+sedcb(k,i)+SEDOMS(K,I)+SEDNS(K,I)-SEDD(K,I)-sedbr(k,i))*DLT,0.0) ! cb 11/30/06 | 11/30/06 |
| 5 | Control File | Add burial rate for sediment model | This is the change in #4 above implemented in the control file. The new variable SEDBR is added in f8 format after the FSED variable. SEDBR: sediment burial rate in units of per day. SEDIMENT SEDC SEDPRC SEDCI SEDK SEDS FSOD FSED SEDBR WB 1 ON ON 0.00000 0.10000 0.1 1.00000 1.00000 1.0 |  |
| 6 | W2 | Sediment heating and sediment processes | If a model added and subtracted layers that resulted in segment addition and subtraction, sediment fluxes could be incorrectly computed. In the NO3 subroutine: Old code: NO3SED(K,I) = NO3(K,I)*NO3S(JW)*NO3TRM(K,I)*(BI(K,I)-BI(K+1,I))/BH2(K,I) New code: if(k == kb(i)) then NO3SED(K,I) = NO3(K,I)*NO3S(JW)*NO3TRM(K,I)*(BI(K,I))/BH2(K,I) else NO3SED(K,I) = NO3(K,I)*NO3S(JW)*NO3TRM(K,I)*(BI(K,I)-BI(K+1,I))/BH2(K,I) endif New code added in sediment routine: if(k == kb(i))then ! SW 4/18/07 SODD(K,I) = SOD(I)/BH2(K,I)*SODTRM(K,I)*BI(K,I) else SODD(K,I) = SOD(I)/BH2(K,I)*SODTRM(K,I)*(BI(K,I)-BI(K+1,I)) Endif New code added in suspended solids routine: if(k == kb(i))then SSR = EPSILON*DLX(I)*BI(K,I)/VOL(K,I) else SSR = EPSILON*DLX(I)*(BI(K,I)-BI(K+1,I))/VOL(K,I) Endif | 4/18/07 |
| 6 | W2 | (See above) | New code added for heat flux to channel bottom: if(kt == kb(i))then ! SW 4/18/07 SROSED = SROOUT*TSEDF(JW) else SROSED = SROOUT*(1.0-BI(KT+1,I)/BI(KT,I))*TSEDF(JW) Endif if(k==kb(i))then ! SW 4/18/07 TFLUX = CBHE(JW)/RHOWCP*(TSED(JW)-T2(K,I))*BI(K,I)*DLX(I) else TFLUX = CBHE(JW)/RHOWCP*(TSED(JW)-T2(K,I))*(BI(K,I)-BI(K+1,I))*DLX(I) endif New code added for sediment subroutine: if(k == kb(i))then ! SW 4/18/07 SEDAS(K,I) = SEDAS(K,I)+MAX(AS(JA),0.0)*ALG(K,I,JA)*BI(K,I)/BH2(K,I)*(1.0-BI(K+1,I)/BI(K,I)) else SEDAS(K,I) = SEDAS(K,I)+MAX(AS(JA),0.0)*ALG(K,I,JA)*BI(K,I)/BH2(K,I)*(1.0-BI(K+1,I)/BI(K,I)) endif if(k == kb(i))then ! SW 4/18/07 SEDOMS(K,I) = POMS(JW)*(LPOM(K,I)+RPOM(K,I))*BI(K,I)/BH2(K,I) SEDSO = POMS(JW)*SED(K,I)*BI(K+1,I)/BH2(K,I) else SEDOMS(K,I) = POMS(JW)*(LPOM(K,I)+RPOM(K,I))*BI(K,I)/BH2(K,I)*(1.0-BI(K+1,I)/BI(K,I)) SEDSO = POMS(JW)*SED(K,I)*BI(K+1,I)/BH2(K,I)*(1.0-BI(K+1,I)/BI(K,I)) endif | 4/18/07 |
| 7 | W2 | Zooplankton fixes | Several fixes in the zooplankton routine were made. Many thanks to Dr. Kellie Vache, Institute for Landscape Ecology and Resources Management (ILR) Justus-Liebig-University Giessen Heinrich-Buff-Ring 26 35392 Giessen, Germany, for finding these which are documented below: DO K=KT,KB(I) do jz = 1, nzp zgztot=0.0 !kv 5/9/2007 do jjz = 1,nzp ! zooss(k,i,jz)= (zmu(k,i,jz)*zeff(jz)-zrt(k,i,jz)-zmt(k,i,jz))*zoo(k,i,jz) - zgz(k,i,jz,jjz)*zoo(k,i,jz) ! omnivorous zooplankton zgztot=zgztot+zgz(k,i,jz,jjz)*zoo(k,i,jz) !kv 5/9/2007 end do zooss(k,i,jz)= (zmu(k,i,jz)*zeff(jz)-zrt(k,i,jz)-zmt(k,i,jz))*zoo(k,i,jz) - zgztot ! kv 5/9/2007 end do do jjz = 1, nzp ! tgraze(k,i,jz) = tgraze(k,i,jz) + prefz(jz,jjz)*zoo(k,i,jjz) tgraze(k,i,jz) = tgraze(k,i,jz) + prefz(jjz,jz)*zoo(k,i,jjz) !cb 5/17/2007 end do do jjz = 1,nzp ! omnivorous zooplankton ! ZGZ(k,i,jjz,jz) = Zmu(K,I,jz)*ZOO(K,I,jz)*prefZ(jz,jjz)/tgraze(K,I,jz) ZGZ(k,i,jjz,jz) = Zmu(K,I,jz)*ZOO(K,I,jz)*prefZ(jjz,jz)/tgraze(K,I,jz) !kv 5/9/2007 end do | 5/21/07 |
| 8 | PRE | More checks | Added checks for Sediment burial rate and some further checks on grid geometry; added output on SEDS and SEDBR to the pre.opt file; fixed condition where NZP had to equal one to work. | 6/2/2007 |
| 9 | W2 | Array deallocation | The deallocate command on line 7557 was commented out to avoid a deallocation error when the ‘STOP’ button is pushed during execution on a PC: ! deallocate (sedbr,sedbrp,sedbrn,sedbrc) ! SW 6/4/07 No need to deallocate pointers | 6/4/2007 |
| 10 | W2 | Initialization of IUT | For code setting up an external head BC, the variable IUT was not initialized before it was used. This was fixed below: !**** Boundary bottom layers ! IF (UH_EXTERNAL(JB)) KB(IUT-1) = KB(IUT) IF (UH_EXTERNAL(JB)) KB(IU-1) = KB(IU) !cb 6/12/07 IF (UH_INTERNAL(JB)) THEN IF (JBUH(JB) >= BS(JW) .AND. JBUH(JB) <= BE(JW)) THEN ! KB(IUT-1) = MIN(KB(UHS(JB)),KB(IUT)) KB(IU-1) = MIN(KB(UHS(JB)),KB(IU)) !cb 6/12/07 ELSE ! IF (EL(KB(IUT),IUT) >= EL(KB(UHS(JB)),UHS(JB))) THEN IF (EL(KB(IU),IU) >= EL(KB(UHS(JB)),UHS(JB))) THEN !cb 6/12/07 ! KB(IUT-1) = KB(IUT) KB(IU-1) = KB(IU) ELSE ! DO K=KT,KB(IUT) ! IF (EL(KB(UHS(JB)),UHS(JB)) >= EL(K,IUT)) THEN ! KB(IUT-1) = K; EXIT DO K=KT,KB(IU) !cb 6/12/07 IF (EL(KB(UHS(JB)),UHS(JB)) >= EL(K,IU)) THEN !cb 6/12/07 KB(IU-1) = K; EXIT !cb 6/12/07 END IF | 6/17/2007 |
| 11 | W2 | CBOD settling | The CBOD settling rate earlier was not converted from m/d in the control file to m/s in the code. Added code: cbods = cbods/day !cb 7/23/07 | 7/23/07 |
| 12 | W2 | TSR output | The surface width was not correctly being output. Changed BI(KT) to BI(KTWB(JW)). FIX: BI(KTWB(JW),I),SHADE(I),ICETH(I),(ADJUSTR(C2CH(JAC)),JAC=1,NAC), & ! CB 7/26/07 | 7/26/07 |
| 13 | PREW2 | Pumps | The pump control for DOWN or LAT was not being checked properly, also a check on IUPUC was incorrect. Fixed. | 8/14/07 |
| 14 | W2 | Algae | The logic for negative settling velocities for algae had an error. Old code: ! ASR(K,I,JA) = -AS(JA)*(ALG(K+1,I,JA)*B(K+1,I)/(B(K,I)*H2(K,I))-ALG(K,I,JA))*BI(K,I)/BH2(K,I) New code: ASR(K,I,JA) = -AS(JA)*(ALG(K+1,I,JA)*BI(K+1,I)/BH2(K,I)-ALG(K,I,JA)*BI(K,I)/BH2(K,I)) !SP 8/27/07 Shwet Prakash | 8/27/07 |
| 15 | GUI | NZOOP | When # of zooplankton was set equal to zero, there was an array dimensioning error that caused the writing of the control file to only proceed part way. Fixed. | 9/17/07 |
| 16 | W2 | Open channel flow | Variable passed between subroutines had inconsistent declaration between routines. ! REAL, ALLOCATABLE, DIMENSION(:) :: Y, D, B, V, CAREA, TOPW, BELEV, Q, VOLD, YOLD REAL, ALLOCATABLE, DIMENSION(:) :: Y, B, V, CAREA, TOPW, BELEV, Q, VOLD, YOLD ! cb 10/1/07 ! ALLOCATE (Y(NN), V(NN), CAREA(NN), TOPW(NN), BELEV(NN), Q(NN), VOLD(NN), YOLD(NN), D(NN), B(NN)) ALLOCATE (Y(NN), V(NN), CAREA(NN), TOPW(NN), BELEV(NN), Q(NN), VOLD(NN), YOLD(NN), B(NN)) ! cb 10/1/07 ! DEALLOCATE (Y, V, CAREA, TOPW, BELEV, Q, VOLD, YOLD, D, B, YT, VT, VPR, YPR, TAREA, TOPWT, RT, INDX, AL, DAA) DEALLOCATE (Y, V, CAREA, TOPW, BELEV, Q, VOLD, YOLD, B, YT, VT, VPR, YPR, TAREA, TOPWT, RT, INDX, AL, DAA) ! cb 10/1/07 | 10/4/07 |
| 17 | W2 | TKE model | The TKE algorithm had several bugs that have been fixed, these included making the loop over layers go to KBMIN (rather than KB), the original code overwrote the boundary conditions when using the Thomas algorithm, the original code overwrote vertical eddy viscosity at the bed during the averaging process, changed to , TKE array was initialized to zero, TKE was implemented in add/sub layers like AZ. Many of these fixes are a result of the work of Sam Gould (Gould, 2006) who wrote an MS project report at PSU entitled “k-e Turbulence Model.” Further recommendations by Gould (2006) will be incorporated into the next version of CE-QUAL-W2. The old code is shown below as a reference to the new code in the release version. Old Code: ENTRY CALCULATE_TKE USTAR = SQRT(1.25*CZ(I)*WIND10(I)**2/RHO(KT,I)) IF (MANNINGS_N(JW)) THEN HRAD = BHR1(KT,I)/(BR(KTI(I),I)-BR(KT+1,I)+2.*AVH1(KT,I)) if(macrophyte_on.and.mannings_n(jw))then call macrophyte_friction(hrad,fric(i),effric,kt,i) gc2=g*effric*effric/hrad**0.33333333 else if(.not.macrophyte_on.and.mannings_n(jw))then gc2=g*fric(i)*fric(i)/hrad**0.33333333 end if ELSE GC2 = 0.0 IF (FRIC(I) /= 0.0) GC2 = G/(FRIC(I)*FRIC(I)) END IF USTARB = SQRT(GC2)*ABS(0.5*(U(KT,I)+U(KT,I-1))) TKE(KT,I,1) = 0.5*(3.33*(USTAR*USTAR+USTARB*USTARB)+TKE(KT,I,1))*(BH2(KT,I)/BH1(KT,I)) TKE(KT,I,2) = 0.5*(USTAR*USTAR*USTAR+USTARB*USTARB*USTARB*5.0/H1(KT,I)+TKE(KT,I,2))*(BH2(KT,I)/BH1(KT,I)) DO K=KT+1,KB(I)-1 BOUK = MAX(AZ(K,I)*G*(RHO(K+1,I)-RHO(K,I))/(H(K,JW)*RHOW),0.0) PRDK = AZ(K,I)*(0.5*(U(K,I)+U(K,I-1)-U(K+1,I)-U(K+1,I-1))/H(K,JW))**2.0 PRHE = 10.0*GC2**1.25*ABS(0.5*(U(K,I)+U(K,I-1)))**4.0/(0.5*B(K,I))**2.0 IF (MANNINGS_N(JW)) THEN ! v3.5 start HRAD = BHR(K,I)/(BR(K,I)-BR(K+1,I)+2.0*H(K,JW)) ! GC2 = G*FRIC(I)*FRIC(I)/HRAD**0.333 if(macrophyte_on.and.mannings_n(jw))then call macrophyte_friction(hrad,fric(i),effric,k,i) gc2=g*effric*effric/hrad**0.33333333 else if (.not.macrophyte_on.and.mannings_n(jw))then gc2=g*fric(i)*fric(i)/hrad**0.33333333 end if ! v3.5 end END IF PRHK = GC2/(0.5*B(K,I))*ABS(0.5*(U(K,I)+U(K,I-1)))**3.0 UNST = PRDK-TKE(K,I,2) UNSE = 1.44*TKE(K,I,2)/TKE(K,I,1)*PRDK-1.92*(TKE(K,I,2)/TKE(K,I,1)*TKE(K,I,2)) TKE(K,I,1) = TKE(K,I,1)+DLT*(UNST+PRHK-BOUK) TKE(K,I,2) = TKE(K,I,2)+DLT*(UNSE+PRHE) END DO USTARB = SQRT(GC2)*ABS(0.5*(U(KB(I),I)+U(KB(I),I-1))) TKE(KB(I),I,1) = 0.5*(3.33*USTARB*USTARB+TKE(KB(I),I,1)) TKE(KB(I),I,2) = 0.5*(USTARB*USTARB*USTARB*5.0/H(KB(I),JW)+TKE(KB(I),I,2)) AT = 0.0; CT = 0.0; VT = 0.0; DT = 0.0 DO J=1,2 DO K=KT,KB(I) AT(K,I) = -DLT/BH1(K,I)*BB(K-1,I)/SIG(J)*AZ(K-1,I)/AVH1(K-1,I) CT(K,I) = -DLT/BH1(K,I)*BB(K,I)/SIG(J)*AZ(K,I)/AVH1(K,I) VT(K,I) = 1.0-AT(K,I)-CT(K,I) DT(K,I) = TKE(K,I,J) END DO CALL TRIDIAG(AT(:,I),VT(:,I),CT(:,I),DT(:,I),KT,KB(I),KMX,TKE(:,I,J)) END DO DO K=KT,KB(I) TKE(K,I,1) = MAX(TKE(K,I,1),TKEMIN1) TKE(K,I,2) = MAX(TKE(K,I,2),TKEMIN2) AZ(K,I) = 0.09*TKE(K,I,1)*TKE(K,I,1)/TKE(K,I,2) END DO ! Center at cell faces DO K=KT,KB(I)-1 AZ(K,I) = 0.5*(AZ(K,I)+AZ(K+1,I)) AZ(K,I) = MAX(AZMIN,AZ(K,I)) AZ(K,I) = MIN(AZMAX(JW),AZ(K,I)) DZ(K,I) = MAX(DZMIN,FRAZDZ*AZ(K,I)) END DO | 10/4/07 |
| 18 | W2 | Restart | Added TKE to restart variables written out and read in. | 10/5/07 |
| 19 | GUI | ET | The equilibrium temperature option in the drop down menu was ‘EQT’ rather than ‘ET’. Fixed. | 10/9/07 |
| 20 | W2 | Sediment | The SEDIMENT subroutine did not have any computational mistakes, just an error in assigning all array variables to the value at K,I. This resulted in excessive computational time. The fix is shown below: Old Code: sedbr = sedb(jw)*sed(k,i) New Code: sedbr(K,I) = sedb(jw)*sed(k,i) | 10/15/07 |
| 21 | W2 | TKE | Turbulence model had an improper averaging between layers. A new temporary variable was defined to temporarily store the values for AZ prior to averaging to the bottom/top of the layers and the horizontal layers. This also affected the computation of DZ. Fixed. New code defined AZT and allocated memory for it, such that AZT(K,I) = 0.09*TKE(K,I,1)*TKE(K,I,1)/TKE(K,I,2) and AZ(K,I) = 0.5*(AZT(K,I)+AZT(K+1,I)). Similarly for the horizontal averaging and for DZ. Also, the values of DZ were fixed to be at the bottom of a cell and AZ was fixed to be at the bottom right-hand edge of a cell as shown below: | 12/17/07 |
| 22 | W2 | SS settling | The incorrect cell width was used for SSSO. BI(KT,I) was changed to BI(K,I). Old Code: SSSO(K,I) = (TOTSS0+FES(JW)*FPFE(K,I))*BI(KT,I)/BH2(K,I)*DO1(K,I) FPSS(K,I) = FPSS(K,I)*TISS(K,I) New Code: SSSO(K,I) = (TOTSS0+FES(JW)*FPFE(K,I))*BI(K,I)/BH2(K,I)*DO1(K,I) FPSS(K,I) = FPSS(K,I)*TISS(K,I) | 12/17/07 |
| 23 | W2 | Initialization of one-layer | The definition of KBMIN was not updated if the model started out in some segments with only one_layer. This has been fixed. Added highlighted code: DO I=IU,ID IF (KB(I)-KT < NL(JB)-1) IUT = I+1 ONE_LAYER(I) = KT == KB(I) END DO CUS(JB) = IUT ! reinitialize KBMIN DO I=IU-1,ID KBMIN(I) = MIN(KB(I),KB(I+1)) END DO KBMIN(ID+1) = KBMIN(ID) !**** Areas and bottom widths IF (.NOT. TRAPEZOIDAL(JW)) THEN | 12/17/07 |
| 24 | W2 | Bottom Processes | This is a couple more fixes related to bug fix #6 above. The Denitrification rate and epiphyton burial rates could be affected based on unique combinations of adding/subtracting segments that left the value of BI in an inactive layer below KB defined incorrectly. In order to prevent the possibility of problems, the following fixes were made: Old Code: sedNO3(K,I) = NO3(K,I)*NO3S(JW)*NO3TRM(K,I)*(BI(K,I)-BI(K+1,I))/BH2(K,I) EPM(K,I,J) = EPD(K,I,J)*(BI(K,I)-BI(K+1,I)+2.0*H1(K,I))*DLX(I) New code: if(k == kb(i)) then ! SW 12/16/07 sedNO3(K,I) = NO3(K,I)*NO3S(JW)*NO3TRM(K,I)*(BI(K,I))/BH2(K,I) else sedNO3(K,I) = NO3(K,I)*NO3S(JW)*NO3TRM(K,I)*(BI(K,I)-BI(K+1,I))/BH2(K,I) endif if(k == kb(i)) then ! SW 12/16/07 EPM(K,I,J) = EPD(K,I,J)*(BI(K,I)+2.0*H1(K,I))*DLX(I) else EPM(K,I,J) = EPD(K,I,J)*(BI(K,I)-BI(K+1,I)+2.0*H1(K,I))*DLX(I) endif | 12/17/2007 |
| 25 | W2 | CBODS | If the user defined particulate CBOD that settles to the bottom and had SED turned ON, the conversion from oxygen to organic matter was missing in the accumulation on the channel bottom or sides. Old Code: do jd=1,nbod SEDcb(K,I) = SEDcb(K,I)+MAX(cbods(jd),0.0)*cbod(K,I,Jd)*BI(K,I)/BH2(K,I)*(1.0-BI(K+1,I)/BI(K,I)) end do New Code: do jd=1,nbod SEDcb(K,I) = SEDcb(K,I)+MAX(cbods(jd),0.0)*(cbod(K,I,Jd)/O2OM(JW))*BI(K,I)/BH2(K,I)*(1.0-BI(K+1,I)/BI(K,I)) ! 1/16/08 end do | 1/18/08 |
| 26 | W2 | SEDBR | Eliminated a redundant definition of SEDBR in the Sediment routine since it is already defined in the Kinetic rates subroutine. | 1/18/08 |
| 27 | W2 | SEDDK | The first order sediment decay rate is an average of the decay rates of all the influxes of organic matter and their respective decay rates. There was an error in computing this average decay rate for CBOD treated as particulate. Code fix is shown below: Old Code: do jd=1,nbod sedsum = sedsum+MAX(cbods(jd),0.0)*cbod(K,I,Jd)*BI(K,I)/BH2(K,I)*(1.0-BI(K+1,I)/BI(K,I)) end do New Code: do jd=1,nbod sedsum = sedsum+MAX(cbods(jd),0.0)*cbod(K,I,Jd)*BI(K,I)/BH2(K,I)*(1.0-BI(K+1,I)/BI(K,I))*RBOD(JD)*CBODD(K,I,JD)/O2OM(JW) end do | 1/18/08 |
| 28 | W2 | SRO | There are some cases when segments were added/subtracted that the value of BI was not correctly initialized. This code is a fix to prevent such occurrences: Old Code: SRONET = SROIN-SROOUT SROSED = SROOUT*(1.0-BI(K+1,I)/BI(K,I))*TSEDF(JW) New Code: SRONET = SROIN-SROOUT if(k /= kb(i))then ! SW 1/18/08 SROSED = SROOUT*(1.0-BI(K+1,I)/BI(K,I))*TSEDF(JW) else SROSED = SROOUT*TSEDF(JW) endif | 1/18/2008 |
| 29 | W2 | Water Quality | Added several calls to prevent computation of kinetic variables if epiphyton are defined in the control file with NEP=1 or more but is not ACTIVE or turned ON. If the kinetic expressions are non-zero and the initial concentration is given, then this could add source/sink terms to the oxygen balance. This is typical of the code changes – since several of this type were made: Old code: DO JE=1,NEP PO4EG(K,I) = PO4EG(K,I)+EGR(K,I,JE)*EPC(K,I,JE)*EP(JE) PO4ER(K,I) = PO4ER(K,I)+ERR(K,I,JE)*EPC(K,I,JE)*EP(JE) END DO New code: IF (EPIPHYTON_CALC(JW,JE))then ! SW 1/18/2008 PO4EG(K,I) = PO4EG(K,I)+EGR(K,I,JE)*EPC(K,I,JE)*EP(JE) PO4ER(K,I) = PO4ER(K,I)+ERR(K,I,JE)*EPC(K,I,JE)*EP(JE) endif | 1/18/2008 |
| 30 | W2 | Bottom processes | Continuation of bug fix #24 in such places as these: New Code: IF(K == KB(I))THEN xdum=BI(K,I)/BH2(K,I) ! SW 1/18/08 ELSE xdum=BI(K,I)/BH2(K,I)*(1.0-BI(K+1,I)/BI(K,I)) ENDIF SEDAS(K,I) = SEDAS(K,I)+MAX(AS(JA),0.0)*ALG(K,I,JA)*xdum ! SW 1/18/08 SEDOMS(K,I) = pomS(JW)*(LPOM(K,I)+RPOM(K,I))*xdum !sw 1/18/08 cb 10/22/06 IF(K==KB(I))THEN ! SW 1/18/08 SEDSO = 0.0 ELSE SEDSO = sedS(JW)*SED(K,I)*BI(K+1,I)/BH2(K,I)*(1.0-BI(K+1,I)/BI(K,I)) Endif DO K=KT,KB(I) IF(K == KB(I))THEN xdum=BI(K,I)/BH2(K,I) ! SW 1/18/08 ELSE xdum=BI(K,I)/BH2(K,I)*(1.0-BI(K+1,I)/BI(K,I)) ENDIF DO JA=1,NAL SEDASp(K,I) = SEDASp(K,I)+MAX(AS(JA),0.0)*ap(ja)*ALG(K,I,JA)*xdum ! SW 1/18/08 END DO DO JE=1,NEP IF (EPIPHYTON_CALC(JW,JE))LPOMEPp(K,I) = LPOMEPp(K,I)+EPOM(JE)*ep(je)*(EMR(K,I,JE)*EPC(K,I,JE)) END DO do jd=1,nbod This code is repeated similarly in many of the sediment routines. | 1/18/2008 |
| 31 | W2 | Add segment initialization | The DEPTHM and DEPTHB were not initialized correctly when a segment was added – this does not affect internal computations, just output for SPR and SNP files. Old Code: BKT(I) = BH1(KT,I)/H1(KT,I) DEPTHB(K,I) = H1(KT,I) ! DEPTHM(K,I) = H1(KT,I)*0.5 New Code: BKT(I) = BH1(KT,I)/H1(KT,I) DEPTHB(KT,I) = H1(KT,I) ! SW 1/27/08 DEPTHM(KT,I) = H1(KT,I)*0.5 ! SW 1/27/08 | 1/27/08 |

## W2 V3.2 Bug Fixes, Enhancements, and User Manual Changes

Table 7. Bug fixes and enhancements for Version 3.2.

| # | Code: W2 or PREW2 or GUI | Fix or Enhancement Type | Description of Bug/Enhancement | Date Bug Fixed or Enhancement Added |
| --- | --- | --- | --- | --- |
| 1 | W2 | Waterbody-waterbody connection | When there was negative velocities at a waterbody-waterbody connection, there was a possibility (dependent on the bathymetry of the connection at the waterbody-waterbody intersection) that there could be temperature or concentration anomalies. | 8/31/04 |
| 2 | W2 | Lateral_ withdrawal | Added limit to the DLRHOMAX function: Old code: DLRHOMAX=MAX(DLRHOT,DLRHOB) New code: DLRHOMAX=MAX(DLRHOT,DLRHOB,1.0E-10) | 1/25/05 |
| 3 | W2 | Branch connectivity | Logic in branch connectivity set-up was fixed Old code: IF(UHS(JB) == DS(JJJB))EXIT New code: IF(abs(UHS(JB)) == DS(JJJB))EXIT | 1/25/05 |
| 4 | W2 | Pumpback | Pumpback logic was corrected – this is legacy code that will probably be removed from later versions of W2. Old code: DO JB=1,NBR IF (JB == JBP) JWBP = JW END DO New code: DO JW=1,NWB DO JB=BS(JW),BE(JW) IF(JB == JBP) JWBP = JW END DO END DO | 1/25/05 |
| 5 | W2 | CPL write | Switched order of implied DO loop on CPL write statement for output of constituents | 1/25/05 |
| 6 | W2 | PRF write | Changed output format for PRF output for constituents from f10.2 to e13.6 | 1/25/05 |
| 7 | W2 | Heat balance | Added the Idso and Jackson long wave radiation equation when air temperatures are below 5C. The Swinbank model underpredicts long wave incoming radiation at low air temperatures by as much as 10%. The computation of long wave atmospheric radiation is done using the approach of Swinbank (1963) unless air temperatures are less than 5oC, when the Idso and Jackson (1969) formula is used (Wells, et al., 1982). The Swingbank formula for clear sky long wave atmospheric radiation is where units are W/m2, oC at 2 m height. Below 40oF (5oC) the formula of Idso and Jackson is recommended (above 10oC both equations are almost identical): where units are W/m2 and Ta is in units of oC. The Stefan-Boltzmann constant = 5.62E-8 W/m2/(oK)4. | 1/25/05 |
| 7 | W2 | Layer addition algorithm | Mistyped subscript K instead of I: Old code: IF (KB(I) > KBI(I)) THEN B(KB(K),I) = 0.0 DX(KB(I),I) = 0.0 KB(I) = KB(I)-1 IF (I /= DS(JB)+1) KBMIN(I) = MIN(KB(K),KB(I+1)) IF (I /= US(JB)-1) KBMIN(I-1) = MIN(KB(I-1),KB(I)) New Code: IF (KB(I) > KBI(I)) THEN B(KB(I),I) = 0.0 ! SW 3/2/05 DX(KB(I),I) = 0.0 KB(I) = KB(I)-1 IF (I /= DS(JB)+1) KBMIN(I) = MIN(KB(I),KB(I+1)) ! SW 3/2/05 IF (I /= US(JB)-1) KBMIN(I-1) = MIN(KB(I-1),KB(I)) | 3/2/05 |
| 8 | W2 | Variable initialization | In some cases when there was a layer subtraction and a time step violation immediately afterward, the variable SW was not initialized properly. This caused problems in the Tomas Algorithm for the water surface computation. The following line of code was added to the SUB layer algorithm: SW(KT-1,IU-1:ID+1) = 0.0 !TC 3/9/05 Also, the variable AVHR was defined in the Update variables for DS+1. The following new code was added: AVHR(KT,DS(JB)+1)=H1(KT,DS(JB)+1) !SW 03/08/05 | 3/9/05 |
| 9 | W2 | Interpolation multipliers | Possible index error when there are multiple waterbodies. Old code: RATZ(K,JW) = AVH2(K-1,I)/AVH2(K,I) CURZ1(K,JW) = 2.0*H(K,JW)**2/(AVH2(K-1,I)+AVH2(K,I))/AVH2(K-1,I) CURZ2(K,JW) = -2.0*H(K,JW)**2/(AVH2(K-1,I)*AVH2(K,I)) CURZ3(K,JW) = 2.0*H(K,JW)**2/(AVH2(K-1,I)+AVH2(K,I))/AVH2(K,I) END DO New code: RATZ(K,JW) = AVH2(K-1,DS(BE(JW)))/AVH2(K,DS(BE(JW))) CURZ1(K,JW) = 2.0*H(K,JW)**2/(AVH2(K-1,DS(BE(JW)))+AVH2(K,DS(BE(JW))))/AVH2(K-1,DS(BE(JW))) CURZ2(K,JW) = -2.0*H(K,JW)**2/(AVH2(K-1,DS(BE(JW)))*AVH2(K,DS(BE(JW)))) CURZ3(K,JW) = 2.0*H(K,JW)**2/(AVH2(K-1,DS(BE(JW)))+AVH2(K,DS(BE(JW))))/AVH2(K,DS(BE(JW))) | 5/10/05 |
| 10 | W2 | Spillway and Gates | Older code in order to check if it was submerged or not used the elevation difference relative to the channel bed on either side of the weir, rather than the weir crest. Also removed this code line: IF(ELDN>ESP(JS))DH+ELUP-ELDN | 5/10/05 |
| 11 | W2 | Reaeration | Corrected formula errors in Thackston and Krenkel formula: Old code: USTAR=SQRT(ADEPTH*SLOPE(JB)*32.2)**0.5 REAER(I) = 24.88*(1.0+SQRT(0.176*UAVG/SQRT(ADEPTH)))*USTAR New code: USTAR=SQRT(ADEPTH*SLOPE(JB)*32.2) REAER(I) = 24.88*(1.0+SQRT(0.176*UAVG/SQRT(ADEPTH)))*USTAR/ADEPTH Similar changes were made to the updated Thackston model (Eqn 10) | 5/10/05 |
| 12 | W2 | Violations NV | The variable BI and VOL was not initialized properly during a time-step violation. | 8/25/05 |
| 13 | W2 | ADD a layer | The variable BI was not initialized properly during an ADD layer. | 8/25/05 |
| 14 | W2 | TRIDIAG subroutine | Insert Deallocate Statement in Tridiag: SUBROUTINE TRIDIAG(A,V,C,D,S,E,N,U) USE PREC INTEGER, INTENT(IN) :: S, E, N REAL(R8), DIMENSION(:), INTENT(IN) :: A(E),V(E),C(E),D(E) REAL, DIMENSION(:), INTENT(OUT) :: U(N) REAL(R8), ALLOCATABLE, DIMENSION(:) :: BTA, GMA ALLOCATE (BTA(N),GMA(N)) BTA(S) = V(S) GMA(S) = D(S) DO I=S+1,E BTA(I) = V(I)-A(I)/BTA(I-1)*C(I-1) GMA(I) = D(I)-A(I)/BTA(I-1)*GMA(I-1) END DO U(E) = GMA(E)/BTA(E) DO I=E-1,S,-1 U(I) = (GMA(I)-C(I)*U(I+1))/BTA(I) END DO Deallocate (BTA, GMA) <----- ! SW 10/17/05 END SUBROUTINE TRIDIAG | 10/17/05 |
| 15 | W2 | SUB layer | In SUB Layer/Sub Seg - eliminated parentheses which caused a sign error: IF (.NOT. TRAPEZOIDAL(JW)) THEN BI(KT,IU-1) = B(KTI(IU-1),I) H1(KT,IU-1) = H(KT,JW)-Z(IU-1) BH1(KT,IU-1) = B(KTI(IU-1),IU-1)*(EL(KT,IU-1)-EL(KTI(IU-1)+1,IU-1)-Z(IU-1)*COSA(JB))/COSA(JB) <------ ! SR 10/17/05 IF (KT >= KB(IU-1)) BH1(KT,IU-1) = B(KT,IU-1)*H1(KT,IU-1) DO K=KTI(IU-1)+1,KT BH1(KT,IU-1) = BH1(KT,IU-1)+BH1(K,IU-1) END DO ELSE | 10/17/05 |
| 16 | W2 | SUB layer for shallow systems | Layer SUB - improved model running in shallow segments. !** Water surface minimum thickness DO JW=1,NWB KT = KTWB(JW) ZMIN(JW) = -1000.0 KTMAX = 2 <----- ! SR 10/17/05 DO JB=BS(JW),BE(JW) DO I=CUS(JB),DS(JB) IF(KB(I) > KTMAX) KTMAX = KB(I) <----- ! SR 10/17/05 IF (Z(I) > ZMIN(JW)) THEN IZMIN(JW) = I JBIZ = JB END IF ZMIN(JW) = MAX(ZMIN(JW),Z(I)) END DO END DO ADD_LAYER = ZMIN(JW) < -0.85*H(KT-1,JW) .AND. KT /= 2 SUB_LAYER = ZMIN(JW) > 0.60*H(KT,JW) .AND. KT < KTMAX <------- ! SR 10/17/05 .... !******** Upstream active segment IUT = US(JB) IF (SLOPE(JB) /= 0.0) THEN DO I=US(JB)-1,DS(JB)+1 IF (KB(I) < KT )THEN <------------------- ! SR 10/17/05 KB(I) = KT B(KB(I),I) = 0.000001 DX(KB(I),I) = DXI(JW) ... !****** Additional layer subtractions ZMIN(JW) = -1000.0 DO JB=BS(JW),BE(JW) DO I=CUS(JB),DS(JB) ZMIN(JW) = MAX(ZMIN(JW),Z(I)) END DO END DO SUB_LAYER = ZMIN(JW) > 0.60*H(KT,JW) .AND. KT < KTMAX <------ ! SR 10/17/05 END DO END DO Also done for the initial set-up of the branch geometry: !**** Upstream active segment and single layer IF (SLOPE(JB) /= 0.0) THEN DO I=US(JB)-1,DS(JB)+1 IF (KB(I) < KT ) THEN <----- ! .AND. I /= IZMIN(JW) SW 10/17/05 B(KT,I) = 0.000001 | 10/17/05 |
| 17 | W2 | Shade algorithm | No errors; just an improvement in computational efficiency. Delete this from the SHADING subroutine: !** Set the angles for which topographic shade data are available DO II=1,IANG ANG(II)=((II-1)*(360.0/FLOAT(IANG)))*PI/180.0 END DO GAMMA = (2*PI)/IANG And change the occurrences of gamma to gama (only in shading subroutine): ANG2 = (TOPO(I,J+1)-TOPO(I,J))/GAMA <---- ! SW 10/17/05 TOPOANG = TOPO(I,J)+ANG2*ANG1 ENDIF END DO IF (AZ00 > ANG(IANG) .AND. AZ00 <= 2*PI) THEN ANG1 = AZ00-ANG(IANG) ANG2 = (TOPO(I,1)-TOPO(I,IANG))/GAMA <----! SW 10/17/05 ADD a line to the module SHADEC: MODULE SHADEC PARAMETER (IANG=18) REAL, PARAMETER :: GAMA=(3.1415926*2.)/REAL(IANG) <--- ! SW 10/17/05 REAL, DIMENSION(IANG):: ANG <----! SW 10/17/05 REAL, ALLOCATABLE, DIMENSION(:) :: A00, DECL, HH, TTLB, TTRB, CLLB, CLRB <------- ! SW 10/17/05 REAL, ALLOCATABLE, DIMENSION(:) :: SRLB1, SRRB1, SRLB2, SRRB2, SRFJD1, SRFJD2,SHADEI REAL,ALLOCATABLE, DIMENSION(:,:) :: TOPO LOGICAL, ALLOCATABLE, DIMENSION(:) :: DYNAMIC_SHADE DATA ANG /0.00000, 0.34907, 0.69813, 1.04720, 1.39626, 1.74533, 2.09440, 2.44346,& 2.79253, 3.14159, 3.49066, 3.83972, 4.18879, 4.53786, 4.88692, 5.23599, 5.58505, 5.93412/ <----- ! SW10/17/05 END MODULE SHADEC Delete allocation statement for ang: ALLOCATE (SRLB1(IMX), SRRB1(IMX), SRLB2(IMX), SRRB2(IMX), SRFJD1(IMX), SHADEI(IMX), SRFJD2(IMX)) ALLOCATE (TOPO(IMX,IANG)) <---- !SW10/17/05 ALLOCATE (QSW(KMX,NWDT), CTR(NCT,NTRT), HPRWBC(NHY,NWB)) Delete ang from the deallocate statement: DEALLOCATE(TTLB, TTRB, CLLB, SRLB1, SRRB1, SRLB2, SRRB2, SRFJD1, SHADEI,SRFJD2, TOPO, QSW, CTR) <----! SW 10/17/05 | 10/17/05 |
| 18 | W2 | Epiphyton algorithm | Several changes were made that corrected errors in shallow systems where adding and subtracting layers did not reinitialize macrophyte layers when the current KT was below KB. The epiphyton burial rate was greater than specified in the control file. Epiphyton that are buried become part of the 1st order organic sediment (as before). Epiphyton mortality now becomes part of the LPOM pool (based on the EPOM fraction) and is settled and transported downstream rather than going into the organic 1st order sediment model directly. Currently this is non-photosynthesizing – but this will change in the next version. | 5/26/06 |
| 19 | W2 | ADD/SUB layers | There was a bug in addition and subtraction of layers that led to water quality variables not being initialized correctly during riverine shallow flow. | 5/26/06 |
| 20 | User Manual | Typos corrected | The manual had a few typos that were corrected. | 6/11/2006 |
| 21 | W2 | Waterbody-waterbody connection | The subroutine Upstream_velocity under specific conditions did not maintain flow continuity across a waterbody-waterbody connection. | 6/29/2006 |
| 22 | W2 | SNP output | The algal limiting nutrient SNP output had a bug under specific conditions in writing out the information. | 6/30/2006 |
| 23 | W2 | Sediment heating and sediment processes | If a model added and subtracted layers that resulted in segment addition and subtraction, there was the possibility that sediment fluxes were incorrectly computed. In the NO3 subroutine: Old code: NO3SED(K,I) = NO3(K,I)*NO3S(JW)*NO3TRM(K,I)*(BI(K,I)-BI(K+1,I))/BH2(K,I) New code: if(k == kb(i)) then NO3SED(K,I) = NO3(K,I)*NO3S(JW)*NO3TRM(K,I)*(BI(K,I))/BH2(K,I) else NO3SED(K,I) = NO3(K,I)*NO3S(JW)*NO3TRM(K,I)*(BI(K,I)-BI(K+1,I))/BH2(K,I) endif New code added in sediment routine: if(k == kb(i))then ! SW 4/18/07 SODD(K,I) = SOD(I)/BH2(K,I)*SODTRM(K,I)*BI(K,I) else SODD(K,I) = SOD(I)/BH2(K,I)*SODTRM(K,I)*(BI(K,I)-BI(K+1,I)) Endif New code added in suspended solids routine: if(k == kb(i))then SSR = EPSILON*DLX(I)*BI(K,I)/VOL(K,I) else SSR = EPSILON*DLX(I)*(BI(K,I)-BI(K+1,I))/VOL(K,I) Endif | 4/18/07 |
| 23 | W2 | (See above) | New code added for heat flux to channel bottom: if(kt == kb(i))then ! SW 4/18/07 SROSED = SROOUT*TSEDF(JW) else SROSED = SROOUT*(1.0-BI(KT+1,I)/BI(KT,I))*TSEDF(JW) Endif if(k==kb(i))then ! SW 4/18/07 TFLUX = CBHE(JW)/RHOWCP*(TSED(JW)-T2(K,I))*BI(K,I)*DLX(I) else TFLUX = CBHE(JW)/RHOWCP*(TSED(JW)-T2(K,I))*(BI(K,I)-BI(K+1,I))*DLX(I) endif New code added for sediment subroutine: if(k == kb(i))then ! SW 4/18/07 SEDAS(K,I) = SEDAS(K,I)+MAX(AS(JA),0.0)*ALG(K,I,JA)*BI(K,I)/BH2(K,I)*(1.0-BI(K+1,I)/BI(K,I)) else SEDAS(K,I) = SEDAS(K,I)+MAX(AS(JA),0.0)*ALG(K,I,JA)*BI(K,I)/BH2(K,I)*(1.0-BI(K+1,I)/BI(K,I)) endif if(k == kb(i))then ! SW 4/18/07 SEDOMS(K,I) = POMS(JW)*(LPOM(K,I)+RPOM(K,I))*BI(K,I)/BH2(K,I) SEDSO = POMS(JW)*SED(K,I)*BI(K+1,I)/BH2(K,I) else SEDOMS(K,I) = POMS(JW)*(LPOM(K,I)+RPOM(K,I))*BI(K,I)/BH2(K,I)*(1.0-BI(K+1,I)/BI(K,I)) SEDSO = POMS(JW)*SED(K,I)*BI(K+1,I)/BH2(K,I)*(1.0-BI(K+1,I)/BI(K,I)) endif | 4/18/07 |
| 24 | W2 | Algae | The logic for negative settling velocities for algae had an error. Old code: ! ASR(K,I,JA) = -AS(JA)*(ALG(K+1,I,JA)*B(K+1,I)/(B(K,I)*H2(K,I))-ALG(K,I,JA))*BI(K,I)/BH2(K,I) New code: ASR(K,I,JA) = -AS(JA)*(ALG(K+1,I,JA)*BI(K+1,I)/BH2(K,I)-ALG(K,I,JA)*BI(K,I)/BH2(K,I)) !SP 8/27/07 Shwet Prakash | 8/27/07 |